| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4217 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-AV: Addressing Editor’s notes |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This contribution proposes a number of updates to H.TPS-AV resulting from the discussion of several Editor’s notes.

# Discussion

The output draft H.TPS-AV (TD 645R1/WP2) from the Geneva, November-December 2011 meeting contained a number of Editor’s notes. The Contributors have reviewed these notes and have proposed modifications to H.TPS-AV as a result of the review. The discussion of each of the editor’s notes is captured as Contributor’s notes in the proposal section below.

The Contributors have also performed an editorial review of the draft and have proposed a number of additions/changes to the references, definitions, abbreviations and bibliography.

# Proposal

It is proposed to add the following changes indicated by the change marks to H.TPS-AV. Change marks are against TD 645R1/WP2 from the Geneva, Switzerland meeting 21 November – 02 December 2011 meeting.

Note to Editor: The contributor’s notes should not be added to draft H.TPS-AV.

Draft new ITU-T H.TPS-AV

Audio/Video Parameters for Telepresence systems

…

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	ITU-T Recommendation X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[ITU-R BT.709]	ITU-R Recommendation BT.709-5 (04/2002), Parameter values for the HDTV standards for production and international programme exchange

[ITU-T H.245]	ITU-T Recommendation H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.323]	ITU-T Recommendation H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T P.56]	ITU-T Recommendation P.56 (12/2011), Objective measurement of active speech level.

[ITU-T Q.931]	ITU-T Recommendation Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF CLUE FW]		IETF draft-ietf-clue-framework-01, Framework for Telepresence Multi-Streams.

[ISO 11664-2]		ISO 11664-2:2007 (CIE S 014-2/E:2006) (2007), Colorimetry -- Part 2: CIE standard illuminants.

[ISO 140-4]		ISO 140-4:1998 (1998), Acoustics -- Measurement of sound insulation in buildings and of building elements -- Part 4: Field measurements of airborne sound insulation between rooms.

[ISO 717-1]		ISO 717-1:1996 (1996), Acoustics – Rating of sound insulation in buildings and of building elements -- Part 1: Airborne Sound Insulation.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal. A capability may also relate to how a terminal captures or renders signal.

3.2.2	Cycles Per Degree: Is a measurement of how many alternating white and black lines an eye can be differentiate per degree of visual field.

{Contributor’s note: In defining parameters related to capabilities it is important to understand in the general sense what a capability may be. It is expected that this definition will evolve. Further definitions may be required.}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| 3D | Three dimensional |
| --- | --- |
| CPD | Cycles Per Degree |
| CRI | Colour Rendering Index |
| RAS | Registration, Admission and Status |
| RTP | Real Time Protocol |
| RTCP | RTP Control Protocol |
| SRI | Sound Reduction Index |
| STC | Sound Transmission Class |
| VUI | Video Usability Information |

…

##### 7.1.2.2	Effective Resolution

{Contributor’s note: An abbreviation and definition has been proposed above, thus it is proposed to delete the editor’s note.}

Human visual acuity is the ability to distinguish fine detail.  It is generally measured in cycles per degree (CPD), and therefore measures an angular resolution.  People with excellent visual acuity can resolve 50 CPD – 1.2 arc minute per line pair (or 0.35 mm line pair at 1 meter).(See [b-ImagePro])..

Camera resolution is usually measured in lines across the entire horizontal or vertical field of view.  Black and white lines are alternated.  A cycle is equivalent to 2 lines of camera resolution – that is,  a black+white line pair comprises a cycle.  The field of view of a camera is measured in degrees.  Therefore the cycles per degree for a camera can be computed as lines/(2*field_of_view).

Similarly display resolution is measured in pixels (width by height).  For modern flat panel displays, a line is equivalent to a row or column of pixels.  The cycles per degree for a display depends on the subtended angle of the display, which in turn depends on the size of the display and the distance of the display surface from the viewer.  Therefore, the cycles per degree for a display can be computed as pixels/(2*subtended_angle).

##### 7.1.2.2.1	Example

A typical 3-screen telepresence system uses 3 cameras with horizontal field of view of 25 degrees.  It uses 65 inch 16x9 displays, and is calibrated to show images at actual size.

Each display has a width of 56.65 inches (1439 mm), and the viewers are seated 3.245 meters from the display.  At this distance, the horizontal subtended angle is also 25 degrees, matching the camera field of view.

The native resolution of each display is 1920 x1080 pixels (1080p); which corresponds to 1920/(2*25) cycles per degree – or about 38.4 CPD.

Similarly, 720p corresponds to about 24 CPD. A 65” 4K display at this viewing distance provides about 71 CPD, which exceeds the resolving power of the human eye.

| Identity: | EffectiveResolution |
| --- | --- |
| Description: | This parameter indicates effective resolution of a rendered video stream as perceived by the viewer. |
| Format: | Floating Point number |
| Possible Values: | Positive numbers in Cycles per Degree |
| Default: | None. |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems should provide effective resolutions between 25 CPD and 50 CPD.

…

### 7.3.4	Reverberation time

{Contributor’s note: The proposed values for Reverberation time appear consistent with P Suppl.16.}

Reverberation time should be typically between 0.3 and 0.4 seconds, but at a minimum of 0.2 seconds, when measured in the 125 Hz-4 kHz octave bands. Such values would apply to medium size meeting rooms with a volume of approximately 100 m3 or less. For larger meeting rooms, a slightly higher reverberation time would be acceptable.

This represents somewhat lower reverberation times than it is usual in meeting rooms today, but these values are absolutely attainable in practice.

The audio quality will worsen gradually with increasing reverberation times above 0.4 s.

| Identity: | ReverberationTime |
| --- | --- |
| Description: | This parameter indicates the reverberation time used in a telepresence meeting room environment. |
| Format: | TBD |
| Possible Values: | between 0.3 s and 0.4 s |
| Default: | none |
| Reference: | See clause 3 / [b-P Suppl. 16] for further information regarding reverberation considerations. |
| Interface: | none |

### 7.3.5	Background noise

{Contributor’s note: The value of < 30 dBA exceeds the maximum levels provided by Table 1 / P Suppl.16] i.e. 35db for a 50 person conference room. The supplement also provides a range of values depending on the size of the conference room.}

Care should be taken to minimize the background (ambient) noise level, including both noise from technical installations (air-conditioning, in particular) and any other external noise transmitted into the room. For a high quality experience, the noise level should be less than 30 dBA.

This can be a difficult goal to meet in some buildings, and most telepresence systems will work well with higher background noise levels. However, the quality of experience will in general gradually decrease with increasing noise level. For more information regarding rating of sound insulation buildings see [ISO 717-1].

| Identity: | BackgroundNoise |
| --- | --- |
| Description: | This parameter indicates the background noise level used in a telepresence meeting room environment. |
| Format: | TBD |
| Possible Values: | < 30 dBA |
| Default: | none |
| Reference: | See clause 3 / [b-P Suppl. 16] for further information regarding ambient noise. |
| Interface: | none |

### 7.3.6	Sound insulation

{Contributor’s note: P Suppl. 16 does not detail sound reduction. ISO 140 (parts 1-14) detail the sound reduction index (SRI). In particular ISO 717-1 discusses airborne sound insulation in buildings and building elements. There appears to be some inconsistency between the symbols and terminology. The following terms are used, but are not the same according to the ISO specifications:

- Apparent sound reduction index ( Symbol R’)

- Weighted apparent sound reduction index (Symbol R’w)

- Sound reduction index (Symbol R)

The contributors believe that the intent is to use “Weighted apparent sound reduction index (R’w) however this would need to be checked with the previous contributor. Consistent use of this term is proposed.}

Recommendations for sound insulation will vary according to the local building plan and external noise sources. However, one universal recommendation for privacy can be stated. To ensure that confidential information cannot be overheard from outside the room, the weighted apparent sound reduction index (SRI) (R’w) of the room boundaries is recommended to be R’w ≥ 48 dB. See [ISO 140-4] for more information.  This SRI approximately corresponds to a sound transmission class STC ≥ 50.

Note: STC is another measure of sound reduction. See [b-ASTM E90] and [b-ASTM E413] for more information.

Note that the doors of the meeting room should have a R’w ≥ 35 dB (STC ≥ 35).

| Identity: | SoundInsulation |
| --- | --- |
| Description: | This parameter indicates the weighted apparent sound reduction index (R’w) used in a Telepresence meeting room environment. |
| Format: | TBD |
| Possible Values: | ≥ 48 dB |
| Default: | none |
| Reference: | None. |
| Interface: | none |

Annex A

<Annex Title - Placeholder>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

Bibliography

[b-ImagePro]	John C.Russ, The Image Processing Handbook, CRC press, 2006.

[b-ASTM E90]	ASTM E90 - 09 Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements

[b-ASTM E413]	ASTM E413 - 10 Classification for Rating Sound Insulation.

_________________