| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | TD-32 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | F.USN-SM, “Capabilities of ubiquitous sensor network (USN) for supporting requirements of smart metering services” (New): Output draft (Geneva, 20-24 Feb. 2012) |  |  |  |
| Purpose: | Information |  |  |  |

Summary

This document contains output texts of draft new Recommendation ITU-T F.USN-SM, “Capabilities of ubiquitous sensor network (USN) for supporting requirements of smart metering services,” based on the discussion results during the ITU-T WP2/16 rapporteur meeting (Geneva) on February 2012. Moreover, ITU-T F.USN-SM has a plan for Consent at the next ITU-T SG16 meeting on April 2012.

The following table shows discussion results for contributions.

| Contribution No. | Contribution title and proposals | Agreements |
| --- | --- | --- |
| AVD-4212a | Proposed modification on F.USN-SM |  |
|  | - A step for flows of schedule information from smart metering operator to smart meters is added in scenario I, III, IV in clause 7 and is also reflected in the figures of each scenario. | - Agreed as the proposal, but “setting a schedule mode” in each figure is not relevant, so this needs to be revised in further contribution |
|  | - In clause 7.5, the scenario is more specifically described with 5 steps, and the new figure for the scenario V is added. | - Agreed as the proposal |
|  | - “Distributed processing” is newly added in clause 10.10 | - Agreed as the proposal |
|  | - Editorial comments in the whole document | - Agreed as the proposal |
| AVD-4194 | - Add the new term, “USN gateway” in clause 3.1.6 | - Agreed as the proposal |
|  | - Clause 3.2 is rearranged as an alphabetical order | - Agreed as the proposal |
|  | - A new paragraph for better understanding is added in clause 6. | - Agreed as the proposal |
|  | - The last paragraph is moved to the front in clause 6.1, and the second paragraph is not relevant so removed. | - Agreed as the proposal |
| AVD-4195 | - A new paragraph for better understanding is added in clause 8. | - Agreed as the proposal |
|  | - “Time synchronisation” is added in clause 8.1. | - Agreed as the proposal |
|  | - “Minimal time delay” is added in clause 8.3. | - Agreed as the proposal |
|  | - “Change of parameters for regularly scheduled reporting including tariff configuration” is added, and “Monitoring and management of smart meters” is added in clause 8. | - Agreed as the proposal, but the two clauses have similarities about management, so the two clauses needs to be merged into one as clause 8.9. |
|  | - In clause 8.7, “Security support” is changed to “Security support including authorisation of operator and data confidentiality”. | - Agreed as the proposal |
|  | - “Authentication of smart meters” is added in clause 8.8. | - Agreed as the proposal |
|  | - “Time synchronisation” is added in clause 9.1. | - Agreed as the proposal |
|  | - “Remote configuration setting and re-setting of sensor nodes” is added, and “Monitoring and management of sensor nodes” is added in clause 9. | - Agreed as the proposal, but the two clauses have also similarities about management, so the two clauses needs to be merged into one as clause 9.12. |
|  | - In clause 9.8, “Security” is changed to “Security supporting confidentiality, integrity check, authorisation and authentication” and new paragraphs are added for better understanding. | - Agreed as the proposal |
| AVD-4197 | - Propose use of capital letters in the main title such as “Capabilities of Ubiquitous Sensor Network …” | - Not agreed, because the capital letters are recommended in the title according to the author’s guide. |
|  | - Several sentences are added in the Summary for clarification. | - Agreed as the proposal |
|  | - There are two similar keywords: “meter reading data” and “metering information”. Clarify them and resolve this for making consistency | - As the proposal, the two key words have the similar meaning, so, the keyword “meter reading data” is changed to “metering information”. |
|  | - Relocate this clause 9 into clause 6 as clause 6.3. The clause 7, 8, and 10 are well mated each other, but clause 9 describes functional components and a typical configuration of USN to support smart metering services. It looks like an overview description in terms of USN. This clause had better go into clause 6, “Overview of smart metering” as clause 6.3. | - Agreed as the proposal |

The following table shows discussion results for comments from the floor.

| Comments | Agreements |
| --- | --- |
| - According to review results in advance, this document is mature enough to be in progress for consent. | - Agreed as discussed, but unnecessary texts (e.g., editors’ note and Justification of this Recommendation) should be removed through the further contribution. |
| - Not only in clause 8.7 but also in the whole document, two similar keywords: “meter reading data” and “metering information” are used. This also needs to be clarified and resolved for making consistency. | - Of course, the two keywords are used in the whole document, but the two keywords have slightly different meaning and should be used separately in some context, so the two keywords should be alternatively merged according to context. |

Justification of this Recommendation:

(Editor’s note)  This is not a part of the main body of F.USN-SM and was initially prepared for information. It is because it could keep used for information until its final state This will be removed before Consent.

Gap analysis on smart metering of ITU and other SDOs

There are two related activities for smart metering issues in the ITU and other SDOs.

- ITU-T

In ITU-T, there is an activity to deal with a topic about smart metering. The title is “Smart metering using connected objects in ubiquitous home networking environments,” and deals with the concept, characteristics, scenario and functional architecture for smart metering in telecommunication perspectives.

- ETSI

ETSI TC M2M (Machine-to-Machine) published Smart Metering Use Cases. This introduces M2M application for smart metering and examples, and it describes the use cases for the smart metering M2M applications.

However, these two activities do not deal with USN capabilities for supporting smart metering services.

Contents

Pages

1	Scope	7

2	References	7

3	Definitions	7

3.1	Terms defined elsewhere	7

3.2	Terms defined in this Recommendation	8

4	Abbreviations and acronyms	8

5	Conventions	8

6	Overview of smart metering	8

6.1	Smart grids and smart metering	9

6.2	Technical overview of smart metering	10

6.3	USN-based smart metering services	11

7	Smart metering service scenarios	12

7.1	Scenario I: regularly scheduled remote meter reading	12

7.2	Scenario II: on-demand remote meter reading	13

7.3	Scenario III: demand response	14

7.4	Scenario IV: tariff configuration	15

7.5	Scenario V: meter reading data aggregation	16

8	Requirements of smart metering services	17

8.1	Time synchronisation	17

8.2	Reliable information delivery	17

8.3	Minimal time delay	17

8.4	Real-time delivery of meter reading data	17

8.5	Bi-directional communication between meters and operators	18

8.6	Security support including authorisation of operator and data confidentiality	18

8.7	Authentication of smart meters	18

8.8	Meter reading data processing	18

8.9	Monitoring and management of smart meters	18

9	USN capabilities for smart metering services	18

9.1	Time synchronisation	18

9.2	Reliable transmission	19

9.3	Scalability	19

9.4	Mobility support	19

9.5	Delivery latency	19

9.6	Fault detection and recovery	19

9.7	Security supporting confidentiality, integrity check, authorisation and authentication	19

9.8	Connectivity	19

9.9	Unicasting and Multicasting	19

9.10	Data aggregation	20

9.11	Distributed processing	20

9.12	Monitoring and management of sensor nodes	20

Bibliography	20

Draft new Recommendation ITU-T F.USN-SM

Capabilities of ubiquitous sensor network (USN) for supporting requirements of smart metering services

AAP Summary

To be provided before Consent.

Summary

This Recommendation identifies capabilities of Ubiquitous Sensor Network (USN) for supporting requirements of smart metering services. To this end, an overview of smart metering is described with clarification between smart grids and smart metering. This Recommendation takes into account a few typical use case scenarios of smart metering and identifies technical requirements and USN-based smart metering services to support the use cases. Finally this Recommendation defines USN capabilities based on the identified requirements to provide smart metering services.

Keywords

Smart grid, smart metering, USN

# Scope

The main purpose of this Recommendation is to identify capabilities of Ubiquitous Sensor Network (USN) which supports the requirements of smart metering services. The scope of this Recommendation covers the following:

- Overview of smart metering;

- Smart metering use case scenarios;

- Requirements of  smart metering services;

- USN capabilities for supporting requirements of  smart metering services;

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2221]	ITU-T Recommendation Y.2221 (2010), Requirements for support of ubiquitous sensor network (USN) applications and services in the NGN environment

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	sensor [ITU-T Y.2221]: An electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

3.1.2	sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

3.1.3	sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with  capabilities of sensed data processing and networking.

3.1.4	smart grid [ITU-T FG-Smart]: An electricity network that can intelligently integrate the actions of all users connected to it - generators, consumers and those that do both - in order to efficiently deliver sustainable, economic and secure electricity supplies.

3.1.5	ubiquitous sensor network (USN) [ITU-T Y.2221]: A conceptual network built over existing physical networks which makes use of sensed data and provides knowledge services to anyone, anywhere and at any time, and where the information is generated by using context awareness.

3.1.6	USN gateway [ITU-T Y.2221]: A node which interconnects sensor networks with other networks.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	demand response: A smart metering feature that allows consumers to reduce or change their use patterns of electricity, gas and water during peak demand usually in exchange for a financial incentive.

3.2.2	sensor network gateway: Sensor network element that connects a sensor network to another network with different architectures or protocols, permitting information exchange between them. See also USN gateway.

NOTE: Sensor network gateway functionalities may include either address or protocol translation or both.

3.2.3	smart meter: A premise device to monitor and control electrical power, gas and water usage of home appliances based on demand response information from home appliances.

3.2.4	smart metering: An operation to provide information to consumers and smart metering operator about energy consumption. The information includes how much energy the consumers are consuming or generating and how much it costs.

3.2.5	smart metering gateway: See USN gateway

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ETSI | European Telecommunications Standards Institute |
| --- | --- |
| M2M | Machine-to-Machine |

# Conventions

None.

# Overview of smart metering

Smart meters are utility meters, for example electricity, gas, water and heat meters, which may bring about the end of estimated bills and meter readings, and provide customers and energy distributors and suppliers with accurate information on the amount of utility being used.

Smart metering provides:

- Customers with the information they require to become energy savvy and make smarter decisions about their energy usage,

- Energy suppliers with the means to better understand and service their customers,

- Distributors with an effective tool to better monitor and manage their networks.

In addition, smart metering enables those customers who choose to generate their own electricity (micro generators) to be financially rewarded for their contribution to the national grid and distributors to better manage this contribution [b-ESTI TR 102.691].

Smart metering may be regarded as one of the key technologies for smart grid system.

## Smart grids and smart metering

Smart grid is a type of electrical grid that attempts to predict and intelligently respond to the behaviour and actions of all electric power users connected to it - suppliers, consumers and those that do both – in order to efficiently deliver reliable, economic, and sustainable electricity services including:

- Enhancement of reliability

- Reducing peak demand

- Shifting usage to off-peak hours

- Lower total energy consumption

- Actively managing electricity charging

- Actively managing other usage to respond to other renewable resources

Smart grids technologies have already used in other applications, such as manufacturing and telecommunications. In general, smart grid technology may be divided into seven areas, such as integrated communications, sensing and measurement, smart metering, advanced components and control, improved interfaces and decision support, and smart power generation. In other words, smart meters are part of the smart grids as key components, and also smart metering is one of crucial features for the smart grids.

Figure 1: Smart Grid Architecture [b-NIST]

## Technical overview of smart metering

Not only governments and utility companies, such as electricity, gas, and water suppliers, but also researchers have been interested in automatic meter reading based on communication system, known as smart metering because smart metering can bring following benefits to customers and governments as well as utility companies for examples:

- Lower metering cost

- Energy savings for residential consumers

- Reliability of supply

- Various pricing schemes to make new consumers attracted

- Easier detection of fraud and outage

- Automated billing

Smart metering performs metering and exchanging meter information between smart meters and utility companies. Various technologies for metering and exchanging meter information have been implemented. Power line communication has been used as means for delivering electricity power to consumers and transmitting gas and water measurements to the utility providers. In addition, messaging over GSM network, for example, has been utilized for an automatic meter reading system.

Sensor network technologies may be used for metering and collecting information of utility usages, and communication networks can be used for exchanging the information. Figure 2 depicts an overall diagram of smart metering systems. The meter information, obtained from home appliances by sensor nodes is collected and delivered to operators of utility companies.

The operators manage the collected information and inform consumers of variable pricing information or enforced load control messages. Such operators’ action may lead to consumers’ reducing energy consumption.

Figure 2: Overview of smart metering

## USN-based smart metering services

The smart metering services shown in Figure 3 require technological facilities to support metering and exchanging metering information between smart meters and metering operators in utility control centers. Basic characteristic of USN applications and services is gathering data by sensor networks and transmitting data to a remote server through communication networks. Also, control information is transmitted to sensor networks from the remote server. Furthermore, if USN middleware is deployed, the middleware can provide USN applications or sensor networks with various functions such as data query, data mining, event processing, sensor network metadata directory service, data filtering, context-aware rule processing and sensor network management. Those features of USN can be applied to the smart metering services. Sensor nodes in sensor networks have capabilities of metering and delivering metered information to a server at a utility control center. USN middleware can provide functions which are required by on-demand remote meter reading scenario and demand response meter reading scenario.

A smart metering system can be implemented by sensor nodes and USN middleware as follows.

- Smart metering sensor nodes: Each sensor node may have capabilities of smart metering, therefore sensor nodes can act as smart meters supporting of processing smart metering information for the smart metering procedures, such as meter reading collection and tariff setting.

- Smart metering gateway: A sensor network gateway or a USN gateway is capable of connecting a sensor network to another network with different architectures or protocols, permitting information exchange between them. Therefore a sensor network gateway or a USN gateway can be used as a smart metering gateway. Smart metering gateway is only responsible to deliver information between the sensor nodes (smart meters) and metering operators. In case a gateway has capabilities of smart metering, it collects meter reading data and then delivers to the metering operator(s).

- USN middleware: USN middleware provides data filtering, data query, data mining and context-aware rule processing, etc. These features of USN middleware satisfy meter reading data processing requirement and can be applied to on-demand remote meter reading scenario and demand response meter reading scenario.

Figure 3: Smart metering service based on USN

# Smart metering service scenarios

## Scenario I: regularly scheduled remote meter reading

Scenario I describes procedures that meter reading data are delivered to smart metering operators at regularly scheduled intervals.

- Smart metering operator sends a message including schedule information to smart meters.

- Smart meters are set up as the schedule for measurement and data transfer.

- According to the schedule, smart meters measure energy consumption from home appliances and obtain measurement data.

- The smart meters deliver the data to the smart metering operator.

Figure 4: Regularly scheduled remote metering

## Scenario II: on-demand remote meter reading

Scenario II describes procedures that smart meters measure energy consumption and delivered the results to smart metering operator on demand.

- When a smart metering operator is required to collect measurement data from the smart meters, the operator sends a task message to smart meters in order to collect measurement data.

- The smart meters verify the operator’s message and then measure energy consumption if the message is valid.

- The result data are delivered to the smart metering operator.

Figure 5:  On-demand remote meter reading

## Scenario III: demand response

Scenario III describes how customers respond to metering operator’s demands.

- Smart metering operator sends a message including schedule information to smart meters.

- Smart meters are set upas the schedule for measurement and data transfer.

- When the number of home appliances turned on simultaneously is on the increase, the total amount of energy consumption also steeply increases.

- Information about the increasing amount of energy consumption is being reported to the smart metering operator at scheduled intervals, and price of the energy may also be increased by tariff policies depending on supply and demand.

- (a) The smart metering operator sends a message which may inform consumers of the increased price.

(b) The smart metering operator sends load control messages to enforce to reduce energy consumption or shut off by force due to management policy.

- (a) The smart meters display the message to the consumers, and the consumers may decide to reduce energy consumption, or the home appliances may be switched off by a consumer.

(b) When receiving the load control messages, the smart meter display the message and shut off. The connected home appliance is automatically switched off.

Figure 6: Demand response

## Scenario IV: tariff configuration

Scenario IV describes how price of energy consumption is decided and configured into the meters.

- Smart metering operator sends a message including schedule information to smart meters.

- Smart meters are set up as the schedule for measurement and data transfer.

- The smart meters regularly deliver meter reading data to the metering operator according to regularly scheduled intervals.

- The price of the electricity may be dynamically changed by pricing policies depending on supply and demand.

- When the price is changed, a message including changed price information is delivered to all of the smart meters.

- The changed cost information may be shown on their displays, and the changed cost is applied.

Figure 7: Tariff configuration

## Scenario V: meter reading data aggregation

This scenario describes procedures that meter reading data, obtained at regularly scheduled intervals by smart meter, are collected at aggregating smart meter (or at aggregating smart metering gateway) and then delivered to metering operators.

- Smart metering operator sends smart meters message to request aggregated metering data.

- Smart meters may be scheduled to obtain meter reading data.

- According to the schedule, smart meters measure energy consumption and obtain meter reading data from home appliances.

- The obtained meter reading data are collected in an aggregating smart meter (or aggregating smart metering gateway).

- Finally, meter reading data are transferred to a smart metering operator.

Figure 8: Meter reading aggregation

# Requirements of smart metering services

This clause specifies the requirements of smart metering services. However, all the requirements of smart metering services are not specified here but the requirements which may be related with sensor networks or USN are specified.

## Time synchronisation

In smart metering services, exchanged data uses time stamp between smart meters and the systems of smart metering operators. Therefore, accurate and secure time synchronisation should be supported in smart metering systems.

## Reliable information delivery

In the smart metering services, meter reading data can be frequently transferred from distributed meters, and also operators can send many control messages to smart meters. If delivery has failed, the operators cannot collect all of meter reading data from smart meters. As a result, missing meter reading data would lead to undesirable results. Therefore, the smart metering services are required to provide reliable information delivery, such as meter reading data and control messages described in the scenario.

## Minimal time delay

In case of on-demand remote meter reading, meter reading request and response should be delivered to the peer with minimal time delay.

## Real-time delivery of meter reading data

Demands of smart metering operators require real-time responses of consumers. Therefore, meter reading data are required to arrive at the operators within a given amount of time. In the scenario I-3 and II-3, if the meter reading data are not delivered within the given time, the operators cannot obtain real-time results of smart metering. Therefore, smart metering services are required to guarantee real-time delivery.

## Bi-directional communication between meters and operators

Meter reading data are delivered from smart meters to smart metering operators as shown in the scenario I-3, II-3, and III-2, and the control messages are also sent from the operators to the smart meters vice versa (in scenario III-3 and IV-3). For doing this, the smart metering services are required to support bi-directional communication between meters and operators.

## Security support including authorisation of operator and data confidentiality

Operators need to control meters in order that the meters take actions according to the indication of operators. In this case, the operators should be authorized by smart meters to take actions to secure against unauthorized access (scenario II-2).

Because metering information may include personal information, it should not be revealed to unauthenticated third parties. Therefore, the metering information is required to be secured to unauthenticated third parties as well as support of integrity check of the data exchanged.

## Authentication of smart meters

Smart meters should be authenticated by smart metering applications. Authentication of smart metering devices can be achieved by smart metring applications directly or the authenticated smart metering gateways.

## Meter reading data processing

Either operators or smart meters should be able to perform data processing on meter reading such as filtering, validation and aggregation. In some case, data mining processing is necessary, for example, analysing patterns and predicting some events.

## Monitoring and management of smart meters

Smart meters or smart metering gateways should be monitored proactively in order to attempt to prevent and correct errors. In addition, the following management capabilities should be supported from the network side:

- Secure software and firmware provisioning

- Configuration management

- Auto-configuration functions for smart meters area networks

Also, during operation of smart metering service, smart metering applications should be able to specify a regularly scheduled reporting (scenario I) of specific parameters for specific metring devices. Also, smart metering application should be able to modify the value of the requested time period. When tariff configuration is necessary to be changed, smart metering applications should be able to change the tariff configuration of smart meters.

# USN capabilities for smart metering services

## Time synchronisation

USN is required to support accurate and secure time synchronisation among sensor nodes, sensor network gateways and the smart metering applications.

## Reliable transmission

Smart metering requires reliability of metering information delivery to ensure correct results. Therefore, USN is recommended to guarantee reliable transmission for data and message delivery.

## Scalability

New smart meters (sensor nodes) can be added to, or one of the existing meters (sensor nodes) can be removed from an existing smart metering group (sensor networks). Such a change should not degrade the performance of the smart metering.  Therefore, sensor networks in USN are required to support scalability.

## Mobility support

In some cases, a consumer can change the positions of home appliances. This change may require changes a meters’ position in the intra-sensor network or inter-sensor networks. Therefore, sensor networks in USN are recommended to support mobility of smart meters (sensor nodes).

## Delivery latency

Meter reading data should be delivered with minimal time delay or within a given time. For this requirement, USN is recommended to guarantee data delivery with minimal time delay or within a given time.

## Fault detection and recovery

There is a possibility of link failure between nodes due to the characteristics of wireless transmission when sensor nodes use wireless communications,. Such a failure can give negative effects on reliability and delivery latency of the smart metering services. USN is recommended to detect faults and recover from such faults.

## Security supporting confidentiality, integrity check, authorisation and authentication

Meter reading data and messages may be transferred to/from operators and meters by hop-by-hop transmission among sensor nodes. Therefore, hop-by-hop security among sensor nodes is recommended to be implemented and USN is recommended to provide integrity check of the data exchanged. In addition, USN is recommended to provide smart meters with authorisation capabilities for access to smart meters of smart metering operators.

Smart meters should be authenticated by smart metering applications directly or through smart metering gateway. Therefore USN is recommended to provide authentication capability to smart metering applications for authentication of sensor nodes (smart meters).

## Connectivity

Meter reading data, obtained from each sensor node, are collected by a gateway, and then the collected meter reading data are delivered to smart metering operators through transport networks.  In addition, control messages of the operators are also delivered to each sensor node. In order to achieve this, connectivity between sensor nodes, connectivity between sensor nodes and the gateway, and connectivity between the gateway and outer networks are recommended to be guaranteed.

## Unicasting and Multicasting

A unicast networking service is required to be supported among sensor nodes so that meter reading data are delivered to smart metering operators. Furthermore, control messages of the metering operators should be delivered to all sensor nodes. Therefore, multicast networking support is also required.

## Data aggregation

Meter reading data from smart meters is collected and aggregated at smart meter or smart metering gateways. And then they are transferred to the smart metering operator(s). Therefore, USN sensor nodes and gateways are required to support data aggregation.

## Distributed processing

If there are a number of sensor nodes (smart meters) in smart home or smart building, burst meter reading data can be created and transferred to smart metering operator simultaneously. To prevent node and service failure because of burst data centralisation, distributed processing is required.

## Monitoring and management of sensor nodes

Sensor nodes (smart meters) or sensor networks gateways (smart metering gateways) are recommended to be monitored and managed proactively in order to attempt to prevent and correct errors including secure software and firmware provisioning and configuration management as well as auto-configuration functions for sensor networks.

Also, smart metering applications require change of parameters in smart meter for regularly scheduled reporting as well as change of tariff configuration. For satisfying this requirement, USN is recommended to support remote configuration setting and re-setting of sensor nodes acting as smart meters.

Bibliography

[b-ESTI TR 102.691]	ESTI TR 102.691 v1.1.1, “Machine-to-Machine communications (M2M); Smart Metering Use Cases,” May 2010.

[b-ITU-T Smart-I-6]		ITU-T FG-Smart Grid, Smart-I-6, “Smart metering using connected objects in ubiquitous home networking environments,” June 2010.

[b-Khalifa] 			T. Khalifa et. al., “A Survey of Communication Protocols for Automatic Meter Reading Applications,” IEEE Communications surveys and tutorials, Vol. PP, Issue 99, January 2010.

[b-NIST] 				NIST Special Publication 1108, NIST Framework and Roadmap for Smart Grid Interoperability Standards, Release 1.0, January, 2010

[TBD]

____________________