| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4231 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.RTPTOPO |  |  |  |
| Title: | H.248.RTPTOPO “RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations” (New): Input draft (Ed. 0.11) |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the editor’s input for draft new H.248.RTPTOPO “RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations”.

2.	Discussion

This document is a copy of TD 748R1/WP2 from the SG16 meeting in Geneva, Apr/May 2012. There are some editorial changes to the document.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.RTPTOPO Ed. 0.1 | Initial draft (TD-38) from WP 2/16 Research Triangle Park (USA) meeting (2010-11), based on AVD 4000 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1011_Res/TD-38.zip |
| H.248.RTPTOPO Ed. 0.2 | Input (TD 458 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0458 |
| H.248.RTPTOPO Ed. 0.3 | Input (TD 458R1 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0458R1 |
| H.248.RTPTOPO Ed. 0.4 | Input (AVD-4051) to WP 2/16 Andover (USA) meeting (2011-07) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/AVD-4051.zip |
| H.248.RTPTOPO Ed. 0.5 | Output draft (TD-39) from WP 2/16 Andover (USA) meeting (2011-07) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/TD-39.zip |
| H.248.RTPTOPO Ed. 0.6 | Input (TD 620 (WP2/16)) from ITU-T SG16 Geneva meeting, Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0620/ |
| H.248.RTPTOPO Ed. 0.7 | Output (TD 620R1 (WP2/16)) from ITU-T SG16 Geneva meet., Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0620R1 |
| H.248.RTPTOPO Ed. 0.8 | Input (AVD-4162) to WP 2/16 Geneva (USA) meeting (2012-02) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/AVD-4162.zip |
| H.248.RTPTOPO Ed. 0.9 | Input (TD 748 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0748 |
| H.248.RTPTOPO Ed. 0.10 | Output (TD 748R1 (WP2/16)) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0748R1 |
| H.248.RTPTOPO Ed. 0.11 | Input (AVD-4231) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4231.zip |
| H.248.RTPTOPO Ed. 0.12 | Output (TD-xx) from WP 2/16 Brisbane meeting (2012-09) Location: |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 7 3.1 Terms defined elsewhere 7 3.2 Terms defined in this Recommendation 7 4 Abbreviations 7 5 Conventions 8 6 Background to RTCP handling by H.248 MGs 8 7 RTP topologies versus RTCP handling in H.248 MGs 9 7.1 Single H.248 ”IP” Termination per Context 9 7.2 Two H.248 ”IP” Termination per Context 10 7.3 More than two H.248 ”IP” Terminations per Context 10 7.4 Others 11 7.5 Notes to the H.248 Connection Model 11 7.6 Analysis to be done (RTCP handling) 12 7.7 Relation to modes of operations of IP-to-IP media gateways 12 8 RTCP services 13 8.1 RTCP packet structure 13 8.2 Important RTCP information elements (from H.248 MG perspective) 14 8.2.1 Information elements related to the identification of an RTP entity 14 8.2.2 Other relevant information elements 14 8.3 RTCP service levels 14 8.3.1 Basic RTCP services 14 8.3.2 RTP profile dependent RTCP services 14 8.3.3 Supplementary RTCP services 14 9 Scope of RTP topology control 15 10 RTP Topology Package 16 10.1 Properties 16 10.1.1 RTP Topology 16 10.2 Events 18 10.3 Signals 18 10.4 Statistics 18 10.5 Error Codes 18 10.6 Procedures 18 10.6.1 General 18 10.6.2 MGC level information for RTP topology control 18 10.6.3 Topology dependent procedures 18 Bibliography 20 Appendix I Living List related to RTP Topology Package 21 Appendix II Living List – Discussion of RTP Topologies in case of SRTP 22 II.1 Use-case #1: ITU-T H.248 MG for peering IP and circuit-switched networks 22 II.2 Use case #2: ITU-T H.248 MG for peering IP networks 23 II.3 Use case #3: Transparent SRTP forwarding 24

List of Tables

Page

Table 1 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Single H.248 ”IP” Termination per Context 10 Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Two H.248 ”IP” Terminations per Context 10 Table 3 – RTP Topologies versus RTCP Handling versus H.248 MG Models – More than two H.248 ”IP” Terminations per Context 11 Table 4 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Others 11

List of Figures

Page

Figure 1 (copy of Fig. G.3/[b-ETSI TR 183 068]): BGF modes - H.248 Local and Remote Descriptor: SDP "m="-line specification combinations 13 Figure 2 - RTCP packet structure (Compound RTCP packet) 13 Figure 3 – Level of RTCP services 15 Figure II.1 – Use case #1: SRTP to circuit-switched ITU-T H.248 MG 23 Figure II.2 – Use case #2: SRTP to RTP ITU-T H.248 MG 24 Figure II.3 – Use case #3: ITU-T H.248 MG with transparent SRTP forwarding 25

Draft new ITU-T Recommendation H.248.RTPTOPO

Gateway control protocol: 
RTP Topology dependent RTCP Handling 
by H.248 Media Gateways with IP Terminations

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

The scope of this Recommendation is related to RTCP handling by H.248 Media Gateways (MG). The required MG behaviour is fairly clear for the very majority of use cases. However, there are some scenarios which benefit from or even require explicit signalling indication for as an unambiguous control of MG RTCP (but also RTP) traffic handling behaviour.

This Recommendation covers

- information about history and motivation behind RTP topologies and RTCP handling and the available H.248 tool landscape (clause 6);

- overview tables concerning “RTP topology versus RTCP handling” aspects and relevant H.248 connection models (clause 7);

- RTCP service aspects (clause 8);

- the RTP topology configurations related to the various RTCP service levels covered by the package defined in this Recommendation (clause 9); and

- an H.248 package for basic RTP topology control (clause 10).

This Recommendation may be useful in scenarios where an MG supports multiple RTP topologies and/or multiple RTP/RTCP services which may lead to ambiguous behaviour. In these scenarios a H.248 profile specification may use the package in this Recommendation to alleviate this behaviour.

# 2	References

{Editor’s note: this is a preliminary list of documents related to RTP/RTCP. This is just a living list.}

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3, including Amendment 2 (12/2009).

[ITU-T H.248.30]	Recommendation ITU-T H.248.30 (06/2008), Gateway control Protocol: IP NAPT traversal package.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control Protocol: RTCP XR Block Reporting Package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control Protocol: NAT traversal toolkit packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (06/2010), Gateway control Protocol: RTP control protocol package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (06/2008), Gateway control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control Protocol: RTCP support packages.

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3611]	IETF RFC 3611 (11/2003), RTP Control Protocol Extended Reports (RTCP XR).

[IETF RFC 4585]	IETF RFC 4585 (07/2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4961]	IETF RFC 4961 (07/2007), Symmetric RTP / RTP Control Protocol (RTCP).

[IETF RFC 5093]	IETF RFC 5093 (12/2007), BT's eXtended Network Quality RTP Control Protocol Extended Reports (RTCP XR XNQ).

[IETF RFC 5117]	IETF RFC 5117 (01/2008), RTP Topologies.

[IETF RFC 5124]	IETF RFC 5124 (02/2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5506]	IETF RFC 5506 (04/2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF RFC 5725]	IETF RFC 5725 (02/2010), Post-Repair Loss RLE Report Block Type for RTP Control Protocol (RTCP) Extended Reports (XRs)).

[IETF RFC 5760]	IETF RFC 5760 (02/2010), RTP Control Protocol (RTCP) Extensions for Single-Source Multicast Sessions with Unicast Feedback.

[IETF RFC 5761]	IETF RFC 5761 (04/2010), Multiplexing RTP Data and Control Packets on a Single Port.

[IETF RFC 5968]	IETF RFC 5968 (09/2010), Guidelines for Extending the RTP Control Protocol (RTCP)).

[IETF RFC 6128]	IETF RFC 6128 (02/2011), RTP Control Protocol (RTCP) Port for Source-Specific Multicast (SSM) Sessions.

[IETF draft monarch]	IETF RFC xxxx (xxx), Monitoring Architecture for RTP.
Note: present IETF state = ‘WGLC’ (draft-ietf-avtcore-monarch)
https://datatracker.ietf.org/doc/draft-ietf-avtcore-monarch/

# 3	Definitions

## 3.1	Terms defined elsewhere

None.

## 3.2	Terms defined in this Recommendation

3.2.1	RTP domain: A network domain with RTP entities which share a common RTP/RTCP-related identifier space. A RTP domain is characterized therefore by a domain wide (and temporal limited) uniqueness of such protocol identifiers, which typically constitute unambiguous RTP traffic flows (e.g., protocol elements SSRC, CNAME, PT).

NOTE 1 – [b-IETF RFC 6222] differentiates the temporal horizon in short-term persistent and long-term persistent identifier value assignments. (tbc)

NOTE 2 – An end-to-end RTP traffic flow could traverse multiple RTP domains, leading to the following observations:

- The end-to-end communication association would be comprised of multiple, concatenated RTP sessions.

- The edge of RTP domains is given by RTP entities with RTP topologies of type “RTP Endsystem” or “Back-to-Back RTP Endsystem” (tbc).

{Editor’s note (2012-04): the term “RTP domain” is so far just a working assumption and dependent whether this term is finally used by this Rec. The definition is also motivated by the fact that the IETF is already also using that term, but appearently without any definition by RFC’s.  For further studies.}

{Editor’s note (2012-04): it might be beneficial to define term “RTCP service” because it is used in clauses 1, 8 & 9, but not any definition available from IETF side. For further discussion.}

# 4	Abbreviations

This Recommendation uses the following abbreviations and acronyms:

| ASY | Asymmetric topologies |
| --- | --- |
| AVP | (RTP) Audio Visual Profile |
| AVPF | (RTP) Audio Visual Profile with Feedback |
| B2BRE | Back-to-Back RTP Endsystem |
| BGF | Border Gateway Function NOTE - synonym to Border Gateway and IP-to-IP Gateway in this Recommendation |
| CNAME | Canonical Name |
| CSRC | Contributing Source |
| IP | Internet Protocol |
| LD | Local Descriptor |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| PT | (RTP) Payload Type; (RTCP) Packet Type |
| RD | Remote Descriptor |
| RR | (RTCP) Receiver Report |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| RTPASM | RTP Any Source Multicast |
| RTPE | RTP Endsystem (or RTP endpoint) |
| RTPMU | RTP Multicast |
| RTPMT | RTP Media Translator |
| RTPRTM | RTP RTCP-Terminating-MCU |
| RTPSSM | RTP Source Specific Multicast |
| RTPTT | RTP Transport Translator |
| RTPVSM | RTP Video-Switch-MCU |
| SAVP | (RTP profile) Secure Audio Visual Profile |
| SDES | Source Description (RTCP Packet) |
| SR | (RTCP) Sender Report |
| SSRC | Synchronization Source |
| VoIP | Voice over Internet Protocol |
| XR | (RTCP) eXtension Report |

# 5	Conventions

None.

# 6	Background to RTCP handling by H.248 MGs

The handling of RTCP packets by H.248 media gateways is often not straightforward, sometimes not fully clear also from IETF RFC side. RTCP handling in general is tightly coupled to the RTP topology model, which drives the processing of RTP/RTCP traffic of a particular H.248 Context.

The aspect of RTP topologies is for many H.248 MG types not really relevant because the handling is obvious. For example: for all H.248 (IP, physical) connection models (as applied for VoIP MGs at the border towards circuit-switched networks). However, for H.248 (IP,IP) MGs the situation is slightly different.  In this case new RTCP services have been defined that increase the possible interactions between RTP topology and these services. This has led to possible ambiguity in the RTCP handling behaviour in the MG.

The problem also has an historical dimension: [IETF RFC 1889] and [IETF RFC 3550] were too vague on that point, which was the motivation to clarify on a more detailed level RTP topology models, published as [IETF RFC 5117]. [IETF RFC 5117] provides a more fine granular topology classification and detailed description.

NOTE - It may be noted that the problem is not specific to H.248, rather concerns also other gateway/policy control protocols, which are applied for IP-IP gateways or policy enforcement points. For example: [ITU-T H.248.48]  considers different RTCP processing functions (e.g., local generation of RTCP reports, forwarding/filtering of RTCP reports). [ITU-T H.248.48] is actually applicable for multiple RTP topologies and could be categorized as a supplementary RTCP service (see clause 8.3.3). It has had to consider these interaction issues.

Generally RTP topology and RTCP handling may be summarized:

- Local RTCP handling by an H.248 MG is basically linked to the applied RTP topology, and the RTP topology is given by the Terminations/Streams in a H.248 Context.

- Without this Recommendation explicit H.248 means are not defined to control RTP topology. Thus, H.248 profiles that do not utilise this Recommendation assume implicit behaviour (“which may lead to misinterpretations”).

- Existing H.248 protocol elements for RTP/RTCP control are for other, different purposes, i.e.:

  - [ITU-T H.248.1] Annex E.12 “RTP Package”: event and statistics

  - [ITU-T H.248.30] “RTCP extended performance metrics packages”:
- RTCP XR Base Package,
- RTCP XR Burst Metrics Package

  - [ITU-T H.248.48] “RTCP XR Block Reporting Package”: statistics

  - [ITU-T H.248.50] “RTP Keep Alive Request Package”: “RTP signals”
{Editor’s note: Signal “Keep Alive Packet Type” indicates some RTP/RTCP related packet types, which represent the semantic of RTP topology “RTP endsystem” (sender only).
Example of a controversial situation: the H.248 Stream endpoint may be enabled with signal e.g. kar/kapt/cn (-> RTP endsystem) and also part of a RTP transport translator, leading to some contradictions …}

  - [ITU-T H.248.57] “RTP Control Protocol Package”: L4 port allocation control

  - [ITU-T H.248.58] “RTP Application Data Package”: statistics

  - [ITU-T H.248.71] “RTCP support packages”:
- RTCP Source Description Package, 
- Received RTCP Package, 
- RTCP Feedback Message Package

The topic as such, correct RTCP handling by H.248 MGs, is important in order not to jeopardize end-to-end RTP-based services.

# 7	RTP topologies versus RTCP handling in H.248 MGs

This clause provides an attempt to structure RTP Topologies vs RTCP Handling vs H.248 MG Context configurations.

## 7.1	Single H.248 ”IP” Termination per Context

See Table 1.

Table 1 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Single H.248 ”IP” Termination per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| I.1 | RTP Endsystem (RTPE) (aka RTP Endpoint) | F-1,2,3,5 | (RTP, non-RTP) | VoIP Residential -, Access – or Trunking MG |

## 7.2	Two H.248 ”IP” Termination per Context

See Table 2.

Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Two H.248 ”IP” Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| II.1 | Back-to-Back RTP Endsystem (B2BRE) | F-1,2,3,4,5,6,7 | (RTP, RTP) | IP-to-IP MG located at the border between two RTP domains (e.g., SRTP-to-RTP interworking) |
| II.2 | RTP Transport Translator (RTPTT) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, without any media processing |
| II.3 | RTP Media Translator (RTPMT) in general (NOTE 1) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, with processing of the RTP payload |
| II.3.1 | RTP Media Translator without media format change (RTPMTlight) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., transpacketization (changing packetization times), jitter reduction between two IP networks |
| II.3.2 | RTP Media Translator with media format change (RTPMTfull) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., audio transcoding |
| NOTE 1 – The RTPMT topology is sometimes further sub-classified here with II.3.1 & II.3.2 (see e.g. [b-IETF ECN]). |  |  |  |  |

NOTE - Some media-level functions may be achieved by more than one RTP topology. For example, audio transcoding may be done in B2BRE or RTPMT topologies. However, there would be different RTCP handling functions.

## 7.3	More than two H.248 ”IP” Terminations per Context

See Table 3.

Table 3 – RTP Topologies versus RTCP Handling versus H.248 MG Models –
More than two H.248 ”IP” Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| III.1 | RTP Mixer (RTPM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.2 | RTP Video-Switch-MCU (RTPVSM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.3 | RTP RTCP-Terminating-MCU (RTPRTM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server where each H.248 RTP Termination relates to the “RTP Endsystem” topology |
| III.4 | RTP Multicast (RTPMU) | TBD (see clause 7.6) | TBD |  |
| III.4.1 | RTP Source Specific Multicast (RTPSSM) | TBD (see clause 7.6) | TBD |  |
| III.4.2 | RTP Any Source Multicast (RTPASM) | TBD (see clause 7.6) | TBD |  |

## 7.4	Others

See Table 4.

Table 4 – RTP Topologies vs RTCP Handling vs H.248 MG Models –
Others

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| IV.1 | Asymmetric topologies (RTPASY) | TBD (see clause 7.6) | (RTP, MUX, RTP … RTP) (TBD). | H.248 Contexts with internal H.248 MUX terminations … |
| IV.2 | Topology combinations | TBD (see clause 7.6) | (TBD). | Cascaded H.248 Contexts … |

## 7.5	Notes to the H.248 Connection Model

H.248 profiles typically use the notation of “IP” for IP-based ephemeral terminations (e.g. in order not to exclude any IP applications, thus a transport protocol and application protocol notation). This Recommendation uses“RTP” instead because there may be a difference between following connection models:

- (RTP, RTP)

  - MG is “RTP-aware”, which enforcesa transport-protocol aware mode of operation

  - MG may be media aware or media agnostic

- (IP, IP)

  - MG may be “RTP-agnostic”, i.e., aware of the transport protocol (UDP), but unaware of higher protocol layers, thus RTP packets may transparently go through such an H.248 Context.

However, the used naming scheme for ephemeral terminations in H.248 connection models is minor and not relevant to this discussion.

NOTE -  A H.248.TerminationID should not be used to indicate the RTP Topology.

## 7.6	Analysis to be done (RTCP handling)

There is not yet any comprehensive analysis done concerning the check in identifying the concrete RTCP functions in above Tables 1 to 4. There might be following functions on RTCP packets or RTCP related functions:

F-1:	RTP node identifier: usage (or not) of MG-local CNAMEs (per MG; per termination?);

F-2:	RTP source identifier: allocation (or not) of MG-local SSCRs per RTP session;

F-3:	RTCP protocol endpoint for basic services: origination/termination of RTCP packets (reports);

F-4:	RTCP packet forwarding: unmodified forwarding of RTCP packets (reports);

F-5:	RTCP packet modification: rewrite of RTCP packets;

F-6:	RTCP protocol endpoint for supplementary services: local generation of RTCP reports (such as for RTCP XR based applications);

F-7:	RTCP packet filtering: filtering of RTCP reports (e.g., either entire RTCP packet or just an individual RTCP report within a compound packet is filtered);

F-8:	others

Editor’s note: some items are overlapping, list may need to be revised again …

## 7.7	Relation to modes of operations of IP-to-IP media gateways

There is a side question concerning the possible relation between RTP topologies and defined modes of operations for IP-to-IP media gateways?

There are many modes of operations discriminated for IP-to-IP media gateways, like e.g. “media-format agnostic, transport protocol type aware”, etc. [b-ETSI TR 183 068] provides an established categorization which could be summarized according Figure 1.

The modes of operation are related to the media description in the local and remote descriptors. The present classification is based on the SDP “m=”-line only.

Open items:

- such H.248 modes of operation for (IP,IP) connection models may be firstly disjoint to RTP topologies (e.g., the modes "media-agnostic" and "media-aware" may be applicable for the same RTP topology);

- however, the used RTP profile (AVP, AVPF, SAVP, …) may appear in the “m=” line transport field, and thus the specific RTP profile may provide associated RTCP behaviour …

Such type of questions should be clarified, too.

Figure 1 (copy of Fig. G.3/[b-ETSI TR 183 068]): 
BGF modes - H.248 Local and Remote Descriptor: 
SDP "m="-line specification combinations

# 8	RTCP services

Purpose of this clause is to describe the handling of RTCP packets, dependent on the RTP topology and used RTCP services.

## 8.1	RTCP packet structure

Figure 2 illustrates the basic RTCP packet structure at the example of a compound RTCP packet.

Figure 2 - RTCP packet structure (Compound RTCP packet)

The RTCP packet structures need to be considered to identify key information elements (see next subclause).

## 8.2	Important RTCP information elements (from H.248 MG perspective)

Not every information element of RTCP packets is relevant from a H.248 perspective. There is however some elements which demand careful consideration when processing RTP/RTCP traffic by H.248 MGs.

### 8.2.1	Information elements related to the identification of an RTP entity

There are primarily three information element types:

- SSRC (per originating RTP source),

- CSRC (for contributing RTP source), and

- SDES.

These elements define the spatial horizon of RTP media and RTCP control flows, i.e. the "end-to-end" significance of RTCP services. See for instance the detailed evaluation of example H.248 measurement architectures by [ITU-T H.248.48] and IETF monitoring architectures for RTP by [IETF draft- monarch].

### 8.2.2	Other relevant information elements

TBD

{Editor’s note: there might be RTCP information elements of primary interests for RTP end-systems only …FFS.}

## 8.3	RTCP service levels

### 8.3.1	Basic RTCP services

Every RTP implementation must be compliant to [IETF RFC 3550] and thus basically support all the RTCP services defined by this RFC. They may be called basic RTCP services and cover the four RTCP packet types of SR (200), RR (201), SDES (202) and BYE (203).

NOTE - The codepoint for application defined RTCP packet types APP (204) is also introduced by this RFC, but the definition of application specific services is delegated to subsequent RFCs.

Basic RTCP services are indicated to the MG by e.g. RTCP port allocation (see [ITU-T H.248.57]) for processing RTCP control flow traffic by an H.248 context.

### 8.3.2	RTP profile dependent RTCP services

RTP profiles may add additional RTCP services like e.g. the RTP profiles RTP/AVPF and RTP/SAVPF.

The use of RTP profile dependent RTCP services are basically indicated to the MG by the protocol field value in the SDP "m=" line, as part of the LD-/RD-embedded SDP media description.

### 8.3.3	Supplementary RTCP services

RTP sessions may be supplemented by further RTCP-based services. [IETF RFC 5968] defines the generic extension principles for RTCP. Such kind of supplementary services are for instance the RTCP extension report (XR) based capabilities, as defined for measurement and monitoring architectures.

Supplementary RTCP services are indicated to the MG by e.g. RTCP report/packet type specific signalling elements like e.g. H.248 packages [ITU-T H.248.30] and [ITU-T H.248.48], or correspondent SDP attributes as part of the LD-/RD-embedded SDP media description.

{Editor’s note: next step would be the completion of Tables 1 to 4 concerning RTCP handling input. The table input would have to address

- the guidelines from RFC 5117 for each topology,

- the key RTCP information elements, and

- the RTCP service levels.

One result might be that the RTP topology is basically just impacting the basic RTCP and RTP profile dependent services. And supplementary RTCP services would be an overlay on top of RTP topologies … For further studies.}

# 9	Scope of RTP topology control

There is a relation between RTP topology and RTCP services. However, the scope of RTP topology control needs to be limited due to the plethora of RTCP services and in order to handle complexity. Figure 3 summarizes again the main RTCP service categories (from clause 8).

Figure 3 – Level of RTCP services

The scope of RTP topology control, - based on the H.248 rtpt package defined by this Recommendation -, could be pragmatically limited on basic (I) and RTP profile dependent (II) RTCP services.

{Editor’s note (2012-04): discussion of C-802 was soliciting some text on the notion of “RTCP services”, which might be subject of clause 9 or 8.3. C-802 mentions some example generic service categories such as performance monitoring, media encryption, codec rate control, congestion control, generic feedback, keep alive, etc.… }

Example:

The H.248 RTP stream in an H.248 IP-IP MG could be configured for RTP topology “Topo-Translator”, which implies RTCP services such as

- no allocation of local SSRC values,

- no generation of RTCP RR reports, and

- forwarding of RTCP BYE packets.

The same H.248 RTP stream could be additionally enforced for H.248.48 type performance monitoring capabilities, which could lead to RTCP services such as

- allocation of local SSRC values for measurement points, or/and

- generation of RTCP XR reports.

The H.248.48 related supplementary RTCP services are not in contradiction to the RTP topology associated ones.

# 10	RTP Topology Package

|  | Package Name: | RTP Topology Package |
| --- | --- | --- |
|  | Package ID: | "rtpt" (0x????) |
|  | Description: | The package allows a MGC to indicate to a MG which particular RTP topology an RTP stream is operating under. The RTP Topologies are fundamentally defined by [IETF RFC 5117] and complemented by this Recommendation. A single H.248 Stream allows the definition of two RTP streams thus the topology is defined at a Local and Remote Descriptor level. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

### 10.1.1	RTP Topology

|  | Property Name: | RTP Topology |
| --- | --- | --- |
|  | Property ID: | rtptopo (0x0001) |
|  | Description: | This property indicates which RTP topology (as primarily defined by [IETF RFC 5117]) that an RTP Stream is acting under. |
|  | Type: | Enumeration |
|  | Possible values: | MG (0x0000) MG Determined PP (0x0001) Topo-Point-to-Point (Note 1) MC (0x0002) Topo-Multicast TR (0x0003) Topo-Translator MX (0x0004) Topo-Mixer VS (0x0005) Topo-Video-Switch-MCU RT (0x0006) Topo-RTCP-terminating-MCU TA (0x0007) Topo-Asymmetric TF (0x0008) Topo-Transparent-Forwarding (Note 2) NOTE 1 – A pair of two connected Stream endpoints (between two RTP terminations within a context) and PP topology is synonymous with a Back-to-Back RTP End system (B2BRE) topology from the perspective of an end-to-end RTP session. NOTE 2 – An H.248 IP-IP MG could be “RTP aware” (due to correspondent content in the H.248 Stream Descriptor), but required to forward RTP packets in a transparent manner. |
|  | Default: | MG |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

{Editor’s note (2012-04): the necessity of the “TF codepoint” is still questioned because there might be two alternatives:

- H.248.1 basic behaviour by LD/RD settings: “identical media descriptions” imply “transparent media processing” by the MG. That’s implicit semantic in H.248. That’s not really clear is the actual protocol layer of transparent forwarding of Lx-PDUs. Just the application layer PDU or a lower layer PDU.

- H.248 profiles: some profiles for border gateways provide explicit semantic when media is not processed. However, there are two issues: the first one is similar like list item 1, and the second issue relates to the fact that “transparent media processing” could be achieved by at least two options (either “-“ based or by “identical fully specified media descriptions”).

Further, the notion of “transparent forwarding” implies that the MG shall not provide any protocol termination of RTP (i.e., not any dejittering, not any loss concealment, not any time synchronization, etc etc). There might be e.g. use cases for transparent forwarding of RTP packets (e.g., by enforced forwarding of UDP packets) which are complemented by monitoring services such as measurement functions, inspections functions or simple event detection functions such as rtp/pltrans. The activation of such an RTP-specific event shouldn’t be in contradiction with the TF topology}

{Editor’s note (2012-04): the necessity of the “TF codepoint” is still questioned because there may be two alternatives already supported:

- H.248.1 basic behaviour by LD/RD settings: “identical media descriptions” imply “transparent media processing” by the MG. That’s implicit semantic in H.248. What’s not really clear is the actual protocol layer of transparent forwarding of Lx-PDUs. Just the application layer PDU or a lower layer

- Transparency media processing may also be enforced by lack of corresponding layer parameters in the LD/RD.

We need firstly a precise definition for the MG behaviour of “Topo-Transparent-Forwarding” mode, and in a second step we may check whether existing H.248 means would be sufficient to control such a behaviour unambiguously. Contributions are solicited}

{Editor’s note (2012-04): there was a discussion which RTP topology supports “RTP packet dejittering” or not? For information, G.799.3 distinguishes between multiple levels of dejittering, which are all relevant for H.248 IP-IP MGs:

- jitter elimination or complete jitter reduction;

- jiter adjustment and

- MGs without any jitter buffer support}

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error Codes

None.

## 10.6	Procedures

### 10.6.1	General

This package allows the MGC to indicate to the MG which particular RTP topology that an RTP stream is under acting under. The type of RTP topology may have impacts on various bearer level functions. It may have an impact on RTCP handling procedures.

In most cases a MG is able to determine itself the RTP topology that is used due to the H.248 Connection Model (i.e. the combination of Terminations/Streams and associated properties in a Context). For example: it can determine the difference between a point to point topology (i.e. a two Termination Context) versus a mixer (i.e. three or more Termination Context). Thus the MGC does not need to signal the "RTP Topology" (rtpt/topo) property.

However in some cases the MG is unable to distinguish between topologies. For example: in some network scenarios it may not be able to distinguish between point to point and translator topologies. This package may be used in scenarios where a more deterministic method is needed to indicate what the RTP topology of a RTP Stream is.

It is expected that Packages extending this package or profiles utilising this package specify what the function specific impact of the RTP Topology property would be.

### 10.6.2	MGC level information for RTP topology control

RTP topology control (at MGC level) could be based on

- call/session control signalling or/and

- local policies.

### 10.6.3	Topology dependent procedures

“MG Determined” indicates the MG shall determine RTP topology and subsequent RTCP handling based on the implicit information derived from the applicable H.248 descriptors. This behaviour may also be subject to profile specification.

#### 10.6.3.1	Topology “Point-to-Point”

The MG shall provide an own, local CNAME.

The MG shall allocate local SSRC values per RTP session endpoint (which corresponds to a H.248 flow pair of RTP media and RTCP control traffic; often identical to the H.248 Stream endpoint (SEP)).

The MG shall originate/terminate RTCP packets of type:

- at least: “SDES”, “SR”, “BR” and “BYE”

[…]

#### 10.6.3.2	Topology “Multicast”

Not used.

#### 10.6.3.3	Topology “Translator”

The MG may provide an own, local CNAME.

The MG shall not allocate local SSRC values for RTP traffic under this topology.

The MG may allocate local SSRC values for supplementary RTCP services.

The MG shall not originate/terminate RTCP packets of type:

- at least: “SR”, “BR” and “BYE”

The MG may originate/terminate RTCP packets of type:

- e.g. “XR”

[…]

#### 10.6.3.4	Topology “Mixer”

Not used.

#### 10.6.3.5	Topology “Video-Switch-MCU”

Not used.

#### 10.6.3.6	Topology “RTCP-terminating-MCU”

Not used.

#### 10.6.3.7	Topology “Asymmetric”

Not used.

#### 10.6.3.8	Topology “Transparent Forwarding”

The H.248 Stream Descriptor could contain RTP/RTCP related information (such as LD-/RD-embedded SDP media descriptions or other H.248 signalling elements related to RTP/RTCP). Such kind of H.248 information could lead to the activation of RTP/RTCP processing functions in the MG.

Property value “TF” of (rtpt/topo) property shall be then used when the MG shall transparently forward RTP packets in order to guarantee deterministic traffic processing.

{Editor’s note (2012-04): The need of this topology is still questioned. See above Editor’s note in clause 10.1.1.}

Bibliography

[b-ETSI TS 183 068]	ETSI TS 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC6222]	IETF RFC 6222 (2011), Guidelines for Choosing RTP Control Protocol (RTCP) Canonical Names (CNAMEs).

[b-IETF ECN]	IETF draft-ietf-avtcore-ecn-for-rtp-0x, Explicit Congestion Notification (ECN) for RTP over UDP.

Appendix I
Living List related to RTP Topology Package

(This appendix does not form an integral part of this Recommendation)

{Editor’s note: This Appendix provides an example RTP Topology Package, which may serve the ongoing discussion on package aspects for such a potential H.248 extensions. There are already a couple of points mentioned, like e.g.

- Stage 2 for H.248, - elaboration of use cases which benefit for an H.248 RTP topology property? E.g., example scenarios:

  - Scenario 1.1: H.248 IP-IP MG may have to provide (due to LD/RD settings) transparent forwarding of RTP packets, which would be done in RTP translator mode. Further on could be H.248.30 (or even more challenging Draft H.248.30) performance monitoring services requested. 
It would be beneficial to indicate the MG explicitly whether it should follow a B2BRE or RTPTT topology, in order to get consistence RTP processing (and possible also related H.248 statistics) in the IP network.

  - Scenario 1.2: RTCP packets related to ECN (note: latest status is the decoupling of RTP topologies and ECN; but statement should be revisited again when RFC is finalized.)

- Number of RTP topologies:

  - identical to RFC 5117;

  - or just a subset, the ones which needs to be discriminated;

  - or even more than RFC 5117 (“because the RFC 5117 RTP topologies are just focusing on the basic and RTP profile dependent RTCP services (see clause 8.3.1 and 8.3.2), but not considering any other RTP/RTCP add-on services which could benefit from more detailed RTP topology models”)).

- Level of RTP topology property: there might be following options

  - Context level (-> new Context Attribute)

          - Pro: RTP topology is a characteristic above the Stream level

          - Con: there might be different, Stream-dependent RTP topologies in case of multiple-Stream-per-Termination connection model

  - Topology descriptor  (-> syntactical or/and semantical extension)
=> not any concrete idea was presented

  - Stream level (-> new Property, see e.g. example below rtpt/rtptopo)

          - Pro: satisfies the requirement of  Stream level RTP topologies

          - Con: the MGC must ensure that all Stream endpoints, related to the RTP topology, must be configured for the same topology. Thus, any invalid signalling combination must rejected by an error code by the MG.

- Scope (semantic) of RTP topology property: could be just limited on dedicated RTCP services (see cl. 8.3), instead of “all”?

- Dynamic of RTP topology property value: there appears the question whether the RTP topology is a static characteristic over the lifetime of a Stream, or whether the RTP topology could even dynamically change? Some possible use cases:

          - Scenario 5.1: a Context is created with Terminations T1 (RTP) and T2 (Phy). T1 is thus in PP topology. Later on T2 is replaced (by SUBtract and MODify) by T3 (RTP), and T1 and T3 should now be connected in TR topology.
=> Transient RTP topology of T1 (PP -> TR).

          - Scenario 5.2: to a Context with two RTP Terminations (T1, T2)  in RTP Media Translator topology would be later a third RTP termination (T3) added, and the media processing should be done in “mixing mode”, thus a MX topology. 
=> again transient RTP topology of T1 and  (TR -> MX).

Contributions are solicited.}

Appendix II
Living List – Discussion of RTP Topologies in case of SRTP

The baseline text is a copy of Appendix II “Sample use-cases of SRTP bearer encryption” from [ITU-T H.248.77]. The copied text is in italics.

Comments are added to the individual use cases.

{Editor’s note (2012-04): see analysis by C-800 and the conclusion that SRTP is one example which is RTP topology dependent.}

This appendix illustrates some network level scenarios that employ SRTP bearer encryption.

## II.1	Use-case #1: ITU-T H.248 MG for peering IP and circuit-switched networks

Figure II.1 illustrates an ITU-T H.248 connection model of (IP, physical). This model is often employed for peering a circuit-switched and an IP network at a residential, access or trunking MG. The RTP session is terminated by the ITU-T H.248 MG. The MG is consequently behaving as a RTP end system (see clause 3 of [IETF RFC 3550]).

Any application of SRTP as a means for media security implies the termination of the SRTP session by the corresponding ITU-T H.248 stream.

Figure II.1 – Use case #1: SRTP to circuit-switched ITU-T H.248 MG

[Comment (from H.248.RTPTOPO perspective]:

This use case is clear, related to the RTPE topology only.

## II.2	Use case #2: ITU-T H.248 MG for peering IP networks

ITU-T H.248 IP-IP MGs are widely used as, e.g., border routers, border gateways, policy enforcement points, firewalls with session-dependent filter rules, NAT devices, media transcoders, etc.

Figure II.2 outlines a scenario, where such a gateway is located between two IP domains: one domain without any media security and another domain using SRTP encrypted media. The ITU-T H.248 MG behaves as two, back-to-back RTP end systems due to the termination of SRTP in one ITU-T H.248 stream.

Figure II.2 – Use case #2: SRTP to RTP ITU-T H.248 MG

[Comments (from H.248.RTPTOPO perspective]:

H.248.77 indicates the B2BRE topology option. This topology model is believed to be the right one due to the SRTP concept of a cryptographic context (cl. 3.2.3/[IETF RFC 3611]). The 3-tuple identifier of the cryptographic context includes the SSRC element. This concludes that the SRTP-to-RTP interworking function by a H.248.77-capable MG implies also the change of the SSRC value. And the change of SSRC identifiers excludes other RTP topologies like e.g. RTPTT.

The RTPTT or RTPMT could be however alternative topologies on a first glance, because just the payload plus some header elements are translated. But it is not expected that the SSRC value could be maintained in the SRTP to RTP scenario.

It should be also noted that the SSRC value is part of the key generation algorithm.

## II.3	Use case #3: Transparent SRTP forwarding

It is possible for an MG to transparently forward SRTP packets, treating them as unprotected UDP or RTP packets. Such a scenario is illustrated in Figure II.3.

As stated in clause 1, this use-case is outside the scope of this Recommendation. It is presented here for the sake of completeness.

Figure II.3 – Use case #3: ITU-T H.248 MG with transparent SRTP forwarding

[Comments (from H.248.RTPTOPO perspective]:

H.248.77 indicates a transparent forwarding topology option. The specific topology is intentionally not detailed by H.248.77, because there are multiple alternative for achieving “transparent SRTP forwarding”:

L4-to-L4 topology (like the indicated UDP-to-UDP topology in above figure, for RTP-over-UDP encapsulation).

  - the MG could be basically L4+ agnostic, i.e. SRTP and RTP unaware

  - the MG might be also SRTP and RTP aware, i.e. knowing that the UDP payload carries SRTP/SRTCP packets

RTPTT topology, e.g. in case that the MG would be requested to check the UDP payload for correct (S)RTP payload and packet types. So called RTP PT value policing: the MG would transparently forward correct SRTP packets and e.g. block other SRTP/RTP traffic. The SSRC would be not touched.

RTPMTlight topology, e.g. in case that the MG would be requested for jitter reduction between to IP networks. Such a function is based on RTP SN and TS processing. Both RTP header fields are not encrypted in case of SRTP. Thus, functions such as jitter reduction would be feasible because independent of RTP encryption or not.

B2BRE topology: this RTP model seems to be questionable due to the concept of the SRTP cryptographic context. Contributions are invited.

Others?

____________________