| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4245 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: DPI Policy Rule Emulation Package |  |  |  |
| Purpose: | Information |  |  |  |

Summary

There are other policy control protocol alternatives besides H.248. However, present support of DPI between these protocols is different. The ITU-T Rw policy control interface should provide functional parity between different protocol alternatives. A correspondent H.248 package is considered for alignment with the Diameter Rw profile.

NOTE: This contribution couldn’t be prepared in due time because of the amount of other contributions. The package design is not yet fully complete. However, we want at least provide the motivation behind this activity.

Motivation

There are different protocol solutions for policy control interfaces, see Fig. 1 for the example of ITU-T RACF Rw reference point. Diameter (Q.3303.3) is one option besides the H.248 Rw profile (Q.3303.2). The H.248 Rw profile is derived and aligned to a certain extend with the ETSI TISPAN H.248 Ia profile. The Diameter Rw profile is not related to TISPAN, but tightly coupled with the 3GPP pendant of a policy control interface, the 3GPP Gx reference point of the PCC (Policy Control & Charging (susbsystem)).

Figure 1 – Protocol alternatives for policy control interfaces (here: Rw from RACS)

The Rw protocol alternatives should provide functional parity, i.e., any policy control application should not be dependent or limited by the selected protocol. However, the initial protocol design between all alternatives was not really equivalent, primarily due the different originating of each protocol (here: Diameter as an AAA related protocol without any bearer scope; H.248 as a gateway control protocol with scope on network protocol stack of bearer traffic).

The Diameter Rw profile did consider (future) DPI policy rules from the beginning (see Dynamic-Firewall-Working-Mode AVP), whereas the H.248 Rw profile did focus on the non-DPI policy rule part firstly (such as H.248.43 defined filter rules or H.248.53 traffic policing rules).

An option for concrete DPI support in Diameter is the 3GPP defined PCC/QoS rule extensions for application detection and control (the “ADC rules”), as already elaborated in the 2012-02 meeting (see AVD-4168).

Policy control alignment, with respect to a DPI capable Rw, could be for instance achieved by an H.248 DPI policy rule emulation package, considering Diameter Gx DPI support.

Figure 2 indicates a typical use case for such an H.248 package: the operation of harmonized “policy control systems” by service/network providers demands for aligned protocol solutions for policy control.

Figure 2 – Chain of vertical policy control interfaces – Need for protocol alignment in case of harmonized “policy control systems” (such as defined by ETSI, ITU-T, 3GPP or MSF)

It may be noticed that the session-based policy decision function (SPDF in Fig. 2) provides a mapping between received policy rules from the vertical northbound interface to the pushed/pulled policy rules at the southbound interface. Such a rule mapping may imply a protocol mapping between Diameter to H.248.

Protocol alignment might be achieved by e.g. an emulation approach: the H.248 protocol emulates Diameter (to a certain extend) in order to simply and minimize any (additional) mapping effort (here: by SPDF).

A H.248 DPI policy rule emulation package would have to provide a signalling object for a DPI policy rule, similar to a correspondent signalling object from Diameter.

H.248 package design – Discussion

The Diameter signalling object of a so called ADC rule is attached to this contribution. The rule structure as such was already evaluated by AVD-4168. It contains primarily a set of possible rule actions, without any support of explicit rule conditions, which are referred by a pointer mechanism.

There are at least two basic package design options: using H.248 Properties per signalling information element; or using H.248 Events (or Signals?). We’ve selected the Event type of package, mainly due to following two aspects:

- Events define conditions for traffic detection in general, - and policy rule conditions would exactly match that aspect; and

- the ObservedEventsDescriptor could be a means for directly binding actions to the rule conditions.

Table 1 provides an high level mapping example between a Diameter ADC policy rule to H.248, using an Event-based package design:

Table 1 – Mapping option between Diameter ADC policy rule to H.248

| ADC-Rule-Definition AVP | Generic information element | H.248 package design |
| --- | --- | --- |
| ADC-Rule-Name | Name of DPI policy rule | Event name |
| TDF-Application-Identifier | Pointer to actual rule conditions | EventsDescriptor parameter (NOTE 1) |
| Flow-Status | Policy rule action related to gate control (NOTE 2) | e.g., embedded Signal |
| QoS-Information | Policy rule action related to traffic policing (NOTE 3) | e.g., embedded Signal |
| Monitoring-Key | Identifier for policy rule actions, related to usage parameter control across an aggregated traffic structure | EventsDescriptor parameter (NOTE 4) |
| Redirect-Information | Policy rule action related to traffic redirection | EventsDescriptor parameter (NOTE 5) |
| — | Policy rule action related to reporting the result(s) of policy rule enforcement (NOTE 6) | ObservedEventsDescriptor parameter |
| NOTE 1 – E.g., similar as property “Pointer to rule conditions” from Inspection rule pointer package. NOTE 2 – Relation to StreamMode and H.248.43 properties to be clarified. NOTE 3 – Relation to H.248.53 properties to be clarified. NOTE 4 – TBD. NOTE 5 – Network address plus associated information. NOTE 6 – This type of information is outside of the ADC rule AVP, but could be directly coupled in H.248 by the ObservedEventsDescriptor. |  |  |

Summary

A high level design for a possible H.248 DPI policy rule emulation package is presented.

Change proposal

None.

Attachment – Excerpt from ETSI TS 129 212:

…

5.3.87	ADC-Rule-Definition AVP

The ADC-Rule-Definition AVP (AVP code 1094) is of type Grouped, and it defines the ADC rule sent by the PCRF. The ADC-Rule-Name AVP uniquely identifies the ADC rule and it is used to reference to an ADC rule in communication between the PCRF and the PCEF within one IP-CAN session. The TDF Application Identifier AVP(s) determines the traffic that belongs to the application.

If optional AVP(s) within an ADC-Rule-Definition AVP are omitted, but corresponding information has been provided in previous Sd messages, the previous information remains valid.

Monitoring-Key AVP contains the monitoring key that may apply to the ADC rule.

AVP Format:

ADC-Rule-Definition ::= < AVP Header: 1094 >

{ ADC-Rule-Name }

[ TDF-Application-Identifier ]

[ Flow-Status ]

[ QoS-Information ]

[ Monitoring-Key ]

[ Redirect-Information ]

*	[ AVP ]

___________________

Authentication, Authorization and Accounting; positioned as a replacement of RADIUS

such as statefull inspection, DPI mode

see H.248.37 latching Signal, which is actually more of type Event than Signal