| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4254 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Update proposal for Appendix II |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update the examples of use cases (Appendix II).

List of proposed changes:

- change of sequence of use cases and example call flows: renumbering Appendix I to II (and vice versa in AVD-4253);

- new use case with DTLS/UDP to non-TLS/TCP interworking.

NOTE: this contribution correlates with AVDs 4253 and 4297, and the WebRTC proposal by AVD-4274/4275.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4232, the input document for this meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix I

Sample use-cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use-case #1: Terminal-to-MG TLS session, provisioned TLS service negotiation

This use case illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain
{Editor’s note: MGC is shown as being outside of the  the bearer TLS-domain, although the MGC might need to provide the MG with  information about the applicable TLS-domain.}

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by SIP/TLS/TCP.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS

- Bearer path: Application/TLS/TCP/IP

- UE: TCP- and TLS-client, MG: TCP- and TLS server (NOTE: this assumption could be satisfied with SDP “a=setup:” attribute)

- H.248 MG TCP-mode: TCP-proxy.
{Editor’s note: Any other TCP-modes and potential transitions between different TCP-modes are to be studied. See also work on H.248.TCP}

- Mutual TLS-authentication via certificates, MG’s certificate is signed, UE’s certificate is self-signed

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG

Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG

## I.2	Use case #2: Terminal-to-MG TLS session, MGC-controlled TLS service negotiation

This use case is an extension of the first use case (clause II.1) by allowing the MGC to influence the TLS service negotiation at bearer level. Use case #1 is based on preconditions which are partially subject of network provisioning. Use case #2 is motivated by the fact that there are a number of reasons which demand for a MGC involvement, such as to bound the TLS service parameters, to limit the negotiation time, etc.

{Editor’s note: this clause will be updated when the design and definition of H.248 packages for TLS will be more mature …}

## II.3	Use case #3: WebRTC to NGN/IMS interworking function with DTLS-to-TLS support

To be inserted.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Bibliography

[b-IETF rtcweb-data]	IETF draft-ietf-rtcweb-data-channel (2012), RTCWeb Datagram Connection.
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-data-channel/

[End Proposed Correction]

____________________