| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4266 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal for clause I.9 (use case #8) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clause I.9.

Discussion

The editor’s note request more descriptive text. A correspondent proposal is made.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4234, the input draft to this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

## I.9	Use case #8: MP relocation by loopbacking

The relocation of a MG-local MP to the remote RTP source endpoint might be beneficial under certain conditions (see also clause I.5). Such a MP relocation could be principally achieved by enabling a loopback point (LP), see Figure I.8.

Figure I.8 – Use case #8: MP relocation by loopbacking

This use case is even more advanced than the one of clause I.5 because the MP is relocated to a previous MG (here BGW node Nk-1), instead of the actual “loopback source” (i.e., OP of UEA here).

General discussion and evaluation of loopbacking in the context of performance monitoring:

- Prerequisites for loopbacking

- The “loopbacked media” must contain all necessary information as required by the performance metric(s). This requirement may be denoted as “location-independent measurement parity”.
Here: the measurement results (of the MP’s) at locations Nk-1 (with loopback) and Nk (without loopbacking) must be identical.

- This condition may be fulfilled for some metrics, but not the plethora of all envisioned RTCP XR defined metrics. The specific measurement algorithm of a performance metric determines the principal applicability (of MP relocation) or not.

- Drawbacks of loopbacking

- There is essentially a doubling of transport capacity at the loopback path.

- Motivation for MP relocation by loopbacking

- Cost factor of MG-local MP (“too expensive for node Nk“), see also clause 8/[ITU-T H.248.79];

- Congestion factor, e.g. temporary unavailability of sufficient measurement capacity;

- Implementation factor, e.g., still lacking measurement capability;

- RTCP factor, e.g., a missing “RTCP XR” interface would prevent the reporting of local measurements, whereas loopbacking just requires the usual RTP interface (“LP in favour of RPRTCP”).

- MP relocation to “previous RTP node” (here node Nk-1), instead of RTP source endpoint (here: UEA)?

- Background: The relocated MP could be “somewhere” in the loopback path. There are thus two principal options in this use case (node Nk-1 or UEA).

- The option of node Nk-1 is actually out of scope of [IETF LOOPBACK], but principally feasible in network scenarios with interim H.248 IP-IP gateways (as above illustrated);

- The “node Nk-1” option provides following advantages:

e.g. mitigate the basic drawback of increased network transport capacity, or to the example of lacking measurement capability in UE;

  - actually, the loopback service might be not visible at all for the UE, when node Nk-1 would act as a “virtual loopback source” in above scenario.

Thus, as an overall conclusion: MP relocation by loopbacking is not a scalable option at network level (see also [IETF LOOPBACK]), but might be beneficial and justified under specific conditions.

[End Proposed Correction]

____________________

The “loopbacked media information“ is primarily subject of the enforced loopback mode: [IETF LOOPBACK] supports the three basic modes of "encapsulated packet loopback", "direct loopback", and "media loopback". These are distinguished by the treatment of incoming RTP packets at the loopback mirror.