| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4268 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.82: Appendix II progress |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to Appendix II.

Discussion

There are some pending discussion items related to Appendix II, which are in scope of this presentation:

- Qualitative vs quantitative considerations

- Temporal horizon of control loop (“time scale considerations”)

- Mono- vs multimedia type of ETS calls (see editor’s note).

Figure 1 tries to summarize the coverage of the examples discussed by Appendix II.

We think that we got already a sufficient number of typical use cases.

Figure 1 – Inventory of Appendix II/H.248.82 (Status: 2012-05)

With regards to 1 and 2:

- we’d like to suggest to add a Note to clause I.1, clarifying the scope of the Appendix

With regards to 3:

- additional example is proposed

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4239, the input draft to this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix II

Possible interaction between ECN and ETS

(This appendix does not form an integral part of this Recommendation)

## II.1	Introduction

This interaction issue between ECN and ETS can be summarised by the question whether ECN could positively or negatively influence ETS traffic (“Can ECN improve or degrade ETS traffic?”). Any (individual call) enabled congestion control (for IP bearer traffic) definitely has a direct impact on the IP application (here: ETS (for Emergency Telecommunication Service)). Whether the impact is negative or positive, - from perspective of the perceived/experienced quality (i.e. QoE) by the ETS user -, is firstly not straightforward. A disabled congestion control also affects the ETS service quality because (temporal and/or spatial) congestion situations in a network normally decrease the grade of service (GoS) of the entire IP bearer traffic (“of a congested network area”), under the assumption of a best effort infrastructure.

Some examples are evaluated.

NOTE – The evaluations are purely qualitative; any quantitative statements (such as concrete traffic mixes, likelihood of a use case in reality, absolute QoS/QoE objectives, results of simulation studies, time scale of control loop, etc) are out of scope of this Recommendation.

## II.2	Use case examples

The examples provide qualitative considerations. Complementary quantitative evaluations would rather be the subject of network engineering and associated performance investigations (e.g., by traffic simulation studies).

### II.2.1	Example 1: Networks with "significant ETS traffic"

There is basically a mix of ETS and non-ETS traffic. In this case, it is assumed that there is significant ETS traffic.

Evaluation:

An ECN-enabled ETS call could experience a better QoE/QoS than an ECN-disabled ETS call due to a positive effect of an enabled congestion control.

NOTE - The portion of ETS traffic, under normal load conditions, is typically limited in real networks. See e.g. [b-ETSI TS 122 153] for MPS traffic in 3GPP networks (“… MPS traffic volumes should be limited (e.g. not to exceed a regional/national specified percentage of any concentrated network resource, …), so as not to compromise this function.”).

### II.2.2	Example 2: Terminal with two-mode-multirate codec

A terminal with a generic two-mode-multirate codec provides following two operational modes:

- codec 1 "high quality mode" with 60 kbit/s transport capacity and

- codec 2 "good quality mode" with 20 kbit/s transport capacity requirement.

The codec furthermore supports a sender-driven congestion control:

- any notified network congestion would lead to a transition to the 20 kbit/s mode (“fallback to lower transport capacity”).

Consider the following example scenario:

  - The call starts in 60 kbit/s mode and experiences (in a later point in time) a mean packet loss ratio of e.g. 5 % due to network congestion.

  - ECN is enabled and network congestion leads to ECN marking.

  - Received ECN indications at sender side activate the sender-driven congestion control method,

  - There is then a transition to the 20 kbit/s mode.

  - This mode then experiences e.g. a mean packet loss of 0.1 %, hence a lower information loss!

  - There are consequently two situations with different QoE as perceived by the user, labelled as QoE60 and QoE40:

    - the QoE60 in the “high quality” codec mode could actually be worse than the QoE40 in the “low quality” codec mode, under network congestion, because the GoS40 would be better than the GoS60 (primarily due to relations “IPLR40 << IPLR60“ and “IPDV40 << IPDV60“).

Evaluation:

An ECN-enabled ETS call could experience a better QoE/QoS than an ECN-disabled ETS call due to the possible positive side effect that the lower transport quality may receive a better GoS.

### II.2.3	Example 3: Network with "sparse ETS traffic"

A network situation with a low portion of ETS traffic, e.g., when “ETS traffic is much less than non-ETS traffic”.

Evaluation:

The ECN-disabled ETS calls could benefit from the ECN-enabled non-ETS calls due to a potential positive effect of more available transport capacity from non-ETS traffic and a reduced overall congestion level (because the congestion control could reduce the consumed bitrate by non-ETS calls).

### II.2.4	Example 4: Limited terminal capability

Let’s consider an (hypothetical) IP terminal equipment with ECN sender-driven congestion control, but without any signalling capability of ECN information. The implemented IP protocol stack may provide ECN support at all possible protocol layers, but the application level logic (“ECN user instance”) may be not available, which does not allow to process ECN information at the IP interface.

Such a terminal limitation prevents a closed control loop (see Fig. 1), - because either would be the congestion notification path interrupted, or the feedback path, or both.

Evaluation:

Disabling ECN for ETS would not harm in such a situation because ECN marking wouldn't really help. ECN-disabled ETS calls might be thus a basic strategy.

### II.2.5	Example 5: Multimedia ETS calls with media type individual ECN configurations

Multimedia ETS calls with media components audio, video and text. An ECN sender-driven congestion control may trigger

- complete deactivation of a specific media component;

- reduction of audio quality; and

- reduction of video quality.

All three control reactions should lead to a reduced IP transport capacity for the ETS call, but still allow sufficient end-to-end communication.

Evaluation:

Enabling ECN for ETS might be beneficial for multimedia calls, given that at least one media component allows sufficient communication.

## II.3	Observations

It generally expected that enabling ECN for non-ETS calls, which normally constitute the biggest part of the served traffic, will have a positive service impact on ETS calls, due to the bandwidth potentially yielded by non-ETS calls.

Enabling ECN for ETS calls is subject to the following considerations:

- ECN-enabled ETS calls: this approach may have a positive service impacts in ETS calls, and thus be justified, if the following conditions apply:

  - Amount of served ETS traffic is significant.

  - ETS terminals have the capability to change to lower codec modes in response to ECN congestion indications, thus lowering their bandwidth consumption.

- ECN-disabled ETS calls: this approach may be justified, in other words there is no obvious positive service impact in ETS calls, when any of the following conditions apply:

C1:	Amount of served ETS traffic is much less than the amount of served non-ETS traffic;

NOTE - This condition may apply to the vast majority of the network scenarios.

C2:	No support of sender-driven congestion control for terminals in “ETS” mode.

[End Proposed Correction]

____________________