| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4270 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Supp.Prio: Editorial improvements for Appendix I |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution addresses the pending action in revising the “contribution style” text of Appendix II into a generic format.

If the text revisions would be agreed, then there would be subsequent updates required of the References/Bibliography and Abbreviations.

A new subclause was inserted in order to provide the missing background assumption of the original clause I.2. The material is based on a contribution to the May 3GPP meeting.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 794 (WP 2/16), the output draft from last Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix I

H.248 profiles and Context Attributes for priority traffic handling

The purpose of this Appendix is an inventory concerning signalling support for priority traffic handling.

## I.1	Status H.248 Profiles

H.248 profile definitions include Context Attribute support information (see profile definition template in [ITU-T H.248.1]). There are in the meanwhile many H.248 profile specifications published by some standard development organizations (SDO). In the context of this Supplement are of particular interest profiles for H.248 IP-IP gateways; such kind as the 3GPP-defined profiles for their H.248-based reference points Ix and Iq (according [ETSI TS 129 238] and [ETSI TS 129 334]; which are both based on ETSI TISPAN Ia profile [ETSI TS 183 018]).

The following table summarizes the status for Ix and Iq:

Table II.1 – 3GPP Profiles and typical support of Context Attributes (status: 2012)

| Context Attribute | Supported | Values Supported |
| --- | --- | --- |
| … |  |  |
| Priority Indicator | NO (NOTE) | 0-15 |
| Emergency Indicator | Yes | YES/NO |
| IEPS Call Indicator | No | NA |
| … |  |  |
| NOTE: This Context Attribute parameter is allowed in ETSI TISPAN Ia Profile version 3. |  |  |

It may be noted that there is actually support of emergency calls only in pure 3GPP environments, and additional priority level support in ETSI TISPAN instantiations.

## I.2	Discussion of principal H.248 signalling options

H.248 supports basically the provisioning of protocol parameter values, which might be sometimes an alternative to the usual signalling of protocol values. There are further different kind of H.248 decomposed gateway models, such as bearer-independent or bearer-dependent style of operation (see clauses 3.2.2 and 3.2.1). The alternatives lead to various options which are outlined in this clause, using the concrete example of DiffServ-based QoS marking as selected technology for priority traffic support.

### I.2.1	Direct signalling: QoS marking codepoint signalled via "native H.248 format"

H.248 signalling (i.e., ADD and MODIFY request commands) carry the native DiffServ codepoint (as e.g. assigned for priority traffic). The MG directly uses that DiffServ codepoint for QoS marking outgoing bearer packets. There are two variants from MGC perspective, see Figure I.1:

Figure I.1 – Controlling of MG-level QoS marking function – Variants 1 and 2

Some observations:

- Variant 1 is based on pure signalling without provisioning needs.

- Variant 2 is typically used when call control signalling either does not support signalling of DiffServ values at all, or when local DiffServ mapping is applied.

### I.2.2	Indirect signalling: QoS marking codepoint indirectly signalled via H.248

The DiffServ codepoint for a particular packet bearer traffic class might be also provisioned at MG level (see Figure I.2).

Figure I.2 – Controlling of MG-level QoS marking function – Variants 1 and 2

Some observations:

- Variant 3 does not require the explicit signalling of DiffServ codepoints at the H.248 interface. Another H.248 protocol element could be used instead.

- Variant 3 needs to take into account that an H.248 profile might support in additional direct DiffServ signalling, which may lead to a mix of both signalling options, and hence the careful consideration of possible interaction issues.

## I.3	IEPS/ETS traffic support: question of signalling solutions?

There are different signalling alternatives, as outlined by the previous clause, in order to satisfy a requirement for IEPS/ETS traffic support:

- Native approach: the direct signalling method based on the explicit H.248 signalling element for that purpose, the ContextAttribute IEPS call indicator.

- Emulation approach: the indirect signalling variant by reuse of existing signalling capabilities, which would relate here to the support of ContextAttribute Priority indicator for IEPS/ETS.

Discussion:

Option (A) looks pretty straightforward, but there’s a catch with this approach: this ContextAttribute was just introduced with H.248.1 Version 3, which is often not yet supported by existing H.248 profiles as published by SDOs.

For instance, there is reluctance (e.g., in case of the examples of clause I.1) to upgrade from H.248.1 Version 2 (as the core protocol baseline for these profiles) to Version 3, just due to a single additional protocol element. However, the introduction of H.248.1 V3 might be basically mitigated when a stepwise addition of V3 capabilities would be considered (“instead of the all or nothing approach”). Appendix III providesr a more detailed discussion of such a phased proceeding.

[End Proposed Correction]

____________________