| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4276 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Supp.Prio: Clause 3.2, terminology added as used in Appendix II |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes term definitions for clause 3.2, which are yet missing but used in Appendix II.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 794 (WP 2/16), the output draft from last Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

## 3.2	Terms defined in this Supplement

3.2.1	Bearer-dependent decision point (synonym to technology-dependent decision point): the MGC as the primary decision point with respect to the mapping of call control level information to gateway control signalling and the concerned H.248 context, termination and/or stream entity as bearer level element. The MGC exhibits all necessary information about bearer plane protocol stack, supported QoS technologies, etc.

NOTE 1 – This is the usual model in the master-slave relationship of H.248 gateways.

NOTE 2 – The notion of “technology-dependent …” / “technology-independent …” originates from the ITU-T Resource and Admission Control Function (RACF) architecture (see [ITU-T Y.2111]).

3.2.2	Bearer-independent decision point (synonym to technology-independent decision point): this is in contrast to clause 3.2.1, the MGC is still the primary decision point with respect to the mapping of call control level information to gateway control signalling but is lacking partially or fully bearer related information. The MGC applies therefore some kind of generic signalling and delegates the final mapping to the MG.

NOTE 3 – The notion of “bearer-independent …” originates from the Bearer Independent Call Control (BICC) architecture (see ITU-T Q.19xx-series of Recommendation); applied e.g. in the H.248 profile “Bearer independent call bearer control protocol” according [ITU-T Q.1950].

[End Proposed Correction]

____________________