| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4288 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei technologies CO., LTD |  |  |  |
| Title: | F/H.TPS-Arch: Six Screen Telepresence Room Design |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes the addition of an example of a six screen Telepresence Room Design to Rec. F/H.TPS-Arch.

# Introduction

Presently F/H TPS-Arch includes three exemplary room layouts.  One illustrates a three screen telepresence room with one row of participants; the other illustrates a three screen telepresence room with two rows of participants.  At the last ITU-T Q.5/16 meeting, a new room design with four screens was proposed, discussed and accepted.

We propose the addition of a six screen room design.

# Discussion

The six screen example is distinguished from the current examples in several respects:

- Use of a long table.

- Use of an additional wall projector.

- New arrangement for cameras.

- Two rows of screens.

- Use of screens with narrow frame borders.

The use of two rows of screens allows for display scenarios not possible with a single row of screens. This is particularly useful in scenarios with more than two end points in a call. For example the following scenarios are possible:

- The simultaneously display of two sites, with one site shown in the first row and the other in the second row;

- Four adjacent screens, e.g. the left two and the middle two, displaying a scaled image of the current speaker, and the remaining two screens displaying in life size;

- All the screens displaying switched speakers, behaving like a switched video wall. Note: this is possible in a single row. An additional row allows for more participants

- The upper row displaying active speakers while the lower row displays presentation and auxiliary information;

- The upper row displaying the local site while the lower row displays the remote site.

The addition of this example would better illustrate the breadth of possible room designs in particular it would illustrate that multiple rows of screens are possible.

# Conclusion

It is proposed that the attached text be added to of F/H.TPS-Arch.

Attachment
Proposed text

### 9.1.4 Single row seating 6-screen telepresence system

{Contributor’s note: It is also proposed to introduce “seating” into the other sub-headings under section 9.1 to minimize confusion between rows for screens and seating.}

The following figures depict the room layout of a 6-screen telepresence system, with the following characteristics:

–	Three main cameras and six screens with narrow frame borders.

- Three screens in one row, two rows of screens.

- Main cameras placed between thescreens of the bottom and top row.

–	Three regions for participants in the room, and each region containing two or three seats.

–	Each main camera captures images of one region.

–	One wall projector displays a selected presentation/stream.

As can be seen in Figure 19, a set of three screens are arranged in one row. Each screen shows the far end participants at their true size. Three main cameras are placed at eye level and sandwiched between two rows of screens; each camera is placed in the centre of the bottom frame of the higher screen and in the centre of the top frame of the lower screen.

Camera 1

Camera 2

Camera 3

Screen 1

Screen 2

Screen 3

Screen 4

Screen 5

Screen 6

Figure 19: Screen and camera layout for Six-screen Telepresence

The use of two rows of screens allows for other display scenarios not possible with a single row of screens. This is particularly useful in scenarios with more than two end points in a call. For example the following scenarios are possible:

- The simultaneously display of two sites, with one site shown in the first row and the other in the second row;

- Four adjacent screens, e.g. the left two and the middle two, displaying a scaled image of the current speaker, and the remaining two screens displaying in life size;

- All the screens displaying switched speakers, behaving like a switched video wall.

Note: this is possible in a single row. An additional row allows for more participants

- The upper row displaying active speakers while the lower row displays presentation and auxiliary information;

- The upper row displaying the local site while the lower row displays the remote site.

As shown in Figure 20, each main camera captures a region of two (or three) seats along a table. Presentation screens and microphones are mounted on the long table. An additional wall projector is provided to display desired streams/presentations upon user demand.

Since screens with narrow frame borders are used, the displayed images need more manipulations to obtain a continuous view. So image processing methods, such as cropping and scaling, are required to provide better visual experience when interoperating with screens with broader frame borders.

{Contributor’s note: Image processing is also required when adjacent capture regions overlap.}

Camera

Projector

Seat

Screen

Microphone

Presentation screen

Region A

Region B

Region C

Figure 20: Six-screen Telepresence room layout

_____________________