| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4296 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | New work item: H.248.TCP "Gateway Control Protocol: TCP support packages" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes the start of a new work item (called H.248.TCP) with regards to the support of dedicated H.248 signalling elements for TCP bearer traffic.

1	Introduction

This item is a spin-off activity from work on

- H.248.43: control of filter capabilities for TCP traffic

- H.248.84:

  - control of TCP modes

  - NAT-T procedures related to TCP

- H.248.TLS: control of TCP connections, which are a precondition for TLS session establishment

- H.248.WEBRTC: interworking of WebRTC data traffic, such as non-TCP-to-TCP scenarios

It turned out that more explicit control means for influence of TCP connection control might be beneficial, not in general, but in some situations.

The considered H.248 control capabilities are primarily related to:

- Explicit assignment of TCP client/server roles to an H.248 SEP

- Explicit control of “cut-through” behaviour

- Initiation of TCP connection state dependent H.248 activities

- Explicit capability in separated bearer connection (here TCP) establishment and bearer session (such as TLS) establishment

- Explicit capability in reuse of TCP connections

2	Motivation for separate H.248.TCP work item

The work item is driven by following considerations:

- Separate H.248.x-Recommendation?

Initially we considered a tight coupling of TCP connection control together with TLS session control, hence H.248.TLS as an appropriate place. However, both functionalities are basically disjoint, particularly (considered) “H.248 TCP support package(s)” might be useful for non-TLS environments as well.

- Why explicit H.248 signalling elements at all?

We think that many TCP scenarios will not need any explicit TCP connection control support; the MG just knows the correct behaviour concerning TCP role assignment, correct TCP connection establishment/release procedures towards remote TCP entities, etc.

Such a MG mode is often termed as “MG autonomous behaviour” with the main benefit in offloading the MGC of “bearer specific control activities”.

The considered H.248.TCP signalling elements would provide tools for “MGC strictly controlled behaviour” of the MG.

Thus, there might be two options for individual TCP applications. Or specific scenario demand for explicit H.248.TCP support.

Example:

TCP connection control for TLS-to-non-TLS interworking might be achieved with both options. An H.248 profile could just mandate MG autonomous behaviour, leading to a basic TCP connection establishment/release service.
Additional support of H.248.TLS would be optional, e.g. in order to support advanced services.

- New concept?

Actually not really. The work on networks with separate planes for calls and bearers, - bearer-independent call control (BICC) architecture -, got to consider support of connectionless and connection-oriented packet bearer technologies by H.248 MGs leading to generic bearer connection control tools.

- Existing H.248 packages?

Possibly some of the (BICC) Q.1950 defined H.248 packages should be cross-checked. However, there are two issues:

1st BICC/Q.1950 did not consider bearer type “TCP”, hence lacking TCP specific information.

2nd H.248.TCP is primarily considered to be positioned in “non-BICC network infrastructures”, such as NGN/IMS or peer-to-peer networks.

- TCP support by existing H.248.x-Recommendations?

There is already something available by H.248.84. H.248.TLS should then provide a pointer to correspondent TCP support tools in other H.248.x-Recommendations.

3	Scope of work item: H.248.TCP

Following items should be in scope:

- Control models: description of the two H.248 control options “MG autonomous behaviour” and “MGC strictly controlled behaviour” with respect to the

  - TCP client/server role assignment

  - TCP mode assignment

  - establishment and release of TCP connections

  - reuse of idle TCP connections

  - cut-through variants

  - and other TCP bearer aspects

Note: the distribution of “bearer control intelligence” between MGC and MG is not black/white as possibly concluded by both behaviour models. What’s actually meant with “MGC strictly controlled” is also intentionally still open at the beginning of such a work item.

- Use cases: identification of use cases for each control model

- Control model assignment to MG: the applied control model might be a MG-level or Stream-/Termination-level capability in case of TCP applications (as opposed to other services with two control options support such a T.38, which are pure at MG level).
The relevant control behaviour might be specified by an H.248 profile, implicitly derived by the MG from H.248 signalling, or explicitly controlled via e.g. a new property.

- TCP connection control: identification of a simplified state model, - related to an individual H.248 SEP or the entire Context? -, due to the assumption of statefull H.248 control behaviour for TCP.
Does the MGC need to maintain the bearer states, or not?

- Generic bearer connection control versus bearer-dependent: consideration possibly benefit of a generic package plus technology specific extension packages (e.g.. motivated by the fact of separated state models for TCP connections and superior TLS sessions in case of TLS applications)?

- Definition of H.248 package(s) for support of TCP.

- Relation to other H.248.x-series Recommendation with scope on TCP: description / link

- others

4	Proposal

It is proposed to accept this new work item for Q.3/16. Parallel contribution AVD-4297 provides a first idea of a TCP connection control package (item 4 in above list).

It has to be noted that both contributions were more or less last minute activities, hence still incomplete in their text.

Annex A
Proposed new ITU-T Recommendation H.248.TCP

| Question: | 3/16 | Proposed new ITU-T Recommendation | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Recommendation ITU-T H.248.TCP "Gateway Control Protocol: TCP support packages" |  |  |  |  |
| Base text: | Contribution AVD-4296 |  | Timing: | 2013 |  |
| Editor(s): | Jens Guballa, ALCATEL-LUCENT, Jens.Guballa@alcatel-lucent.com |  |  |  |  |
| Scope: |  |  |  |  |  |
| Following items should be in scope: - Control models: description of the two H.248 control options “MG autonomous behaviour” and “MGC strictly controlled behaviour” with respect to the - TCP client/server role assignment - TCP mode assignment - establishment and release of TCP connections - reuse of idle TCP connections - cut-through variants - and other TCP bearer aspects Note: the distribution of “bearer control intelligence” between MGC and MG is not black/white as possibly concluded by both behaviour models. What’s actually meant with “MGC strictly controlled” is also intentionally still open at the beginning of such a work item. - Use cases: identification of use cases for each control model - Control model assignment to MG: the applied control model might be a MG-level or Stream-/Termination-level capability in case of TCP applications (as opposed to other services with two control options support such a T.38, which are pure at MG level). The relevant control behaviour might be specified by an H.248 profile, implicitly derived by the MG from H.248 signalling, or explicitly controlled via e.g. a new property. - TCP connection control: identification of a simplified state model, - related to an individual H.248 SEP or the entire Context? -, due to the assumption of statefull H.248 control behaviour for TCP. Does the MGC need to maintain the bearer states, or not? - Generic bearer connection control versus bearer-dependent: consideration possibly benefit of a generic package plus technology specific extension packages (e.g.. motivated by the fact of separated state models for TCP connections and superior TLS sessions in case of TLS applications)? - Definition of H.248 package(s) for support of TCP. - Relation to other H.248.x-series Recommendation with scope on TCP: description / link - others |  |  |  |  |  |
| Summary |  |  |  |  |  |
| This Recommendation – describes H.248 control options for TCP bearer traffic; – specifies H.248 signalling elements and procedures for explicit TCP connection control; – … |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |  |
| - BICC framework: H.248 packages defined by SG11 as part of Q.1950 - H.248.84, H.248.TLS, H.248.TLSPROF |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| Possibly with 3GPP. |  |  |  |  |  |
| Supporting members that have committed to contribute in an active manner to this work item: |  |  |  |  |  |
| Alcatel-Lucent |  |  |  |  |  |

_______________________

see parallel contributions AVD 4274 and 4275