| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4302 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | F.TPS-Reqs: Natural response with lip-sync for telepresence services |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This contribution makes a comment on the section 1 (Lip-sync) of TD898rev1(GEN/12) to COM16-LS300.   SG12 replied that requirements on one-way delay for telepresence services were still under study, but it introduced a contribution submitted to SG12 in 2007 as a reference.
The contribution concluded that 500ms delay might be acceptable for natural response, and 100ms might be acceptable on the time-gap between audio and video for lip-sync.  However, MOS score of the 500ms delay was less than 4.0, and video quality used for the evaluation was QCIF (174x144) and 15fps.  Q5/16 should review those conditions.

This contribution suggests that Q5/16 employs better score on QoE (e.g. better than MOS=4.0 or better than R=90), and better video quality such as HD720p/1080p and 30fps, when Q5/16 studies this delay issue for telepresence services.

# Introduction

SG12-TD898rev1 is a response to COM16-LS300.  The issue on lip-sync was still under study but the response introduced one reference (COM 12-C119rev1).  This C119rev1 was submitted to SG12 in Sep/2007 and described two speech conversation tests and one AV conversation test.
Conclusions of the COM12-C119rev1were;

The results of a conversation test depend to a great deal on the call scenarios. There is a need for more realistic scenarios to test the influence of delay. Many of the conversation scenarios suggested in the literature are not very sensitive to delay. In a normal conversation you reflect on what the other person said and think about what to reply, so there is usually a natural delay before the response. For our test scenarios a delay time around 500 ms can be accepted, while packet loss influenced the conversation quality to a great extent.

For audio-visual conversations it is more important to synchronize the audio and video than to have a short delay. Usually video is delayed more than audio, so generally for e.g. video telephony, audio should be delayed to obtain audio-video synchronization, at least for delays shorter than 600 ms.

In the future we would also like to verify the results with normal phones in a more realistic environment, not only by using a PC client in a lab environment.

Those conclusions can be read that 500ms might be acceptable for natural response, and 100ms (time-gap between 500ms and 600ms) might be acceptable for lip-sync.

# Discussion

## Acceptable delay for natural response

ITU-T G114 has recommended “less than 150ms” for natural response.  The E-model rating R on the 150ms is better than R=90 (around 93).　On the other hand, COM 12-C119rev1 reported that “500ms is acceptable (if 0% packet loss)”, but MOS was less than 4.0 (around 3.75).

According to G107 Table B.1 (shown below), “less than 4.0 of MOS” means “less than R=80 of E-model”.   Here, “less than R=80” means that some users dissatisfied.

Q5/16 should recognize that “150ms” and “500ms” are measured based on different acceptable-range of QoE.   If an acceptable R of E-model is specified as better than 90, one-way delay is up to 150ms, and if the acceptable R is better than 80, one-way delay becomes longer and up to 500ms.

It is suggested that Q5/16 determines a range of acceptable R (of E-model) for video conferencing.

G114 Figure 1 								COM 12-C119rev1 Figure 1

Effects of absolute delay by the E-model			Total conversation quality in MOS vs E2E 										delay time for different packet losses

G107 Table B.1 – Provisional guide for the relation between R-value and user satisfaction

| R-value (lower limit) | MOSCQE (lower limit) | GoB (%) (lower limit) | PoW (%) (upper limit) | User satisfaction |
| --- | --- | --- | --- | --- |
| 90 | 4.34 | 97 | 0 | Very satisfied |
| 80 | 4.03 | 89 | 0 | Satisfied |
| 70 | 3.60 | 73 | 6 | Some users dissatisfied |
| 60 | 3.10 | 50 | 17 | Many users dissatisfied |
| 50 | 2.58 | 27 | 38 | Nearly all users dissatisfied |

## Acceptable time-gap on lip-sync

Past studies of broadcasting industries including ITU-R suggested the time-gap between audio and video was less than 50ms (actual figures were 33ms, 40ms, or 45ms).

On the other hand, the contribution of SG12 in 2007 concluded that 100ms might be acceptable for lip-sync.  But it indicated that 15fps video was used for the evaluation.  Capturing interval of 15fps is 66ms.  This means that the test was not able to measure the effect of less than 66ms.

It is suggested that Q5/16 employs HD (720p or 1080p/i) and 30fps when Q5/16 measures QoE on lip-sync for telepresence services.

# Conclusion

The following figure is basically Fig.1 of COM16-C837 submitted to Q5/16 in May/2012.

Conclusions of the contribution (COM 12-C119rev1) are added to make differences clear.

This contribution suggests the followings to Q5/16.

1) Q5/16 should determine an acceptable range (e.g. higher than R=90) of E-model when Q5 specifies acceptable one-way delay for natural response.

2) Q5/16 should specify the video quality used for QoE tests when Q5 specifies acceptable time-gap for lip-sync.

__________________