| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4305a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | F.TPS-Reqs: Adding a typical use case of presenting viewgraphs |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a use case to display presentation-viewgraphs provided by a laptop computer of a presenter’s endpoint.  It is proposed to add this use case to current Section 6.4 (Use case 4: Presentation) of F.TPS-Reqs.

Also two issues related to presentation are discussed. One is an interoperability issue and the other is an end-user’s operation issue when end-users wish to display presentation-viewgraphs.

# Introduction

## Adding a typical use case of presenting viewgraphs

Figure 4 of Section 6.4 of F.TPS-Reqs is indicating two icons, which are presentation captures and presentation screens.    This is one use case of presentation. (Fig.1 of next page)

Nowadays, almost all of presentations are done with viewgraphs (presentation slides).  In general, the viewgraphs are provided by a laptop computer (Fig.2 of next page) or stored in a control unit.

## Multi-vendor interoperability on presenting viewgraphs

There are several ways to display viewgraphs.  One is to add a new stream used for the presentation.   The other is not to add, but to embed a viewgraph into an existing video stream.

As far as multi-vendor interoperability is concerned, no system-action is better after multi-streams for main telepresence views are established.

For business conferences/meetings today, off-conference-distribution (e.g. by attachments of emails) is used.  Viewgraphs may not be needed to display on a large size in a screen.  If participants can identify which page (viewgraph) is presented at the point of time, the view must be enough.  Details of the viewgraphs can see, for example, in a laptop computer.

## Unified end-user operating procedure on presenting viewgraphs

Video conferencing sometimes stops when a presenter is not familiar on a switching operation of a screen or an activating operation of an additional presentation screen.  One of solutions is to unify end-user operating procedure on presenting viewgraphs among multi-vender telepresence systems.

Fig. 1 (a copy of Figure 4 of section 6.4 of F.TPS-Reqs)

Two-screen

Telepresence endpoint

IP Network

Three-screen

Telepresence endpoint

Presentation capture

Presentation

screen

Presentation capture

Presentation

screen

Three-screen

Telepresence endpoint

Fig.2 (a proposed figure to add)

Personal Computer

Two-screen

Telepresence endpoint

IP Network

Three-screen

Telepresence endpoint

Three-screen

Telepresence endpoint

Personal Computer

# Discussion

## Adding a typical use case of presenting viewgraphs

The Fig.2 is the most popular use case of video conference systems.  Therefore, it is proposed to add as an independent figure, instead of just adding icons (laptop computers) to the original Figure 4 of section 6.4 of F.TPS-Reqs.

## Multi-vendor interoperability on presenting viewgraphs

There are four ways to transport additional video (or still image) streams to remote endpoints. (Fig.3 below)

1) In-stream and In-screen
	  2) In-stream and Out-of-screen (note: This may not be used due to inefficiency.)
	  3) Out-of-stream (adding a new stream) and In-screen
	  4) Out-of-stream (adding a new stream) and Out-of-screen

From the point of view of interoperability, “In-stream/In-screen” is the surest way because no control signals are needed.  There are two ways to control viewgraphs at encoding-endpoint.  One is switching, and the other is mixing at the encoding-endpoint.

Note: The simplest way is switching, but it loses continuous presence.

Note: Dual presentations from different endpoints are also possible by this “In-stream/In-screen”.

It is suggested to address the way of the “In-stream/In-screen” to add description of the Use Case 4 on presentation.

Fig.3

## Unified end-user operating procedure on presenting viewgraphs

This subject is a part of discussion on the other contribution on “unified user operating procedure”, submitted to Q5/16 meeting in Sep/2012, and will be discussed with the contribution.

# Proposal

1) Figure 2 of this contribution is added to Section 6.4 Use Case 4 of F.TPS-Reqs as “Figure 4-2”.

Note:  The existing Figure 4 becomes Figure 4-1.

2) The following text is added, associated to the Figure 4-2 above.

Presentation viewgraphs are provided by a local computer (e.g. a laptop computer) of an endpoint where a presenter is.   The viewgraphs can be distributed to remote endpoints by either adding new steams or embedding (or switching) viewgraphs in an existing video stream.

__________________