| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4307 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | TPS-Reqs/Arch: Unified user operating procedures for telepresence |  |  |  |
| Purpose: | Discussion/Proposal |  |  |  |

Summary

This contribution discusses three issues to enhance reusability of end-user’s operation experiences on telepresence system.  This is aiming for promoting use of telepresence systems further.

The issues are (1) unified end-user operating procedures, (2) unified terminology displayed on end-user terminals, and (3) unified addressing schemes for end-users to set up a video conference.

Note: (1) is a current living list item of F.TPS-Reqs, and (3) is indicated as one of end-user’s issues but it is not planned to be discussed in detail here.

# Introduction

## Unified end-user operating procedures

Table 1 shows where this subject is focused on, that is the left of the table.

One-touch operation or one-touch button is a matter of how to minimize end-user’s actions.  One-touch button is to integrate functions executed simultaneously by the one action, but it is basically not to unify functions to be integrated.

## Unified terminology displayed on end-user terminals

If multi-vendor systems use different terminology displayed on terminals for the same operation or function, the terminology is one of obstacles for end-users to operate the system without reading manuals.   Even if end-user operating procedures are fully unified, they may not be enough to help end-users because of different terminology used.

## Unified addressing schemes for end-users to set up a video conference

Several addressing schemes are used for video conferencing.  E164 (decimal digits), IPv4 (decimal digits separated by dot), and URI (alphanumeric digits separated by dot) are used to specify a remote endpoint, an MCU, a gateway, or a service provider.  Basically it is better for end-users (without technical knowledge) to unify the addressing scheme.  However, these different addressing schemes depend on technology used (or standards like H320 or H323).

# Discussion

## Unified end-user operating procedures

Pre-setting is possible when remote endpoints or MCUs are fixed.  However, open environment (global access environment) should be concerned in order to promote use of telepresence systems.

The unified user operating procedures (or standardized user interactions) make user’s operation experience reusable among different telepresence systems, once the user learns system operations.

Either step-by-step operation or one-touch operation with input-menu can be used for such open access environment.  Regardless of types of operations, operating procedures should be unified for end-users to enable reuse of their operation experience.

Table 2-1 to Table 2-6 indicate outlines of end-user’s basic operating procedures which are expected to be employed among telepresence systems.

## Unified terminology displayed on end-user terminals

It is suggested to use same terminology (displayed on terminals) for the same operation or function, in order to promote use of video conferencing under multi-vendor environment.

Furthermore, technology-oriented terms, such as MCU, Gateway, H323, SIP, IP are not helpful for general end-users (no technical knowledge) to use systems.  As long as end-users need to ask a system- administrator to set up connections, video conferencing will not be used as widely as the industry expects.  Examples of non-technology-oriented terms are;
 - to reserve an MCU can be replaced by “to reserve a virtual conference room”, and 
 - to access Cloud-MCU is can be replaced by “to call video conference service center”.

Figure 1 shows an example of end-user interactions based on terms of virtual conference rooms.  Terms to be unified need to be discussed further.

## Unified addressing schemes to set up a video conference

The addressing scheme depends on technology (or standards H320/ H323/SIP or WEB-oriented) used for video conferencing.  Both H323/SIP-based video conference systems basically use IPv4 (decimal digits separated by dot) address to specify remote end-points or MCUs.   And global IPv6 (hexadecimal digits separated by colons) is expected to be introduced soon.

The realistic solution is to employ address conversion at a gateway or an MCU instead of unifying them.  As an alternate solution, end-user terminals of telepresence systems may have a capability to accept all addressing codes and to identify destinations, such as remote endpoints, MCUs, or gateways (by accessing public DNS if needed).

# Conclusion

This contribution describes clarifications for the living list item 1 of F.TPS-Reqs.

The scope of the unified operating procedures is shown by Table 1 and concrete examples are also shown by Table 2-1 to Table 2-6.

The best way for end-users is that all of user’s operations and operating procedures are fully unified.  But this contribution proposes a minimum set of such requirements from the point of view of a practical approach.  A minimum set is proposed, but it is expected that those unifications promote use of telepresence further.

[Proposal] In order to unify user operating procedures, the minimum set of procedures needs to be specified. Table 2-1 to Table 2-6 are proposed to be an Annex or living list items of F/H.TPS-Arch.

As for unified terminology, especially non-technical terminology for end-user’s operations, further contributions are expected.

# Tables and figures

Table 1

Figure 1

Table  2-1

| Join/Enter |  |
| --- | --- |
| ↓ |  |
| Activate the terminal | Turn on power or activate from a sleep state |
| ↓ |  |
| [Input/Dial a User ID code] | When specified by an admin |
| [Input/Dial the password] |  |
| ↓ |  |
| Input/Dial the destination address | A remote endpoint, a local MCU, a gateway, or an ASP is given in advance |
| ↓ |  |
| [Input/Dial a user ID of ASP] | When accessing an ASP |
| [Input/Dial the authorization code] |  |
| ↓ |  |
| Input/Dial a conference ID | Given in advance |
| Input/Dial the authorization code | Given in advance |
| ↓ |  |
| [Input/Dial a user ID] | If charging individually |
| [Input/Dial the password] |  |
| ↓ |  |
| [Input/Dial rendering position codes] | When asymmetrical screens |
| ↓ |  |
| (On conference) |  |

Table 2-2

| Leave/Exit or Close |  |
| --- | --- |
| ↓ |  |
| Click/Press/Hooking for leaving |  |
| ↓ |  |
| [Input/dial a release code of MCU] | Asked whether releasing the MCU used (if a moderator) Releasing the MCU used is to close the conference at the point of time. |
| ↓ |  |
| [Input/dial a chair-change code] | Asked when not releasing the MCU used. |
| ↓ |  |
| Click/Press/Hooking for confirming | Asked the confirmation by the system |
| ↓ |  |
| Deactivate the user terminal | Turn off power or turn to sleep state |

Table 2-3

| Switch to/from continuous presence |  |
| --- | --- |
| ↓ |  |
| Click/Press/Hooking for switching |  |
| ↓ |  |
| Input/dial a screen code to be switched | The screen code is specified by two decimal digits, XY. The X is to specify a screen, and the number is allocated from the left-edge screen (X=1). The Y is to specify one of remote endpoints, and the number is allocated to the default order for continuous presence. (The left-edge of default positions is Y=1) The code 00 (zero-zero) is to specify switching back to continuous presence to all of displays with default order of remote endpoints. Note: This is local numbering and independent from Section 7 of H.243. |
| ↓ |  |
| Click/Press/Hooking for confirming | Asked the confirmation by the system |
| ↓ |  |
| (On conference) |  |

Table 2-4

| Activate presentation (when adding a new media stream) |  |
| --- | --- |
| ↓ |  |
| [no stream for other presentation] | It is assumed that the system will not accept two distribution streams for presentation simultaneously. |
| ↓ |  |
| Click/Press/Hooking to activate presentation | Activate (add) a new distribution stream for presentation and connect to a device storing viewgraphs. |
| ↓ |  |
| Click/Press/Hooking for confirming the distribution of viewgraphs | Asked the confirmation by the system when the additional stream is set up successfully. |
| ↓ |  |
| Click/Press/Hooking for finishing the presentation | Release the distribution stream for presentation. |
| ↓ |  |
| (On conference) |  |

| Activate presentation [alternate] (when embedding in a main stream) |  |
| --- | --- |
| ↓ |  |
| Click/Press/Hooking to activate presentation | Activate a local mixer to embed viewgraphs in a main media stream and connect to a device storing viewgraphs. |
| ↓ |  |
| Click/Press/Hooking for confirming to add a viewgraph in a screen | Asked the confirmation to use default size and location of a viewgraph in a screen. Note: An advanced end-user can change the default setting of size and location temporarily by inputting/dialing a request code. |
| ↓ |  |
| Click/Press/Hooking for finishing the presentation | Release the local mixer for presentation. |
| ↓ |  |
| (On conference) |  |

Table 2-5

| Reserve/Book conference resources |  |
| --- | --- |
| ↓ |  |
| Activate the terminal | Turn on power or activate from a sleep state |
| ↓ |  |
| [Input/Dial a User ID code] | When specified by an admin |
| [Input/Dial the password] |  |
| ↓ |  |
| Input/Dial the server address | A local server address or an ASP address is given in advance |
| ↓ |  |
| Input/Dial a user ID |  |
| Input/Dial the authorization code |  |
| ↓ |  |
| Input/Dial information requesting | Date, Time and duration, Number of endpoints, Chair’s endpoint code, Recording request, etc. |
| ↓ |  |
| [Input/Dial a communication quality selection code] | When end-users have choices on communication quality (wish very stable or do not care) Note: technically Managed-QoS network or Best effort network. |
| ↓ |  |
| Input/Dial the confirmation code | When asked the confirmation |
| ↓ |  |
| Input/Dial the confirmation code | When a conference ID and its authorization code are given, Additionally when charging information is given. |
| ↓ |  |
| [Input email addresses] | If the moderator of the conference requests the server or ASP to send emails to participants, in order to inform the conference information. |
| ↓ |  |
| Deactivate the user terminal | Turn off power or turn to sleep state |

Table 2-6

| Change or Cancel conference resources |  |
| --- | --- |
| ↓ |  |
| Activate the terminal | Turn on power or activate from a sleep state |
| ↓ |  |
| [Input/Dial a User ID code] | When specified by an admin |
| [Input/Dial the password] |  |
| ↓ |  |
| Input/Dial the server address | A local server address or an ASP address is given in advance |
| ↓ |  |
| Input/Dial a user ID |  |
| Input/Dial the authorization code |  |
| ↓ |  |
| Input/Dial information to change or cancel | Date, Time and duration, Number of endpoints, Chair’s endpoint code, Recording request, etc. |
| ↓ |  |
| Input/Dial the confirmation code | When asked the confirmation |
| ↓ |  |
| Input/Dial the confirmation code | When a conference ID and its authorization code are changed. |
| ↓ |  |
| [Input a request of re-sending emails] | If the moderator of the conference requests the server or ASP to send emails to participants, in order to inform the conference information. |
| ↓ |  |
| Deactivate the user terminal | Turn off power or turn to sleep state |

__________________