| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4308 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | F/H.TPS-Arch: Delay due to mixing streams at an MCU |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This contribution is to discuss how to distribute multiple media streams (either video streams or audio streams) without mixing at an MCU (Multipoint Control Unit).  The aim of not mixing at the MCU is to avoid any increase of end-to-end delay.  The delay increased by mixing will have affect on natural response with lip-sync.  Therefore no mixing of streams at an MCU is suggested for telepresence systems.

# Introduction

Multipoint video conferencing uses an MCU.   Main functions of the MCU are to mix multi-streams from multi-points into a single stream and also distribute streams to destination points (other remote locations).

There are two technical issues for mixing media streams.  One is that delay is increased due to processing time for decoding and re-encoding at the MCU, and the other is that original spatial positions are lost when multiple streams are mixed into a single stream.

This contribution focuses on the delay issue.

# Discussion

In order to provide natural response with lip-sync to end-users of telepresence systems, Q5 should consider how to minimize one-way delay.   Telepresence systems employ higher compression techniques, and the higher compression generally makes one-way delay longer due to longer processing time for coding and decoding.

Basically mixing multiple media streams into one media stream at an MCU is to decode each stream once and to encode again. This process at the MCU increases the one-way delay.  (Fig.1)

No mixing at the MCU, that is no decoding and no re-encoding, is desirable for lower end-to-end delay. (Fig. 2)

As for telepresence, no mixing is also effective to maintain spatial position of each AV stream.

Q5 should review benefits of no mixing at the MCU for multi-point conferencing using telepresence systems.  Technically mixing at the MCU is reasonable, but from the point of view of QoE, the mixing at the MCU is not appropriate due to making end-to-end delay increase.

Fig. 1

Fig. 2

It is possible to distribute without mixing as shown in Fig. 2, but downstream bandwidth (bit rate) of an access line needs more than double that of a single stream bandwidth.   If the bandwidth of access lines is not enough to transport multiple media streams, encoding bit rate needs to be reduced.

Examples to discuss this mixing issue of 3-screen telepresence systems are shown in Fig. 3-1, Fig. 3-2, Fig. 4-1, and Fig. 4-2.

Note: When media streams are mixed at the MCU, profile conversion is also possible.  This profile issue on interworking will be discussed by another contribution.

# Conclusion

This contribution is pointing out mixing issues at MCUs.   The issue relates to delay for natural response with lip-sync.  End-user’s view (QoE) should be given higher precedence to promote use of telepresence.  No mixing at a MCU is suggested, but it involves bandwidth issue of access lines.

As for interworking with legacy conference systems which have been designed for a single media stream, the mixing at the MCU may be needed.  The mixing issue needs to be reviewed under heterogeneous system environment.

# Examples

Fig. 3-1: Video streams from two remote locations are mixed at an MCU.  Three 2-to-1 mixers are equipped for each location.  Downstream-bandwidth to decoders is the same as upstream-bandwidth from encoders.

Fig. 3-2: No mixing at all at an MCU.  Spatial positions of streams can be reconstructed at a decoding end.  But downstream-bandwidth to decoders is double of upstream-bandwidth from encoders.

Fig. 3-1							   Fig. 3-2

Fig. 4-1: Location C uses a legacy system.  One stream from Location C are mixed with one of three streams of telepresence systems at an MCU (two 2-to-1 mixer) and 6 streams from Location A and B are mixed at the MCU (one 6-to-1 mixer).  Downstream-bandwidth to decoders is the same as upstream-bandwidth from encoders.

Fig. 4-2: As for interworking with legacy system (Location C), one 6-to-1 mixer is used for reducing bandwidth, but no mixing is used for between telepresence systems.   Delay issue remains only for interworking between telepresence systems and the legacy system.

Fig. 4-1							 Fig. 4-2

(Supplement)

Followings are basic figures of “mixing” and “multiplexing”. This contribution focuses on a mixing issue.

__________________