| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4312 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | Electric power consumption of telepresence systems |  |  |  |
| Purpose: | Information and discussion |  |  |  |

Summary

This contribution provides information on power consumption of (1) a meeting using telepresence systems and (2) a face-to-face meeting that participants move to a conference room by electric vehicles.  This is information to discuss a study item (of Q5/16) on climate change.

# Introduction

One of study items of Q5/16 is to consider how to further enhance telepresence systems to mitigate negative impact on climate change and to encourage a positive impact reducing greenhouse gas (GHG) emissions.

Past reports on GHG (especially CO2) emissions often described that a video conference system would be an effective tool reducing such gas emitted by transport systems.

Q5/16 has not discussed this environmental issue yet, although it is one of study items.

Telepresence systems employ multiple large screens to enhance immersive image.  They consume electric power larger than conventional single-screen systems.  This means that telepresence systems tend to increase negative environmental impact.  End-users of telepresence systems have to consider the trade-off issues, either more immersiveness or more greenness (energy saving).

Today, an automobile industry is making a great effort to develop electric vehicles which are expected to reduce GHG emissions.

This contribution is aiming for providing information to compare;

- power consumption (estimated) of multi-screen telepresence systems with

- power consumption (latest data) for charging batteries of electric vehicles used for moving to a location where a conference will be held .

# Information

## Power consumption

A key difference between a video conference and a face-to-face conference/meeting is;

- Video conferencing: Power consumption increases proportionally by meeting hours.
- F2F conference: Power consumption does not depend on meeting hours.

In the F2F meeting/conferencing, power consumption increases with the number of participants who use electric vehicles and driving distance to/from a venue of conference.

## Power consumption of telepresence systems (estimated)

Typical power consumption of a single screen (x1) and a three screen (x3) system are shown below.

Data used for estimating power consumption are as follows.

Note: In order to build an economical system, COTS*-based components are selected.
* Commercially available Off-The-Shelf

Camera: 5W 		COTS auto-focus HD camera, handy size but fixed to a display

Display: 200W		COTS 60 inch LED HD display with speakers (both sides of the display

Microphone: 3W	With an echo-cancelling feature

Speaker: (attached to a display)

Control unit	: 160W	COTS quad-core desk top computer with COTS graphical processing unit

GPU: 75W/unit		Graphical processing unit (GPU) when two and more displays

Power consumption estimated for typical models are:

A system with 1 display:	   380W (200x1 +20 +160)
A system with 3 displays:	1,010W (200x3 +25 +160+75x3)

Note: High-end products of telepresence systems consume much larger amount, but this study uses reasonably smallest amount (to be a bottom line).

## Energy consumption of electronic vehicles (CO2 emission per km)

## A case study to compare power consumption

Participants are 2 persons (Location A), 4 persons (Location B) and 4persons (Location C).

As for F2F meeting, 6 persons (Location A and C) move to Location B using 6 electric vehicles.

Electric power consumption (kWh) of the case above is as follows.

- Video conferencing: Power consumption of the conference room except that of room-lights

- F2F conferencing: Discharged electric power of in-vehicle-battery to transport a driver to and from the conference venue.

# Discussion

The case study informs us;

1) Negative impact on climate change due to use of telepresence systems, when;

- the conference uses 3-screen telepresence systems ,
 - the conference is for more than 3 hours, and 
 - offices of all of participants are located within 4km.

2) Positive impact on climate change due to use of telepresence systems, when;

The distance between offices to be concerned is not 40km but 4km (so short), when we compare the energy consumption between telepresence systems and F2F meeting moved by electric vehicles in 2012.

It must be reasonable to assume that participants moves more than 10km when a F2F meetings is held.  So we can still say that the use of 3-screen telepresence systems provides positive impact on climate change.

However we should not ignore that 1-screen systems are much better as far as the positive impact on climate change is concerned.

Battery performance of electric vehicles will improve.  If a drastic improvement occurs in the future, 3-screen telepresence systems might then provide negative impact on climate change in comparison.

Q5/16 should study how to reduce power consumption of telepresence systems.

For example,

- how to promote (as a requirement) an activation of sleep modes of all of components at once at an endpoint, when the conference has just finished

- how to promote a development of energy efficient CODECs used for video conferencing as a real-time interactive application

It is expected to be discussed whether Q5/16 studies further in next study period of ITU-T.

# Conclusion

This contribution is aiming for activating discussion on one of study items if this Q5/16.

__________________