| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4325 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE, China Telecom |  |  |  |
| Title: | H.VSMarch: Proposed updates on adaptable transcoding function |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes some updates on adaptable transcoding function for multiple types of mobile customer unit in mobile visual surveillance system to the ITU-T Rec. H.VSMarch.

# Introduction

In the ITU-T Rec. H.VSMarch, VAU is used to implement the communication between M_CU and the units defined in ITU-T Rec. H.626, such as distributing and transcoding multimedia data. It requires VAU to support media transcoding for different M_CUs which locate in different networks with various bandwidths such as GSM, CDMA, WCDMA and Wi-Fi etc, or have different device capabilities.

# Discussion

When M_CU requires video/audio stream from VAU, it can provide the network type within the request. According to the network type provided by M_CU, VAU queries the corresponding transcoding parameters from SCU. VAU requires the media stream from MDU and performs transcoding based on the parameters, and then delivers the transcoded media stream to M_CU. SCU can maintain and manage the transcoding parameters including resolution, bit rate, encoding type and I-frames interval.

The current version of H.VSMarch does not describe the aspect on M_CUs locating in different types of networks. To make the description more clear and complete, this contribution proposes to add new content about media transcoding based on different network types in clause 7.

# Conclusion

# It’s proposed to accept the following updates to H.VSMarch.

Start proposed content

## Functional Entities

……

### VAU

Video Access Unit (VAU) is the key unit to implement the control between mobile terminals and other units defined in Recommendation ITU-T H.626.The main functions of VAU include: Service control function, media control function (such as PTZ control command transmit), media processing function (such as multimedia transcoding), configuration management function.

Figure 7-2: Function components of VAU

The VAU provides media transcoding function for M_CUs locating in different types of networks; it acquires pre-configuration of transcoding parameters for various mobile networks from SCU. The transcoding parameters include resolution, bit rate, encoding type and I-frames interval

7.3.3	 M_CU

The M_CU is the functional entity within mobile VS system, such as mobile phones used by the visual surveillance end users. It is used to present multimedia information (such as audio, video, image, alarm signal, etc.) to the end user. It implements the following functions:

- Requesting media stream from VAU, based on the network types which the M_CU is locating.

- Multimedia decoding and playback.

- PU list, real-time image/video presentation.

- Console interface for end user to operate the VS system (such as PTZ control).

……

## Reference Points

……

### 7.4.5 Reference Point Vs: VAU -SCU

The reference point Vs is between the VAU and the SCU.

The Reference Point Vs is used by SCU to control VAU registration, access authentication, authorization, accounting.

- Transferring the PTZ Control operation commands from the M_CU by VAU to SCU.

- Delivering control signals between the VAU and the SCU in order to create a media session between PSS and MDU.

- Querying the transcoding parameters from SCU according to the network type the M_CU locates in.

……

## Service Control Flow and Signalling

Editor’s note: This clause will describe the basic service procedure flow and the corresponding signalling and protocols different from those flows in ITU-T H.VSarch, the subsection category can be modified based on contributions in future meeting.

### Real-time Video/Audio Acquisition

When users want to view the real-time surveillance video/audio by mobile terminals, after MSP performs authentication and returns VAU information to M_CU, M_CU choose and request to the VAU for real-time video/audio.

Figure 7-3: High level procedural flows for real-time Media Acquisition

This figure shows the procedural flows for real-time Media Acquisition for one M_CU:

- First the M_CU initiates a request and creates connection with the MSP.

- The MSP initiates a request and creates connection with the CMU.

- The CMU returns the VAU information.

- The MSP gets the request and returns VAU information to M_CU.

- When the M_CU gets VAU information, it requests the VAU for the real-time media address with the network type the M_CU locates in.

- VAU receives this request and requests to SCU for the media address. At the same time, VAU queries the corresponding transcoding parameters from SCU.

- The SCU transfers this request to the MDU.

- After receiving messages, the MDU returns the real-time media address to the SCU.

- The SCU returns the real-time media address to the VAU.

- After receiving the real-time media address, the VAU requests the MDU to start transmitting media stream.

- The MDU sends a request to the PU for initiating a media flow.

- The PU creates media channel with the MDU and sends media flow to the MDU.

- The MDU transfers the media flow to the VAU. VAU performs transcoding according to the transcoding parameters.

- The transcoded media flow is transferred from the VAU to the M_CU. The M_CU decodes and plays the received media flow.

- When the M_CU stops the real-time media acquisition it sends a stop-media request to the VAU.

- VAU stops transcoding after receives the request.The VAU stops sending media stream to the MDU.

- The MDU sends response to the VAU for stopping media flow.

- The VAU requests the SCU to disconnect which then returns a response to the VAU.

- Then the VAU ends this real-time media acquisition and sends response to the SCU.

### Recording and playback

The playback function is used to play audio and video which has been previously recorded and stored.

There are three locations at which recorded media are stored, in platform, in the PU and in the M_CU. With media stored in the M_CU, it plays the recorded media just like playing any local media file. As additional interaction with other parts of the VS system is not needed, the following procedural flows will not include this simple method.

#### Playback in platform

When users want to playback the surveillance point by mobile phones, they must log in the MSP by the M_CU. the MSP performs authentication and returned the playback surveillance list.

After the users choose one surveillance point name, the M_CU sends a request for this surveillance video to the VAU, and then VAU returns the media stream to the M_CU to play.

Figure 7-4: High level procedural flows for playback in platform

This figure shows the procedural flows for playback in platform:

- The M_CU sends a request to the MSP for querying recording information.

- The MSP sends a request to the CMU for querying recording information.

- The CMU sends a request to the MSU for querying recording information.

- The MSU returns the querying result to the CMU.

- When the CMU gets the result, it transfers the result to the MSP.

- When the MSP gets the result, it transfers the result to the M_CU.

- The M_CU selects one playback point and starts a request to playback.

- The MSP sends a request for the playback address to the CMU.

- The request is transferred from the CMU to the SCU.

- Then the request is transferred from the SCU to the MDU.

- The MDU responses to the request and sends the playback address back to the SCU.

- The playback address is sent from the SCU to the CMU.

- The CMU sends the playback address to the MSP.

- The playback address is sent from the MSP to the M_CU.

- The M_CU starts playback request for the VAU with the playback address and the network type.

- The VAU starts playback request for the MSU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

- The MSU returns a response to the VAU and sends the media stream to the VAU.

- Once the VAU gets the media stream, it performs transcoding according to the transcoding parameters and then transfers the transformed stream to the M_CU.

- The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receives the request.

- The VAU sends a stop playback request to the MSU.

- As a response, the MSU stops sending stream.

- At the same time, the MSU notifies the MDU to stop playing.

- The MDU returns a response to the MSU.

#### Playback in PU

Figure 7-5: High level procedural flows for playback in PU

This figur shows the procedural flows for playback in PU:

- The M_CU sends a request to the MSP for querying recording information.

- The MSP sends a request to the CMU for querying recording information.

- The CMU sends a request to the PU for querying recording information.

- The PU returns the querying result to the CMU.

- When the CMU gets the result, it transfers the result to the MSP.

- When the MSP gets the result, it transfers the result to the M_CU.

- The M_CU selects one playback point and starts a request to playback.

- The MSP sends a request for the playback address to the CMU.

- The request is transferred from the CMU to the SCU.

- Then the request is transferred from the SCU to the MDU.

- The MDU responses to the request and sends the playback address back to the SCU.

- The playback address is sent from the SCU to the CMU.

- The CMU sends the playback address to the MSP.

- The playback address is sent from the MSP to the M_CU.

- The M_CU starts playback request for the VAU with the playback address and network type.

- The VAU starts playback request for the PU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

- The PU returns a response to the VAU and sends the media stream to the VAU.

- Once the VAU gets the media stream, it performs transcoding based on the transcoding parameters and then transfers the transformed stream to the M_CU.

- The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receiving the request.

- The VAU sends a stop playback request to the PU.

- As a response, the PU stops sending stream.

- At the same time, the PU notifies the MDU to stop playing.

- The MDU returns a response to the PU.

End proposed content

_____________________