| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4336a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Proposal of revised text of H.VHN (Service Capabilities and Framework for Virtual Home Network) drafted with TD 37 (WP 2/16) |  |  |  |
| Purpose: | Proposal |  |  |  |

Abstract

This contribution proposes the revised text of H.VHN (Service Capabilities and Framework for Virtual Home Network). The revised text of the contribution is based on TD 37, which has been developed at the meeting, Geneva, 20 - 24 February 2012.

Draft Recommendation H.vhn

Service capabilities and framework for virtual home network

AAP Summary

[TO BE ADDED BEFORE CONSENT]

Summary

This Recommendation describes service requirements and framework architecture for Virtual Home Network. In this Recommendation use cases of virtual home network framework and related service capabilities are provided.

Keywords

Virtual Home Network.

CONTENTS

1	Scope	6

2	References	6

3	Definitions	6

3.1	Terms defined elsewhere:	6

3.2	Terms defined in this Recommendation	7

4	Abbreviations and acronyms	7

5	Conventions	7

6	Introduction	8

7	VHN service requirements	8

7.1	General requirements	8

7.2 	Configuration management	8

7.3 	Security	8

7.4 	Fault management	9

7.5 	Mobility	9

7.6 	Multicast	9

8. 	VHN service framework architecture	9

8.1	VHN service control function (VHN-SCF)	10

8.1.1	VHN membership management function	10

8.1.2	VHN multiservice control function	10

8.1.3	VHN service support function	11

8.2	VHN architectural considerations	11

8.2.1	Application level  VHN service provision	11

8.3	Federation capabilities	11

8.4	Security for VHN	11

9	Management functionality for virtual home network	12

10	Use case of virtual home network	12

10.1	P2P based virtual home network service	12

10.2	Overlay network based VHN service	13

10.3	Community service capability of VHN	14

10.4	Federation use case of virtual home network	15

Appendix I  Virtual home networking capability to provide IPTV service over NGN service overlay network	16

Appendix II  Federation capability of VHN: use case of IPTV service	18

Appendix III  Example of application-level home gateway function	20

Appendix IV  Example of Middleware functions for virtual home network	21

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 8-1: Framework architecture for VHN services based on [ITU-T H.622] 10 Figure 8-2: VHN architecture which supports application-level home gateway functions 11 Figure 10-1: A model for P2P based VHN 13 Figure 10-2 A model for overlay network based virtual home network 14 Figure 10-3 A model for community service based virtual home network 15 Figure I-1: IPTV VHN service over NGN service overlay network 16 Figure II-1: Example of IPTV virtual home community service: Home network federation 18 Figure II-2: IPTV Service architecture for virtual home community service 19 Figure III-1: Example of Application-level home gateway function 20 Figure VI-1: UPnP home network system architecture for integrated virtual home network 21

# Scope

This Recommendation identifies service requirements and framework architecture for the support of virtual home network (VHN) services in fixed and wireless network domains as well as at different network levels including intra-access network, inter-access network, and inter-network levels. This includes mobility support at the service level and the transport level.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.622]	ITU-T Recommendation H.622(2008), A generic Home Network architecture with support for multimedia services.

[ITU-T H.622.1]	ITU-T Recommendation H.622.1(2008), Architecture and functional requirements for home network supporting IPTV services.

[ITU-T G.9970]	ITU-T Recommendation G.9970(2009), Generic Home Network Transport Architecture.

[ITU-T J.190]	ITU-T Recommendation J.190(2007), Architecture of Media Home Net.

[ITU-T Y.2012]	ITU-T Recommendation Y.2012, Functional Requirements and architecture of the NGN Release 1.

[ITU-T Y.2201]	ITU-T Recommendation Y.2201, Requirements and capabilities for ITU-T NGN release 1.

[ITU-T Y.2000]	ITU-T Recommendation Y.2000 series – Supplement on NGN release 1 scope.

[ITU-T Y.2291]	ITU-T Recommendation Y.2291, Architecture overview of next generation home network.

[ITU-T Q.1706/Y.2801] ITU-T Recommendation Q.1706/Y.2801 (2006), Mobility management requirements for NGN

[ITU-T X.111]			ITU-T Recommendation X.1111, Framework of security technologies for home network

# Definitions

## Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

3.1.1	 access network [ITU-T Q.1742.1]: Network that connects access technologies (such as a Radio Access Network) to the core network.

3.1.2	home network [ITU-T H.622]: A home network is the collection of elements that process, manage, transport and store information, thus enabling the connection and integration of multiple computing, control, monitoring, communication and entertainment devices in the home.

3.1.3	federation [ITU-T Y.2720]: Establishing a relationship between two or more entities or an association comprising any number of service providers and identity providers.

3.1.4	 mobility [ITU-T Q.1706/Y.2801]: The ability for the user or other mobile entities to communicate and access services irrespective of changes of the location or technical environment.

3.1.5	 mobility management [ITU-T Q.1706/Y.2801]: The set of functions used to provide mobility. These functions include authentication, authorization, location updating, paging, download of user information and more.

3.1.6	 personal mobility [ITU-T Q.1706/Y.2201]: This is the mobility for those scenarios where the user changes the terminal used for network access at different locations. The ability of a user to access telecommunication services at any terminal on the basis of a personal identifier, and the capability of the network to provide those services delineated in the user's service profile.

3.1.7	 quality of service (QoS) [ITU-T G.1000]: The collective effect of service performances, which determine the degree of satisfaction of a user of the service.

3.1.8	 service continuity [ITU-T Y.2201]: The ability for a moving object to maintain ongoing service over including current states, such as user’s network environment and session for a service.

3.1.9 	terminal mobility [ITU-T Q.1706/Y.2201]: This is the mobility for those scenarios where the same terminal equipment is moving or is used at different locations. The ability of a terminal to access telecommunication services from different locations and while in motion, and the capability of the network to identify and locate that terminal.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1 	VHN (Virtual Home Network): A VHN is a logical home network, built over public and/or private network resources, used to support home network services within a group of home members.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DNG | Delivery Network Gateway |
| --- | --- |
| NGN | Next Generation Network |
| VHN | Virtual Home Network |

# Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

# Introduction

The support of virtual home network (VHN) services in fixed and wireless network including mobile environment allows controlled and secured communication services using home network resources within a group of service users.

Types of VHN services include:

- Site-to-site VHNs that link another home networks, home members, and communities of interest;

- Access VHNs that accommodate mobile home members and telecommuters allowing them to securely access their home network from outside the home;

This recommendation builds upon the generic home network architecture with support for multimedia services in [ITU-T H.622].

# VHN service requirements

This clause addresses the requirements for VHN services when provided in sit-to-site and access types including mobile environment. Figure 7.1 illustrates VHN service configuration model in terms of user’s movement and service request at DNG. For example, as shown in figure 7.1, when a user of VHN move one place to another, the service mobility will be requested at the DNG. In this clause, other service function will be discussed in the preceding sub-cluases.

Figure 7-1	VHN service configuration model

## General requirements

For the support of home network services, VHN management function (e.g. delivery network gateway (DNG) at home network) [ITU-T H.622] is required to provide:

- support for fixed and mobile users;

- means for home network user to specify VHN membership;

- accommodation of user-defined VHN addressing schemes (e.g. IP or non-IP addressing);

- transparency to home network user data;

- means for single customer site to belong concurrently to more than one VHN;

- provision for multi-protocol support;

- provision of standards-based interfaces (independent of user's device supplier);

- means to meet user's security requirements (e.g. in terms of different security levels);

- provision of VHN members with secure dynamic access to VHN (e.g. via dial-up);

## Configuration management

VHN gateway (e.g. DNG at home network) is required to provide:

- configuration capabilities for VHN members at both service level (e.g. dynamic creation and modification of VHNs, according to specific application requirements and/or user requirements);

- consistency and coherence verification of VHN user configuration information;

- ability to easily change topology;

- ability to easily add, remove or modify VHN related devices, sites etc.;

- ability to accommodate growth requirements for VHN related devices, sites, etc.

## Security

VHN is required to support mechanisms for:

- controlling (fixed or mobile) user access to the VHN via access control mechanisms (identification, authentication and authorization of (fixed or mobile) users accessing the VHN);

- ensuring the privacy of data being transported by the VHN;

- secure key distribution;

- secure and efficient SLA negotiation;

- security policies to satisfy users and/or providers’ security requirements (e.g. to adjust the configuration of VHN users or VHN groups, to reflect trust relationships).

## Fault management

VHN is recommended to provide:

- information to the VHN customer in event of service disruption and restoration;

- dynamic VHN recovery, non-disruptive and not perceived by the VHN users;

- provisioning of relevant VHN incident reports and summaries.

## Mobility

It is recommended to support:

- nomadism for personal (i.e. VHN user) and terminal (i.e. VHN user terminal) mobility

- service continuity for terminal mobility

- service continuity  in case of VHN voice services.

## Multicast

VHN is recommended to support:

- multicast capabilities within a VHN, e.g. to support specific applications such as linear TV within a VHN.

- creation, operation and release of multicast groups within a VHN;

- mechanisms to manage multicast connections within a VHN (i.e. all multicast trees within that VHN);

- mechanisms allowing VHN members to join and leave multicast groups within a VHN.

# VHN service framework architecture

This clause identifies the service framework architecture for new functions and extensions/modifications of the various functions to support VHN services in home network.

Figure 8.1 shows the VHN service control architecture based on the home functional architecture [ITU-T H.622.] for the provision of VPHN services.

Figure 8-1: Framework architecture for VHN services based on [ITU-T H.622]

## VHN service control function (VHN-SCF)

The VHN service control functions (including VHN membership management and VHN multicast service control functions) support service control functions for VHN applications including interaction with service user profiles. VHN service control functions provide the capabilities to:

- register VHN members;

- authenticate and authorize VHN members;

- select data encryption methods, and key exchanges if required;

- perform VHN mobility management;

- perform session control among VHN members;

- perform VHN membership management;

- perform VHN multicast service control within a VHN.

### VHN membership management function

The VHN membership management functions provide support for:

- creation, maintenance and release of VHNs;

- joining and leaving VHNs;

- partitioning of a VHN into multiple VHN groups (including VHN multicast groups).

The VHN membership management functions interact with multicast group membership management functions for VHN multicast group membership management.

### VHN multiservice control function

The VHN multicast service control functions use multicast service control functions to control VHN multicast groups.

### VHN service support function

The VHN service support functions include support for application developers and providers to develop VHN applications.

This includes functions such as registration, authentication and authorization functions at application level within the VHN. These functions are available to the “virtual home network applications” and “terminal” functional groups in VHN. This work in conjunction with the VHN-SCF to provide end-users and applications with the services within VHN.

## VHN architectural considerations

### Application level interworking for VHN service

VHN supports a functional architecture to create and interwork another virtual service group through application-level home gateways. A VHN service group is created via its required interworking function of an application-level home gateway, and each user will be able to join multiple VHN service group in accordance with their favour application features. Application-level home gateway functions provides to route data and convert into different protocols according to user environment using some interworking protocols. The application-level home gateway function is applied to abstract the service capabilities and features of VHN.

Appendix III and V show an examples of application-level home gateway function for VHN.

## Federation capabilities

[Editor’s Note] related description on federation capability should be included for further understanding

Various types of VHN services will be composed through collaboration among multiple entities of VHN. As shown in figure 8-3, service entities of VHN are federated to create new featured applications in terms of two approaches:

- internal federation through DNG-centric initiation

- external federation interconnecting multiple gateways to form a cooperative overlay across residential networks.

The extwrnal federation enables to offer added value in terms of e.g., resource sharing and

collaborative network optimizations. For example, improved service delivery and wireless neighbourhood optimization in terms of load-balancing, interference management, etc.

The internal VHN federation enables features such as communication, resource, and content sharing among the involved networks as well as a common interface to these networks through the DNG.

Figure 8-3: VHN service federation model

## Security for VHN

VHN has service capabilities such as remote services, multiple device control service, health care service, education and entertainment service, etc. these services significantly need for security. Virtual home network supports the security requirements in accordance with [ITU-T X.1111].

Major security requirements forVHN virtual home network are:

- authentication of the communication entities for policy requests in virtual home network;

- data confidentiality and integrity among users for virtual home network;

- availability and accessibility in virtual home network, upon demand by an authorized entity;

- availability of mechanisms of non-repudiation for preventing one of the entities or parties in communication from denying participation in the whole or part of the communication falsely.

The security considerations in the virtual home network should be in accordance with the security requirements in [ITU-T X.1111] and [ITU-T Y.2701].

# Management functionality for virtual home network

Management functions of VHN do not provide services directly to home network or virt

Management function of VHN is required to support the following functions to provide service management in virtual home network;

- terminal device management

- access control

- service session control

- resource control

- traffic management (e.g. unicast and multicast)

- fault management

- performance management

# Use case of virtual home network

## The revised texts for clause 10 is proposed in the contribution AVD 4335.

# Appendix I

Virtual home networking capability to provide IPTV service over NGN service overlay network

(This appendix forms an integral part of this Recommendation)

One of the solutions to provide virtual home network environment is to use service overlay network functions.

Service overlay network utilizes DNG information of each home network and controls home network with information from DNG. Also service overlay network functions exist above service stratum in NGN and exchange service control information with DNG via ANI. For these reasons, IPTV virtual home network service can be widely applied in different network environments such as NGN, etc.

Figure I-1: IPTV VHN service over NGN service overlay network

Figure I-1 shows an example of service overlay network applied in IPTV virtual home network service. Users in home network request IPTV VHN and service to SON (Service Overlay Network) Entity in service overlay network functions via DNG. SON Entity in service overlay network functions provides requested IPTV home network service by confirming home network information and user profiles.

SON Entity has geographical distributed structure of service provider. IPTV virtual home network services capabilities of each SON provides following functions:

- Easy composition of dynamic virtual home network

- IPTV service interaction between home networks

- Network level QoS and application level QoS guarantee between home networks

- Application security guarantee between home networks

# Appendix II

Federation capability of VHN: use case of IPTV service

(This annex forms an integral part of this Recommendation)

IPTV virtual home community service forms a home network service subscriber centric virtual community that works similarly to the web community for multiple applications of IPTV services on the NGN based home network. Users can subscribe to the community via application. Once the community is formed at service level, virtual home IPTV service connectivity is formed based on NGN connectivity making usage of community information. Security, QoS and other features suitable for virtual home IPTV service community are provided on the NGN based service overlay network.

Required functions to provide IPTV virtual home community service on NGN based service overlay network are as follows:

- virtual home service user management

- home service registration and management

- contents location and processing control support

- service community management

- virtual home community user group management

- application level security support

- community topology management

Service community management manages IPTV virtual home community of application level, and registered IPTV user information provided by the corresponding IPTV virtual home community. IPTV virtual home community user group management manages user groups that use the same IPTV service (within the same community). Security of the user and network is supported by application/transport level security.

Figure II-1: Example of IPTV virtual home community service: Home network federation

Figure II-1 shows an example of IPTV virtual home community service in NGN based service overlay network environment. IPTV virtual home community service forms a community group, which is a group of users who joined the same community in NGN based service overlay network.

Figure II-2: IPTV Service architecture for virtual home community service

Figure II-2 shows the service architecture for IPTV virtual home community service in NGN based service overlay network. IPTV virtual home network supports IPTV community groups (e.g., IPTV virtual home community A …, IPTV virtual home community N) within Application Function for IPTV Service.

# Appendix III

Example of application-level home gateway function

(This annex forms an integral part of this Recommendation)

Most of the service entities in virtual home networks are generally low computing powered devices of special-purpose embedded computers. The devices do not have enough computing power to process all the information carried on general-purpose protocols. Thus, the virtual home gateway functions should process before the information reaches to terminal devices. This is to provide similar perspectives to all the terminal devices and following features are to be satisfied:

- verify the features or location of devices without hardcoded hostnames to addresses;

- use http for application-level home gateway communication for universal usages

- or interwork with ASF (Application service Support Function) in NGN;

- provide unified management method to application and existing systems;

- convert data for devices with limited computational resources.

Application-level home gateway functions of home gateway bridge between http-based control method and various middleware-application control method. Any devices that can generate http-based control protocol such as PCs, PDAs, cellular phone, digital TV sets with a web browsing capabilities

Figure III-1: Example of Application-level home gateway function

Figure III-1 describes application-level home gateway functions. Application-level home gateway consists of four functional components:

- Registration management function;

- Protocol translation function;

- Home network protocol;

- Interworking and http protocol or ASF in NGN.

If application-level home gateway functions receives an http-based control messages, it parses the message, applies the necessary protocol translation and sends out to the corresponding middleware application. The terminal devices see the gateway as if it provides the functions or services that are actually provided by devices on the internet.

# Appendix IV

Example of Middleware functions for virtual home network

(This annex forms an integral part of this Recommendation)

According to UPnP device architecture, UPnP architecture consists of media renderer, media server and control point.

The media renderer obtains content from media server and renders it (e.g., display, decodes). For example the role of media renderer is playing of TV set, MP3 Player, etc. The media server is used to make content available on the home network and transfer it to the terminal devices. It supports multiple transfer protocols and data formats and has functionality of conversion from one format to another one in the real time. The main tasks of control point are synchronization and management of A/V devices in home network. It allows end user to select, play, pause and stop content from media server and display it on media renderer. control point is able to control media renderer characteristic e.g., brightness, contrast, volume, etc.  An example of Control Point is TV set with remote control. All devices that support UPnP should have implemented basic UPnP terminal device profile. The virtual home network system architecture which used UPnP is shown below on figure IV-1.

Figure VI-1: UPnP home network system architecture for integrated virtual home network

Bibliography

[b-IETF RFC3376]	RFC3376 (2002), Internet Group Management Protocol, Version 3

[b-DSLF TR-068]	DSL Forum TR-068 (2005), Base Requirements for an ADSL Modem with Routing

[b-DSLF TR-094]	DSL Forum TR-094 (2004), Multi-Service Delivery Framework for Home Networks

[b-ISO/IEC15045-1] 	ISO/IEC15045-1 (2004), Information technology –Home electronic system (HES) gateway –Part 1: A residential gateway model for HES

[b-IEC62481-1]		IEC62481-1 Ed.1: DLNA Home networked device interoperability guideline – Part 1: Architecture and Protocols

[b-IEC62481-2]		IEC62481-2 Ed.1: DLNA Home networked device interoperability guideline – Part 2: Media Formats

[b-HGI]		Home Gateway Initiative, Home Gateway Technical Requirements: Release 1.0 (1 July 2006).

[b-ITU-T Y.1291]	ITU-T Recommendation Y.1291, An architectural framework for support of Quality of Service in packet networks

___________________________