| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | TD-36 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.LOOPB |  |  |  |
| Title: | H.248.LOOPB “Gateway control protocol: Usage of Loopback in H.248” (New): output draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Editor’s output draft for the draft H.248.LOOPB “Gateway control protocol: Usage of Loopback in H.248” (New).

Document History:

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.LOOPB Ed. 0.1 | Initial proposal draft (TD-39) from WP 2/16 Research Triangle Park (USA) meeting (2010-11) based on AVD-4009 Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1011_Res/TD-39.zip |
| H.248.LOOPB Ed. 0.2 | Input (TD 490/WP2) for ITU-T SG16 Meeting, March 2011 Location: http://www.itu.int/md/T09-SG16-110314-TD-WP2-0490/en |
| H.248.LOOPB Ed. 0.3 | Output (TD 490R1/WP2) for ITU-T SG16 Meeting, March 2011 Location: http://www.itu.int/md/T09-SG16-110314-TD-WP2-0490/en |
| H.248.LOOPB Ed. 0.4 | Input (AVD 4074) for ITU-T SG16 Meeting, July 2011 Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/AVD-4074.zip |
| H.248.LOOPB Ed. 0.5 | Output (TD 36) from ITU-T SG16 Meeting, July 2011 Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/TD-36.zip |
| H.248.LOOPB Ed. 0.6 | Input (TD 640/WP2) to SG16, Geneva, 21st November – 2nd December 2011 meeting Location: http://www.itu.int/md/T09-SG16-111121-TD-WP2-0640/en |
| H.248.LOOPB Ed. 0.7 | Output (TD 640R1/WP2) from SG16, Geneva, 21st November – 2nd December 2011 meeting Location: http://www.itu.int/md/T09-SG16-111121-TD-WP2-0640/en |
| H.248.LOOPB Ed. 0.8 | Input (AVD-4188) for SG16, Geneva, 20 – 24 February 2012 meeting Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/AVD-4182.zip |
| H.248.LOOPB Ed. 0.9 | Output (TD-25) for SG16, Geneva, 20 – 24 February 2012 meeting Changes as a result of : AVD-4172 Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-25a.zip |
| H.248.LOOPB Ed. 0.10 | Input (TD 765/WP2) to SG16, Geneva, 30 April – 11 May 2012 meeting |
| H.248.LOOPB Ed. 0.11 | Output (TD 765R1/WP2) from SG16, Geneva, 30 April – 11 May 2012 meeting Changes as a result of: C-894 Location: http://www.itu.int/md/T09-SG16-120430-TD-WP2-0765/en |
| H.248.LOOPB Ed. 0.12 | Input (AVD-4242) to SG16, Brisbane September 2012 |
| H.248.LOOPB Ed. 0.13 | Output (TD-36) for SG16, Brisbane September 2012 Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-36.zip |

CONTENTS

1	Scope	4

2	References	4

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	5

4	Abbreviations	5

5	Conventions	6

6	Loopback functionality in H.248	6

6.1	Precedence	7

6.2	Sending of media	7

6.3	Monitoring Media Quality	8

Appendix I  Example use cases for  Loopback-based Performance Monitoring in RTP networks	8

I.1	Overview	8

I.2	Use case #1: IP-IP H.248 MG and remote SIP UE – Loopback point located at MG (“local LP”)	8

I.3	Use case #2: IP-IP H.248 MG and remote SIP UE – Loopback point located at remote RTP entity (“remote LP”)	10

I.4	Use case #3: Multiple IP network domains - segment-based measurements – using loopback	12

Draft ITU-T Recommendation H.248.xx (ex H.248.LOOPB)

Gateway control protocol: Usage of Loopback in H.248

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

1	Scope

This Recommendation clarifies the interaction between and use of the H.248 "loopback" mode with the SDP Media Loopback functionality described by [IETF LOOPBACK]. It only describes the interactions between these functionalities rather than re-describing the entire loopback procedures in [IETF LOOPBACK].

2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/05), Gateway Control Protocol: Version 3

[ITU-T H.248.30]	Recommendation ITU-T H.248.30 (01/07), Gateway Control Protocol: RTCP extended performance metrics packages

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/12), Gateway Control Protocol: RTCP XR Block Reporting Package

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3611]	IETF RFC 3611 (2003), RTP Control Protocol Extended Reports (RTCP XR).

[IETF LOOPBACK]	IETF draft-ietf-mmusic-media-loopback-23, An Extension to the Session Description Protocol (SDP) for Media Loopback.

3	Definitions

3.1	Terms defined elsewhere

None.

3.2	Terms defined in this Recommendation

3.2.1	H.248 Loopback: The process where media (including transport packets) received (Local Descriptor) on a termination is sent from the sending side (Remote Descriptor) of the termination without the media being passed between that termination and other terminations in the context.

3.2.2	H.248 Media Loopback: The media traffic covers all types of media which may be described by the H.248 Media Descriptor. This is a superset with regards to IETF loopback media traffic. The term media covers therefore not only packet-switched media traffic, but also media related to H.248 physical terminations.

3.2.3	IETF Media Loopback: The media traffic is limited to "Media-over-RTP" only. The loopbacked media portion relates to the application data encapsulated in RTP packets (i.e., the RTP-SDU part).

4	Abbreviations

This Recommendation uses the following abbreviations and acronyms:

| BGW | Border Gateway |
| --- | --- |
| BR | Basic Reports |
| CP | Collection Point |
| IP | Internet Protocol |
| LP | Loopback Point |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MP | Measurement Point |
| OP | Originating Point |
| RD | Remote Descriptor |
| RP | Reporting Point |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session Description Protocol |
| SDU | Service Data Unit |
| SIP | Session Initiation Protocol |
| SR | Sender Receiver |
| UE | User Equipment |
| XR | Extended Report |

5	Conventions

Elements of the H.248 protocol model, e.g. Context, Termination, Stream, Event are represented using the first letter capitalised. H.248 Property, Event, Signal and Parameter identities are given in italics. When used in the main text, SDP syntax is highlighted in bold i.e. “m=”.

6	Loopback functionality in H.248

H.248 Loopback functionality is supported through the use of the LocalControl Descriptor Mode Property (clause 7.1.7/[ITU-T H.248.1]). The mode property is applicable to all transport types and is independent of any call control protocol used by the MGC.  When a mode property is set to "loopback", the MG shall behave according to clause 7.1.7/[ITU-T H.248.1].  The H.248 loopback is a media level loopback in that the received media stream is processed according to the received Termination and may be sent in the Context (but not to other Terminations).

The mode property, whilst being independent of the call control variant and transport used, is used within an overall network procedure for initiating and co-ordinating loopback. Where the Real Time Protocol (RTP) is used, [IETF LOOPBACK] describes additional SDP parameters and procedures for negotiating and initiating loopback functionality. As the H.248.1 Local and Remote Descriptors contain SDP this introduces an alternate means of specifying loopback functionality.

Figure 1 illustrates the example of an H.248 Stream endpoint for traffic processing following a layered protocol architecture. Figure 1 shows that the loopback path may be “located” on different protocol layers. Such a protocol layer-dependent loopback path is either:

- implicitly given by the available media and transport information in the Local and Remote Descriptor; or

- may be explicitly indicated, like e.g. by the SDP parameters referred in clause 6.1.

Figure 1/H.248.LOOPB – Protocol layer-dependent Loopback (media) path options

6.1	Precedence

As indicated by clause 7.1.7/[ITU-T H.248.1] there may be an interaction between the mode property and the contents of the Local and Remote Descriptor. The mode property is described as taking precedence over any mode specified by the contents of the Local and Remote Descriptors.

[IETF LOOPBACK] defines procedures and attributes for both the initiating and answering side. For the initiating side these procedures relate to the sending of a certain RTP payload which would be controlled through normal the H.248 "Mode" property (i.e. Send, Send/Receive) and setting the appropriate media and connection information in the Remote Descriptor. However for the answering side the procedures of [IETF LOOPBACK] result in a change of mode behaviour and thus may conflict with the setting of the H.248 mode property. Thus the precedence described by clause 7.1.7/[ITU-T H.248.1] is only applicable to the "answerer" (i.e. the Termination performing the loopback (or providing the mirror)).

Therefore if the H.248 mode property is set to "Inactive", "Send",  "Receive", "Send/Receive" this setting will take precedence over any SDP defined by [IETF LOOPBACK] set on the answering termination, even if a=loopback-mirror is set. As a result the media or RTP packets will not be looped (mirrored) back.

However if the H.248 mode property is set to "Loopback" then any SDP defined by [IETF LOOPBACK] is deemed to be complementary to the H.248 mode. Media or RTP packets will be looped back according to the SDP contained in the Local and Remote Descriptors.

This allows the ability in H.248 to discriminate between a Packet or Media loopback as defined in clause 4.1/[IETF LOOPBACK].

6.2	Sending of media

[IETF LOOPBACK] assumes an SDP offer/answer exchange occurs to negotiate the use of the loopback functionality. On receipt of an answer the MGC may then provide the Remote Descriptor of the Termination/Stream in question with the necessary media/connection information at which point the MG will start to generate packets.

6.3	Monitoring Media Quality

Clause 9/[IETF LOOPBACK] indicates that he use of the loopback attribute is intended for monitoring of media quality of the session. Appendix I illustrates some example use cases.

Furthermore it states that an offering or answering entity that is compliant to this specification SHOULD support RTCP per [IETF RFC3550] and RTCP-XR per [IETF RFC3611]. However this may not always be the case, Appendix I illustrates some example use cases that do not use RTCP and RTCP-XR, due to the fact that the measurement and reporting points are at the same location.

It then details which metrics / blocks should be supported. In order to request a MG to collect statistics regarding these metrics the MGC should set the appropriate statistics via [ITU-T H.248.30] and/or [ITU-T H.248.48].

Appendix I

Example use cases for 
Loopback-based Performance Monitoring in RTP networks

(This appendix does not form an integral part of this Recommendation)

I.1	Overview

This Appendix studies a number of example use cases with regards to the network application of loopback-based performance monitoring in RTP networks. Each use case is illustrated by a diagram. The set of considered capabilities is structured in multiple steps (e.g. measurement, reporting and/or filtering points; per traffic direction) in order to point out the specific functionalities behind network performance monitoring.

I.2	Use case #1: IP-IP H.248 MG and remote SIP UE – Loopback point located at MG (“local LP”)

A measurement point (MP) is usually at the same location as the observation point. However  in case of an available transport means for transferring the concerned traffic data from the observation point to a remote location, the MP could be geographically removed from the observation point. “Loopback” is such a capability: the traffic is redirected at the loopback point (LP) in the reverse direction.

Figure I.1 depicts a use case with “media loopback” enabled at the H.248 BGW. The H.248 BGW provides a loopback point (LP) for RTP traffic originating from UE A. The IETF “loopback source” and “loopback mirror” roles are assigned to UE A and BGW respectively in this example. The motivation for using an LP instead of a local MP at an H.248 MG is elaborated in more detail in [ITU-T H.248.RTCPPROF].

Figure I.1 – Use case #1: IP-IP H.248 MG and remote SIP UE – Loopback point located at MG (“local LP”)

The MG assigned LP is motivated by the fact, that the SIP UE is able to report measurements to control plane entities via SIP. The MG reporting capability in terms of H.248 statistics (RPH.248) is not used (5).

The SIP UE provides thus a RPSIP (3) for the local measurements of received incoming RTP traffic (2) and mirrored RTP traffic from the BGW (1).

There is no RTCP reporting capability (for measurement data; RPRTCP) required (4):

- neither from the SIP UE towards the BGW, because the H.248 MG does not use H.248 statistics;

- nor from the BGW towards the SIP UE, because the H.248 MG does not produce local measurements (“lacks a local MP”).

The BGW consequently is not required to support a collection point (CP) and filtering point (FP) for RTCP XR processing.

I.3	Use case #2: IP-IP H.248 MG and remote SIP UE – Loopback point located at remote RTP entity (“remote LP”)

Figure I.2 depicts the same network scenario, but with inverted loopback and reporting capabilities:

- SIP UE A configuration: LP (yes), RPSIP (no);

- BGW configuration: LP (no), RPH.248 (yes).

Thus an use case with an enabled “media loopback” at the remote end from H.248 BGW point of view.

The UE A provides a loopback point (LP), - i.e., the IETF “loopback mirror” role, for RTP traffic originating from UE B.

Figure I.2 – Use case #2: IP-IP H.248 MG and remote SIP UE – Loopback point located at remote RTP entity (“remote LP”)

The RPH.248 (5) for the local measurements of originating RTP traffic (1) and mirrored RTP traffic from the UE A (2).

There is no RTCP reporting capability (for measurement data; RPRTCP) required (4):

- neither from UE A towards the BGW, because the UE does not produce local measurements (“lacks a local MP”).

- nor from the BGW towards UE A, because the UE does not need a control plane reporting capability (“due to the loopback usage”) (NOTE 1).

It may be noted that the BGW provides actually a “virtual loopback source” role, rather than the real loopback source, which is given be the RTP end system UE B (NOTE 2).

NOTE 1 – The UE may need the RTCP XR data for other purposes than control plane reporting. For instance due to a management plane reporting interface, or as input data for the UE-embedded application control logic, or as output information for a local display.

NOTE 2 – Whether real or virtual loopback source would be dependent on the specific RTP topology as assigned to the BGW for this call.

I.4	Use case #3: Multiple IP network domains - segment-based measurements – using loopback

Figure I.3 depicts a loopback scenario, which serves the requirement for measuring a remote network segment by using loopback points at the network demarcations, and local MPs for each LP.

Figure I.3 – Use case #3: Multiple IP network domains - segment-based measurements – using loopback

Example:

Considering the unidirectional traffic flow from UE A (i.e., the OP) towards UE B across four IP network domains (Xd-1, Xd, Xd+1, Xd+2), which are bounded by three H.248 BGWs (Nk-1, Nk, Nk+1). H.248 BGW Nk may be used as measurement point (and control plane reporting point) in this example. There are loopback points enabled at the neighbour BGWs (LPk and LPk+1). There are then three media flows to be measured by H.248 BGW Nk (via the three measurement points MPk-1, MPk and MPk+1).

Performance metric RTP packet loss rate (RPLR) might be measured by the MPs, with e.g. following results: RPLRk-1 = 0,3%, RPLRk = 0,4% and RPLRk+1 = 2,1%. Leading to the conclusion that the contributed packet loss would be estimated as 0,3%, 0,1% and 1,7% by IP network domains Xd-1, Xd and Xd+1 respectively.

_____________________