| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | TD-41 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors H.VSMarch |  |  |  |
| Title: | H.VSMarch “Architectural requirements for mobile visual surveillance” (New): output draft |  |  |  |
| Purpose: | Proposal |  |  |  |

This document contains the Editor’s output draft of the ITU-T Rec. H.VSMarch "Architectural requirements for mobile visual surveillance"(New).

CONTENTS

1	Scope	4

2	References	4

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	6

4	Abbreviations	6

5	Conventions	7

6	Functional Requirements	7

6.1	Video/Audio Acquisition (VAA)	7

6.2	Instant Image Snapshot (IIS)	7

6.3	Recording and Playback (RnP)	7

6.4	PTZ Control	7

6.5	Alarming and Triggered Action (ATA)	7

6.6	Service Management	7

6.6.1	User authentication	7

6.6.2	Management Configuration	8

6.6.3	PU list search and obtaining	8

6.6.4	Objects of interest	8

7	Functional Architecture	8

7.1	Functional Model	8

7.2	Functional Architecture Framework	8

7.3	Functional Entities	9

7.3.1	MSP	9

7.3.2	VAU	9

7.3.3	M_CU	10

7.4	Reference Points	10

7.4.1	Reference Point Cms: CMU-MSP	10

7.4.2	Reference Point Msm: MSP-M_CU	11

7.4.3	Reference Point Mv: M_CU-VAU	11

7.4.4	Reference Point Vm: VAU -M_CU	11

7.4.5	Reference Point Vs: VAU -SCU	11

7.4.6	Reference Point Mdv: MDU-VAU	11

7.4.7	Reference Point Pv: PU-VAU	11

7.4.8	Reference Point Vp: VAU-PSS	12

7.4.9	Reference Point Psm: PSS-M_CU	12

7.5	Service Control Flow and Signalling	12

7.5.1	Real-time Video/Audio Acquisition	12

7.5.2	Recording and playback	13

7.5.3	PTZ Control	16

Bibliography	17

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 7-1: Components for mobile visual surveillance 8 Figure 7-2: Function components of VAU 9 Figure 7-3: High level procedural flows for real-time Media Acquisition 12 Figure 7-4: High level procedural flows for playback in platform 14 Figure 7-5: High level procedural flows for playback in PU 15 Figure7-6: High level procedural flows for PTZ control 17

Draft new ITU-T Rec. H.VSMarch

Architectural requirements for mobile visual surveillance

AAP Summary

[To be provided before Consent]

Summary

The purpose of this Recommendation is to define a visual surveillance system with mobile units. The mobile visual surveillance system provides the display and storage of the multimedia (such as video, audio, image) captured by multiple remote cameras over an IP network for mobile users. It also provides other functionalities such as remote control, alarming and linkage actions, recording and playback.

This recommendation defines the functional requirements, model, architecture, entities, reference points and service control flow of the mobile visual surveillance.

Keywords

Mobile Visual Surveillance, Camera, Alarming, PTZ Control, Recording and playback, Architecture

# Scope

The Visual Surveillance service is a telecommunication service focusing on video (audio included also) application technology, which is used to capture remotely multimedia (such as audio, video, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

The Recommendation H.VSMarch will focus on the architecture of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system. This Recommendation defines the functional requirements, model, architecture, entities, and reference points of the mobile visual surveillance system, based on the architecture defined in the Recommendation ITU-T H.VSarch.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance

[ITU-T H.626]	Recommendation ITU-T H.626 (2011), Architectural requirements for visual surveillance

[ITU-T H.VSPROT]	Recommendation ITU-T H. VSPROT, Signalling and Protocols for visual surveillance

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	application [ITU-T Y.101]: A structured set of capabilities, which provide value-added functionality supported by one or more services.

3.1.2	customer [ITU-T M.60]: An entity which receives services offered by a service provider based on a contractual relationship. It may include the role of a network user.

NOTE 1 – In this document, Customer has the same meaning with End User.

3.1.3	functional architecture [ITU-T Y.2012]: A set of functional entities and the reference points between them used to describe the structure of an NGN. These functional entities are separated by reference points, and thus, they define the distribution of functions.

NOTE 2 – The functional entities can be used to describe a set of reference configurations.  These reference configurations identify which reference points are visible at the boundaries of equipment implementations and between administrative domains.

NOTE 3 – The definition is not only applicable to NGN, but also to other IP packet switch based network.

3.1.4	functional entity [ITU-T Y.2012]: An entity that comprises an indivisible set of specific functions. Functional entities are logical concepts, while groupings of functional entities are used to describe practical, physical implementations.

3.1.5	interface [ITU-T Y.101]: A shared boundary between two functional units.

NOTE 4 – An interface is defined by various characteristics pertaining to the functions, physical interconnections, signal exchanges and other characteristics as appropriate.

3.1.6	media [ITU-T Y.101]: Plural of medium.

3.1.7	medium [ITU-T Y.101]: a) Specific physical support for transmission or storage of information. b) Type of presentation of information (i.e., video, audio, text, etc.).

3.1.8	QoS [ITU-T Y.101]: The collective effect of service performances which determine the degree of satisfaction of a user of the service.

3.1.9	reference point [ITU-T Y.2012]: A conceptual point at the conjunction of two non-overlapping functional entities that can be used to identify the type of information passing between these functional entities.

NOTE 5 – A reference point may correspond to one or more physical interfaces between pieces of equipment.

3.1.10	role [ITU-T Y.110]: The role is a business activity which fits in a value chain. The role is constrained by the smallest scale of business activity which could exist independently in the industry and so a marketplace will exist for every relationship between roles.

3.1.11	service [ITU-T Y.101]: A structure set of capabilities intended to support applications.

3.1.12	customer unit [ITU-T H.626]: A device located at the customer part of a Visual Surveillance system and used to present multimedia information (such as audio, video, image, alarm signal, etc) to the end user.

3.1.13	premises unit [ITU-T H.626]: A device located at the remote part of a Visual Surveillance system and used to capture multimedia information (such as audio, video, image, alarm signal, etc) from a surveilled object.

3.1.14	service platform [ITU-T H.626]: A series of devices and subsystems located at the centred part of a Visual Surveillance system. It is used to integrate all of the capabilities and provide visual surveillance services to customers. The main functions include service control function, media switching, distribution, storage, control and management functions.

3.1.15	surveillant [ITU-T H.626]: Individual performing surveillance.

3.1.16	surveilled object [ITU-T H.626]: The target (such as site, human, and related environment) on which surveillance is performed.

3.1.17	visual surveillance [ITU-T H.626]: A telecommunication service focusing on video (but including audio) application technology, which is used to remotely capture multimedia (such as audio, video, image, alarm signal, etc) and present them to the end user in a user friendly manner, based on managed broadband network with ensured quality, security and reliability.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Mobile Customer Unit

3.2.2	Mobile Service Portal

3.2.3	Video Access Unit

# Abbreviations

This Recommendation uses the following abbreviations and acronyms:

| AAA | Authentication, Authorization and Accounting |
| --- | --- |
| CMU | Centre management unit |
| CU | Customer unit |
| HTTP | Hypertext transfer protocol |
| MCU | Multimedia control unit |
| MDU | Media distribution unit |
| MSP | Mobile service portal |
| MSU | Media storage unit |
| M_CU | Mobile customer unit |
| PC | Personal computer |
| PTZ | Pan/tilt/zoom |
| PU | Premises unit |
| RTSP | Real-time streaming protocol |
| SCU | Service control unit |
| VAU | Video access unit |
| VS | Visual surveillance |

# Conventions

# Functional Requirements

To provide the function for end users to use mobile terminals to access visual surveillance system, there are some new functional requirements for this system.

## Video/Audio Acquisition (VAA)

A Mobile terminal can view authorized PU list and statuses after registered to MSP with username and password.

M_CU gets the information of VAU and gets real-time video stream through VAU to play. At the same time, it can perform operations of pause, resume, and reload.

## Instant Image Snapshot (IIS)

Instant image snapshot (IIS) function is one of the basic capabilities of VS, and is used to obtain instant image information based on video from Surveilled Object by M_CU when users are watching live surveillance video by M_CU in a mobile terminal.

## Recording and Playback (RnP)

Recording function is one of the basic capabilities of VS, and is used to record multimedia (such as audio, image, video) information from Surveilled Objects and stored these multimedia files by operations in mobile terminals.

The playback is an operation mode of media play, which is used to play audio and video which has been stored according to previously entered customer favourites.

## PTZ Control

The PTZ control function is used to pan, tilt and zoom a video camera, switch auxiliary peripherals (such as flashlight, fan and rain brush), preset position of related physical devices. Authenticated users can operate PTZ functions by M_CU in a mobile terminal.

## Alarming and Triggered Action (ATA)

Alarming and triggered action function is one of the basic capabilities of VS, and is used to respond to an input alarm signal event, give an output alarm activity and trigger the related actions. When the alarm event occurs, the VS systems utilises sound, image, or other ways to inform a mobile terminals.

## Service Management

### User authentication

When user visits MSP by M_CU, MSP performs authentication according to user type, user ID and other information submitted by M_CU. If the authentication succeeds, M_CU will return PU list, otherwise it will return fault messages.

### Management Configuration

M_CU configuration information includes server address, customer domain name, connection point, storage path of the instant image snapshot and recorded video, PTZ control parameters, etc.

M_CU can be updated to a new version automatically when the M_CU registered in the service platform. Users can select to download and install the new version M_CU also.

### PU list search and obtaining

After authorization, M_CU registers to service platform, it can request to search by key words and obtain the PU list information, and then show the list to end users. The information includes the user information, static list information and dynamic list information of PUs.

### Objects of interest

Objects of interest are used to save the information of PU list which have been selected by users. Users can view, modify, add, or delete one or multiple PU information from the list.

Editor’s note: Need to consider if there are other service management functions.

# Functional Architecture

Editor’s note: This clause will describe the composed components structure of mobile visual surveillance system, which has been discussed above.

## Functional Model

## Functional Architecture Framework

The high-level functional components are described in Figure 1.

Figure 7-1: Components for mobile visual surveillance

- M_CU: Mobile Customer Unit, such as software clients running on mobile phones used by the visual surveillance end users.

- MSP: Mobile Service Portal, MSP provides the whole access portal for mobile surveillance users, including user authentication portal, PU list portal, etc. MSP provides service management functions for the end users through communication with SCU, such as user authentication, information querying, programme for surveillance content, synchronization of information with SCU, etc.

- VAU: Video Access Unit, VAU is the key unit to implement the communication between M_CU and the units defined in ITU-T Rec. H.626. The main functions of VAU include obtaining management information from SCU, processing the video acquisition request from M_CU, creating connection with MDU and/or PU, distributing and transcoding multimedia data, transferring commands of PTZ control, and dynamically adjusting the media stream according to network status, etc.

## Functional Entities

Editor’s note: This clause will describe all of the new entities which are added for mobile visual surveillance system, includes the composition and functions of the entities.

### MSP

Mobile Service Portal (MSP) is the application server for mobile visual surveillance system.

MSP is used to communicate with SCU to provide service management for M_CU, such as authentication, PU list search and obtaining, video data search, information synchronization with SCU, etc.

### VAU

Video Access Unit (VAU) is the key unit to implement the control between mobile terminals and other units defined in Recommendation ITU-T H.626.The main functions of VAU include: Service control function, media control function (such as PTZ control command transmit), media processing function (such as multimedia transcoding), configuration management function.

Figure 7-2: Function components of VAU

The VAU provides media transcoding function for M_CUs locating in different types of networks; it acquires pre-configuration of transcoding parameters for various mobile networks from SCU. The transcoding parameters include resolution, bit rate, encoding type and I-frames interval

The following describes the function components of VAU.

Service Control Function:

- Initiate, maintain and release the connection with SCU.

- Process service requests from M_CU, forward them to SCU and respond to M_CU.

- Acquire media stream from MDU and/or PU.

Media Control Function:

- Receive media stream from MDU and/or PU according to M_CU’s request.

- Transmit or forward media stream to M_CU.

- Process control and report indications from M_CU.

Media Processing Function:

- Media stream transcoding before transferring stream from MDU and/or PU to M_CU according to the ability of the M_CU.

- Adjust the key frames interval and bit rate of media stream according to the packet loss rate reported by M_CU before VAU transfers the stream from MDU and/or PU to M_CU (in addition, when packet loss rate increases, VAU decrease key frames interval and bitrate, and when the packet loss rate decreases, VAU increase the key frame interval and bitrates).

Configuration Management Function:

- Manage the overall configuration in VAU, including transmission, decoding, transcoding, and other control of stream.

### M_CU

The M_CU is the functional entity within mobile VS system, such as mobile phones used by the visual surveillance end users. It is used to present multimedia information (such as audio, video, image, alarm signal, etc.) to the end user. It implements the following functions:

- Requesting media stream from VAU, based on the network types which the M_CU is locating.

- Media stream receiving, decoding and playback. Media stream packet loss rate reporting to VAU periodically (e.g. if RTP is used for media stream transmitting, M_CU reports the loss rate by RTCP protocol).

- PU list, real-time image/video presentation.

- Console interface for end user to operate the VS system (such as PTZ control).

## Reference Points

Editor’s note: This clause will describe all of the reference points which are added for mobile visual surveillance system, includes the relationship of entities and the functions of reference points.

### Reference Point Cms: CMU-MSP

The reference point Cms is between the CMU and the MSP.

- After user first visiting MSP, the reference point Cms is used to deliver registration, access authentication, authorization, accounting signals between the MSP and the CMU.

- Returning surveillance lists of the user by CMU to MSP.

- Inquiring the media address of the visited surveillance point by MSP to CMU.

### Reference Point Msm: MSP-M_CU

The reference point Msm is between the Msp and M_CU.

- After user first visiting MSP, the reference point Msm is used to deliver registration, access authentication, authorization, accounting signals between the M_CU and the MSP.

- Returning surveillance lists of the user by MSP to M_CU.

- Returning the media address of the visited surveillance point by MSP to M_CU, after user selecting media from M_CU via GUI.

### Reference Point Mv: M_CU-VAU

The reference point Mv is between the M_CU and the VAU.

The Reference Point Mv is used to deliver the related media operations between the M_CU and the VAU.

- Sending media request and receiving response by M_CU to VAU.

- Delivering control instruction of PTZ and receiving response by M_CU to VAU.

- Reporting packet loss rate of media stream to VAU periodically.

### Reference Point Vm: VAU -M_CU

The reference point Vm is between the VAU and the M_CU.

The Reference Point Vm is used by the VAU to distribute, and deliver media to M_CU using streaming methods.

### Reference Point Vs: VAU -SCU

The reference point Vs is between the VAU and the SCU.

The Reference Point Vs is used by SCU to control VAU registration, access authentication, authorization, accounting.

- Transferring the PTZ Control operation commands from the M_CU by VAU to SCU.

- Delivering control signals between the VAU and the SCU in order to create a media session between PSS and MDU.

- Querying the transcoding parameters from SCU according to the network type the M_CU locates in.

### Reference Point Mdv: MDU-VAU

The reference point Mdv is between the MDU and the VAU.

The Reference Point Mdv is used by the MDU to distribute, store and deliver media to VAU using streaming methods

### Reference Point Pv: PU-VAU

The reference point Pv is between the PU and the VAU.

The Reference Point Pv is used to deliver interaction signals directly between the VAU and the PU such as RTSP request or response, as well as to deliver media stream from PU to VAU directly.

### Reference Point Vp: VAU-PSS

The reference point Vp is between the VAU and the PSS.

The Reference Point Vp is used to deliver interaction signals between the PSS and the VAU such as an RTSP request or response, as well as to deliver media stream from VAU to PSS.

- PSS is used to get multimedia stream from VAU, store multimedia data, distribute multimedia stream to M_CU, and provide other streaming server services.

### Reference Point Psm: PSS-M_CU

The reference point Psm is between the PSS and the M_CU.

The Reference Point Psm is used to create a media session and deliver interaction signals between the M_CU and the PSS, such as an RTSP request or response, as well as to deliver media stream from PSS to M_CU.

## Service Control Flow and Signalling

Editor’s note: This clause will describe the basic service procedure flow and the corresponding signalling and protocols different from those flows in ITU-T H.VSarch, the subsection category can be modified based on contributions in future meeting.

### Real-time Video/Audio Acquisition

When users want to view the real-time surveillance video/audio by mobile terminals, after MSP performs authentication and returns VAU information to M_CU, M_CU choose and request to the VAU for real-time video/audio.

Figure 7-3: High level procedural flows for real-time Media Acquisition

This figure shows the procedural flows for real-time Media Acquisition for one M_CU:

- First the M_CU initiates a request and creates connection with the MSP.

- The MSP initiates a request and creates connection with the CMU.

- The CMU returns the VAU information.

- The MSP gets the request and returns VAU information to M_CU.

- When the M_CU gets VAU information, it requests the VAU for the real-time media address with the network type the M_CU locates in.

- VAU receives this request and requests to SCU for the media address. At the same time, VAU queries the corresponding transcoding parameters from SCU.

- The SCU transfers this request to the MDU.

- After receiving messages, the MDU returns the real-time media address to the SCU.

- The SCU returns the real-time media address to the VAU.

- After receiving the real-time media address, the VAU requests the MDU to start transmitting media stream.

- The MDU sends a request to the PU for initiating a media flow.

- The PU creates media channel with the MDU and sends media flow to the MDU.

- The MDU transfers the media flow to the VAU. VAU performs transcoding according to the transcoding parameters.

- The transcoded media flow is transferred from the VAU to the M_CU.  The M_CU decodes and plays the received media flow.

- When the M_CU stops the real-time media acquisition it sends a stop-media request to the VAU.

- VAU stops transcoding after receives the request. The VAU stops sending media stream to the MDU.

- The MDU sends response to the VAU for stopping media flow.

- The VAU requests the SCU to disconnect which then returns a response to the VAU.

- Then the VAU ends this real-time media acquisition and sends response to the SCU.

### Recording and playback

The playback function is used to play audio and video which has been previously recorded and stored.

There are three locations at which recorded media are stored, in platform, in the PU and in the M_CU. With media stored in the M_CU, it plays the recorded media just like playing any local media file. As additional interaction with other parts of the VS system is not needed, the following procedural flows will not include this simple method.

#### Playback in platform

When users want to playback the surveillance point by mobile phones, they must log in the MSP by the M_CU. the MSP performs authentication and returned the playback surveillance list.

After the users choose one surveillance point name, the M_CU sends a request for this surveillance video to the VAU, and then VAU returns the media stream to the M_CU to play.

Figure 7-4: High level procedural flows for playback in platform

This figure shows the procedural flows for playback in platform:

- The M_CU sends a request to the MSP for querying recording information.

- The MSP sends a request to the CMU for querying recording information.

- The CMU sends a request to the MSU for querying recording information.

- The MSU returns the querying result to the CMU.

- When the CMU gets the result, it transfers the result to the MSP.

- When the MSP gets the result, it transfers the result to the M_CU.

- The M_CU selects one playback point and starts a request to playback.

- The MSP sends a request for the playback address to the CMU.

- The request is transferred from the CMU to the SCU.

- Then the request is transferred from the SCU to the MDU.

- The MDU responses to the request and sends the playback address back to the SCU.

- The playback address is sent from the SCU to the CMU.

- The CMU sends the playback address to the MSP.

- The playback address is sent from the MSP to the M_CU.

- The M_CU starts playback request for the VAU with the playback address and the network type.

- The VAU starts playback request for the MSU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

- The MSU returns a response to the VAU and sends the media stream to the VAU.

- Once the VAU gets the media stream, it performs transcoding according to the transcoding parameters and then transfers the transformed stream to the M_CU.

- The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receives the request.

- The VAU sends a stop playback request to the MSU.

- As a response, the MSU stops sending stream.

- At the same time, the MSU notifies the MDU to stop playing.

- The MDU returns a response to the MSU.

#### Playback in PU

Figure 7-5: High level procedural flows for playback in PU

This figur shows the procedural flows for playback in PU:

- The M_CU sends a request to the MSP for querying recording information.

- The MSP sends a request to the CMU for querying recording information.

- The CMU sends a request to the PU for querying recording information.

- The PU returns the querying result to the CMU.

- When the CMU gets the result, it transfers the result to the MSP.

- When the MSP gets the result, it transfers the result to the M_CU.

- The M_CU selects one playback point and starts a request to playback.

- The MSP sends a request for the playback address to the CMU.

- The request is transferred from the CMU to the SCU.

- Then the request is transferred from the SCU to the MDU.

- The MDU responses to the request and sends the playback address back to the SCU.

- The playback address is sent from the SCU to the CMU.

- The CMU sends the playback address to the MSP.

- The playback address is sent from the MSP to the M_CU.

- The M_CU starts playback request for the VAU with the playback address and network type.

- The VAU starts playback request for the PU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

- The PU returns a response to the VAU and sends the media stream to the VAU.

- Once the VAU gets the media stream, it performs transcoding based on the transcoding parameters and then transfers the transformed stream to the M_CU.

- The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receiving the request.

- The VAU sends a stop playback request to the PU.

- As a response, the PU stops sending stream.

- At the same time, the PU notifies the MDU to stop playing.

- The MDU returns a response to the PU.

### PTZ Control

VAU receives a PTZ control request from the M_CU, parses and transfers this request to SCU, after authentication, SCU transfers this request to PU to perform PTZ operation.

Figure7-6: High level procedural flows for PTZ control

This figure shows the procedural flows for PTZ Control:

- M_CU sends a PTZ control request to VAU.

- VAU transfers this request to SCU.

- After authentication, SCU transfers this request to PU.

- PU parses this request and transfers PTZ control commands to related physical devices. Then the devices perform PTZ operations. And PU returns a response to SCU

- SCU transfers this response to VAU.

- VAU returns this response to M_CU.

- SCU writes logs about this PTZ control event such as operation time and operation type, then uploads the logs to CMU.

- CMU returns a response to SCU to complete logs uploading.

Bibliography

[b-ITU-T H.Sup.1]	ITU-T H-series Recommendations – Supplement 1 (1999), Application profile – Sign language and lip-reading real-time conversation using low bit rate video communication.

__________________