|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 4 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 4 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP¦H.248.TLS: Package design framework – Generic vs silo approach? |  |  |  |

# Summary:

There are two options: bearer specific H.248 packages for TLS and TCP or an architecture based on generic capabilities. The discussion of potential reuse of generic, BICC defined H.248 tools is the purpose of this contribution.

Summary:	1

1	Background:	2

2	Discussion: package reuse candidates?	2

2.1	Overview – candidates from BICC/Q.1950	2

2.2	Candidate: H.248 bearer characteristics package (BCP)	3

2.3	Candidate: H.248 generic bearer connection package (GB)	4

2.4	Candidate: H.248 bearer network connection cut-through package (BNCT)	5

2.5	Candidate: H.248 reuse idle package (RI)	7

3	Discussion: signaling flows for generic and TCP specific control	8

3.1	Bearer connection establishment	8

3.1.1	TCP specific control	8

3.1.2	Generic control	9

3.2	Bearer connection release	11

3.2.1	TCP specific control	11

3.2.2	Generic control	11

4	Discussion: generic vs silo approach?	11

5	Conclusions	12

# 1	Background:

H.248 bearer types ‘TLS’ and ‘TCP’ share the common characteristic of a “bearer control protocol (BCP)” (BICC/Q.1950 terminology) due to their connection-oriented nature. The BCPTCP relates to the TCP header flag based control procedures (primarily for connection establishment and release) and the BCPTLS is primarily given by the TLS handshake and alert protocols.

Bearer independent H.248 control procedures might be therefore beneficial instead of bearer specific H.248 packages, also due to the fact that there might be further bearer types beyond TLS and TCP.

The same situation was already faced roughly one decade ago, in context of BICC based network architectures. Thus, we need to revisit again BICC-defined H.248 tools and evaluate their possible reuse.

# 2	Discussion: package reuse candidates?

## 2.1	Overview – candidates from BICC/Q.1950

Figure 1 provides a copy of the Q.1950 architecture. The Recommendation itself defines a call bearer control protocol, which is effectively an H.248 profile specification (but before the H.248.1 profile template was developed) plus a set of H.248 packages.

Figure 1 – BICC "Q.1950-defined H.248 interface"

Q.1950 introduces eleven H.248 packages, following four (see Fig. 2) may be of interested in context of TLS and TCP:

Figure 2 – List of H.248 package candidates from Q.1950

Each of the four package is briefly evaluated in following subsections.

## 2.2	Candidate: H.248 bearer characteristics package (BCP)

The BCP package allow to “defines the characteristics of the bearer network connection”, which relates actually to the bearer technology given by the protocol or protocol stack at the link/network/transport protocol layer(s). Multiple bearer technologies are already supported by Property BCP/BNCChar, see Fig. 3:

Figure 3 – Existing codepoint assignment of Property BNC Characteristics

Conclusions:

- concept is interesting, package might be reused;

- if, then additional codepoint assignments needs to be defined (for TCP, TLS, TLS/TCP?, DTLS, DTLS-SRTP, SCTP, DCCP?)

- codepoint space: Q.765.5 value range “0x0006 to 0x00FF” or within “0x00FF to 0xFFFF”?

- extension package or upversioned package?

Alternative:

- H.248 text encoding: bearer type indication via (LD/RD embedded) SDP media description, given by the 2-tuple of {“c=” line & “m=” line element <proto>}

- Issue with SDP: redundancy in “m=” line element <proto> value assignments due to application specific and application agnostic “transport protocol” values.

## 2.3	Candidate: H.248 generic bearer connection package (GB)

Basic bearer connection control procedures are subject of the GB package, see Figure 4:

Figure 4 – Elements of the GB (generic bearer connection) package

The package provides the functionality to be able to establish/modify/release a bearer connection. H.248 Signals are used for triggering of bearer control protocol procedures (see Figure 5). An H.248 Event supports the (optional) notification of the MGC about state transitioning of the bearer connection.

Figure 5 – MGC triggers the MG level bearer control protocol procedures
(copy of Figures 6 and 7 from Q.Sup35)

Conclusions:

- package supports incoming and outgoing bearer control protocol (BCP) procedures, which are required for active/passive or client/server type of bearer connection endpoints

- package supports MGC notifications, which allows to support “MGC strictly” and “MG autonomous” BCP control models;

- package might be reused;

- open items:

  - the GB package makes an assumption of a state model for (generic) bearer connection endpoints, however, the model is not documented (to be confirmed, at least we couldn’t find something);

  - we need to consider such a state model when trying to get generic H.248 connection control tools for TCP, TLS, TLS/TCP (this aspect is subject of C-3)

- usage of GB package implies also support of the generic (g) package due to connection release handling

Alternative:

- Bearer type specific H.248 packages

## 2.4	Candidate: H.248 bearer network connection cut-through package (BNCT)

What’s cut-through exactly, - from perspective of MG and MGC? The semantic is unfortunately not fully clear:

Q.Sup35:

8.1.4	Cut_Through

The cut through primitive is used to through connect or break the user plane on the terminations. The Cut Through primitive can be used for this purpose in two ways. The implicit cut through is used when the termination is instantiated to allow the BIWF to control when the cut through of the media occurs. The explicit cut through is used when the CSM wants to control when the cut through of the media on the terminations occurs. To break the user plane the explicit cut through is used. The cut through transactions should be applied to the terminations in the context that need to be cut through.

Q.1950:

Term : BNC cut through

Definition : Indicates when the through connection of the BNC has occurred.

Source : Q.1950 (02), 4.4.9

Term : BNC-cut-through-capability

Definition : Used by the BIWF to inform the CSM of the bearer cut-through capability (i.e. commits resources on the receipt of a Bearer SetupReq or confirm).

Source : Q.1950 (02), 4.4.10

There’s a single Property (Fig. 6):

Figure 6 – BNCT Property: Bearer network connection cut-through capability

“This property allows the MGC to ask the MG when the cut through of a bearer will occur, early or late.” Hence, the MGC is not mandating the MG when the cut through shall occur, rather a question about the implemented cut-through behaviour by the MG!?

Conclusions:

- concept needs firstly to be clarified;

- “cut through” behaviour is a relevant aspect in case of TCP-to-TCP interworking, particularly concerning the TCP proxy mode because there’s direct feedback on the end-to-end TCP connection (such as possible TCP packet retransmissions in case of not (yet) forwarded packets by the MG)

- open: interaction between StreamMode and cut-through?

Alternative:

New Property for control of

- TCP connection or

- general connection-oriented IP transport connection

for (MG internal) bearer connection

- cut-through and

- break

behaviour, taking into account

- MGC strictly controlled (“explicit”) and

- MG autonomous (“implicit”)

behaviour.

## 2.5	Candidate: H.248 reuse idle package (RI)

Figure 7 indicates possible use cases of the RI package in context of TLS and TCP:

Figure 7 – RI package Property: Reuse idle indication

Conclusions:

- concept needs firstly to be clarified: what exactly means “idle” bearer? Seems to be the name of a state of a bearer connection endpoint state machine? (See also C-3)

- Requirements:

  - TCP connection reuse?

  - TLS session resumption?

- Open: such requirements transparent for MG? only relevant for MGC?

Alternative:

- Bearer type specific H.248 packages

# 3	Discussion: signaling flows for generic and TCP specific control

Draft H.248.TCP provides already example signalling flows, based on a provisional TCP connection control package. We’d like to compare them with a Q.1950 based alternative.

## 3.1	Bearer connection establishment

### 3.1.1	TCP specific control

Figure 8 recalls again the bearer connection establishment for TCP specific control (i.e., a copy for Figure II.2 from H.248.TCP).

The H.248 Context creation process is characterized by two H.248 signalling cycles:

- Reservation and allocation of two TCP stream endpoint resources; TCP client/server role not yet decided, hence not yet any BCP TCP connection control procedures;

- TCP role assignment, then subsequent BCP TCP connection control procedures (T1: passive open; T2: active open)

This relates to an MGC strictly controlled bearer connection establishment. The MGC did arm the event to be notified about successfully established TCP connection (segments) at each side.

Figure 8 – Bearer connection establishment – TCP specific control

### 3.1.2	Generic control

Figure 9 provides an attempt for a generic control based approach, using three Q.1950 H.248 packages: unmodified GB (and BNCT) package(s) and an extended BCP package with a “TCP” codepoint. It has to be noted that the usage of BNCT is still under discussion (see clause 2.4).

Figure 9 – Bearer connection establishment – Generic control

On a first glance could be concluded that TCP bearer connection establishment could be also achieved via generic H.248 tools.

## 3.2	Bearer connection release

### 3.2.1	TCP specific control

See H.248.TCP.

### 3.2.2	Generic control

For further studies.

# 4	Discussion: generic vs silo approach?

H.248 package design framework: two fundamental options (Figure 10):

Figure 10 – H.248 package design framework: two fundamental options

Some aspects:

- Commonalities: connection-oriented bearer protocols
Note: connection-oriented = statefull connection endpoint

- State model: explicit specification required or not?

- Handling of connection reuse, session resumption, … hm? (Or just exclude such characteristics as H.248 requirement?)
What would be a reused/resumed “MG resource” from H.248 perspective?

- MG mode of operation: support of two options “MG autonomous” & “MGC strictly controlled” complicates the package design (however, actually already considered by BICC, too); see also C-43.

However, following items needs to be considered:

- Differentiators – General:

- Communication types: symmetrical (peer-to-peer), asymmetrical (client-server)
=> different endpoint role behaviour
Hm, any impact on number of states …? (No)
Different state transitioning logic …? (No)
[… because such a bearer connection level characteristic is only visible at the application level … above the service access point (SAP) … and the MG is “below the SAP”]
=> consequences: instead H.248 property for endpoint roles rather H.248 signals/events for state transitioning control? (as H.248 bnct package)?

- Differentiators – TCP:

- Assured transport service: loss detection, timeouts, retransmissions

- Rate control: delayed acknowledgements, window size control, slow start

- Unidirectional connection release: half-close

# 5	Conclusions

The generic approach seems to be a feasible alternative versus bearer specific H.248 control tools. However, there are still some open items, thus not any concrete actions requested yet.

NOTE – Parallel contributions on H.248 package design evolution proposals with respect to

  - H.248.TCP: C-13 and C-11;

  - H.248.TLS: C-14 and C-20.

_________________

Q.1950 (12/2002): Bearer independent call bearer control protocol