|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 5 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 5 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP: Editorial update proposals |  |  |  |

Summary

This contribution provides editorial update proposals for progressing Draft H.248.TCP. The proposed changes are marked up against the proposed input draft to this meeting TD 19 (WP 2/16).

There are concrete editorial changes proposed as well as some electronic comments inserted.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 19 (WP 2/16).

[Begin Proposal]

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.TCP Ed. 0.1 | Input (AVD-4297) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4297.zip |
| H.248.TCP Ed. 0.2 | Output (TD-25) from WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-25.zip |
| H.248.TCP Ed. 0.3 | Input (TD 19 (WP2/16)) from ITU-T SG16 Geneva meeting, Jan 2013 Location: |
| H.248.TCP Ed. 0.4 | Output (TD 19R1 (WP2/16)) from ITU-T SG16 Geneva meet., Jan 2013 Location: |
| H.248.TCP Ed. 0.5 |  |
| H.248.TCP Ed. 0.6 |  |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 5 4 Abbreviations and acronyms 5 5 Conventions 6 6 Motivation and use cases 6 7 Relation to similar H.248 packages 6 7.1 ITU-T Q.1950 6 9 TCP connection control package 7 9.1 Properties 7 9.1.1 TCP connection setup role 7 9.1.2 MG Autonomous Bearer Release Indicator 8 9.1.3 Duplex Closure Indicator 8 9.2 Events 9 9.2.1 TCP connection established 9 9.2.2 TCP connection closed 9 9.3 Signals 9 9.3.1 Close 9 9.4 Statistics 9 9.4.1 Number of Retransmitted TCP-Segments 9 9.4.2 Number of Retransmitted TCP-Octets 10 9.5 Error Codes 10 9.6 Procedures 10 9.6.1 TCP endpoint creation 10 9.6.2 TCP connection establishment 10 9.6.2 Path MTU Discovery 12 9.6.4 Interaction of TCP-retransmissions with traffic control functions 12 Appendix I Sample use-cases of TCP connection control 14 I.1 Use case #1: WebRTC to NGN/IMS interworking function with termination of transport security by MG 14 Appendix II Example call flows 15 II.1 Assignment of TCP mode of operation 15 II.1.1 Mode type: application agnostic TCP-proxy 15 Bibliography 22

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure I.1 – Use-case #1: WebRTC to IMS interworking function 15 Figure II.1 – Configuration “application agnostic TCP-proxy” 16 Figure II.2 – Traffic flow example for an MG autonomous connection setup using the TCP-proxy mode 18 Figure II.3 – Traffic flow example for an MG autonomous release of the TCP connection using the TCP-proxy mode 19 Figure II.4 – Traffic flow example for an MGC controlled connection setup using the TCP-proxy mode 21 Figure II.5 – Traffic flow example for an MGC controlled connection release using the TCP-proxy mode 22

Draft new Recommendation ITU-T H.248.xx (ex H.248.TCP)

Gateway Control Protocol:
TCP support packages

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

{Editor’s note (2012-09): ###). Contributions are solicited.}

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DTLS | Datagram Transport Layer Security |
| --- | --- |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| RD | Remote descriptor |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SIP | Session initiation protocol |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| UDP | User datagram protocol |
| UE | User equipment |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

# 6	Motivation and use cases

# 7	Relation to similar H.248 packages

## 7.1	ITU-T Q.1950

7.2	H.248.84

The TCP enabled SEP will be activated through the indication of the TCP-proxy mode as defined in [ITU-T H.248.84]. This Recommendation H.248.TCP only defines properties that are needed on top of [ITU-T H.248.84] for the control of a TCP enabled SEP.

This Recommendation does not define TCP-related statistics and events which are already covered by [ITU-T H.248.84]. Whenever needed those statistics and events of the related packet as defined by [ITU-T H.248.84] must be used.

…

# 8	Generic bearer transport session or connection control package

{Editor’s note (2012-09): this option is still under study. The well-known idea in separating technology-independent (“bearer-independent”), common control functionality and technology-dependent capabilities (as e.g. package extensions). }

# 9	TCP connection control package

|  | Package Name: | TCP connection control package |
| --- | --- | --- |
|  | Package ID: | tcpcc (0x####) |
|  | Description: | This package provides tools for the MGC for controlling TCP endpoints instantiated on a SEP. For a MGC strictly controlled approach the MG can notify the MGC about state changes of the TCP connection. State changes of the TCP connection will not be propagated between the SEPs. By the use of signals the MGC can control each SEP independently. The following capabilities are provided: - The MGC can trigger the setup of a TCP connection, instructing the TCP endpoint acting either as a TCP-client or as a TCP-server; - The MG can notify the MGC in case the setup of the TCP connection has completed; - The MG can notify the MGC in case the closure of the TCP connection has completed ; - The MGC can instruct the MG to release the TCP connection; In case no explicit TCP connection control is required by the MGC, the TCP connection can be setup and released by the MG in an autonomous manner by propagating TCP connection state changes between the SEPs. |
|  | Version: | 1 |
|  | Extends: | None |

## 9.1	Properties

## 9.1.1	TCP connection setup role

|  | Property Name: | TCP connection setup role |
| --- | --- | --- |
|  | Property ID: | csr (0x0001) |
|  | Description: | This property defines the MG behaviour for the TCP connection setup. |
|  | Type: | Enumeration |
|  | Possible values: | “notset” (0x0001) This value indicates that the MGC has not yet provided any information whether the MG shall act as a TCP server or TCP client. “client” (0x0002) This value indicates that the MG will initiate the TCP connection setup as a TCP-client. “server” (0x0003) This value indicates that the MG will act as a TCP-server. |
|  | Default: | “notset” |
|  | Defined in: | Local |
|  | Characteristics: | Read/Write |

## 9.1.2	MG Autonomous Bearer Release Indicator

|  | Property Name: | MG Autonomous Bearer Release Indicator |
| --- | --- | --- |
|  | Property ID: | abri (0x0002) |
|  | Description: | This property defines if a detected condition leading to the closure of the TCP connection will trigger an MG-autonomous release of the bearer connection of the related SEP as well. |
|  | Type: | Boolean |
|  | Possible values: | True On closure of the TCP connection the MG will autonomously release the bearer connection on the other related SEP as well. False The bearer connection of the other SEP is not affected by the closure of the TCP connection. |
|  | Default: | True |
|  | Defined in: | LocalControl {Editor’s note (2012-09): Its unclear how this property relates to the other Terminations/Streams in the Context. i.e. if a release is triggered as a result of this property are other terminations affected if they don't have this property set? Are events triggerred on these other terminations. Might be also a Context-level property? For further study.} |
|  | Characteristics: | Read/Write |

## 9.1.3	Duplex Closure Indicator

|  | Property Name: | Duplex Closure Indicator |
| --- | --- | --- |
|  | Property ID: | dci (0x0003) |
|  | Description: | This property defines the behaviour in the MG in case a TCP-FIN is received for an established TCP connection. It instructs the MG whether or not to send a TCP-FIN. |
|  | Type: | Boolean |
|  | Possible values: | True Indicate the duplex closure of the connection by sending a TCP-FIN autonomously. False Keep the TCP connection in the half-closed state, i.e. the MG will not generate the TCP-FIN autonomously. |
|  | Default: | False |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 9.2	Events

### 9.2.1	TCP connection established

|  | Event Name: | TCP-connection established |
| --- | --- | --- |
|  | Event ID: | estab (0x0001) |
|  | Description: | This event notifies the MGC that the TCP connection setup is finished, i.e. the TCP connection setup handshake has completetd and application data may flow bi-directionally. |

#### 9.2.1.1	EventsDescriptor parameters

None

#### 9.2.1.2	ObservedEventsDescriptor parameters

None

### 9.2.2	TCP connection closed

|  | Event Name: | TCP connection closed |
| --- | --- | --- |
|  | Event ID: | closed (0x0002) |
|  | Description: | This event allows the MGC to be notified when the TCP connection state returns to the “CLOSED” state as defined in [IETF RFC 793]. |

#### 9.2.2.1	EventsDescriptor parameters

None

#### 9.2.2.2	ObservedEventsDescriptor parameters

None

## 9.3	Signals

### 9.3.1	Close

|  | Signal Name: | Close |
| --- | --- | --- |
|  | Signal ID: | close (0x0001) |
|  | Description: | This signal is used to close the TCP connection. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |
|  | Additional Parameters: | None |

## 9.4	Statistics

### 9.4.1	Number of Retransmitted TCP-Segments

|  | Statistic Name: | Number of Retransmitted TCP-Segments |
| --- | --- | --- |
|  | Statistic ID: | retxseg (0x0001) |
|  | Description: | This statistics logs the number of TCP-fragments which are retransmitted due to the expiration of the acknowledgement timer. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 9.4.2	Number of Retransmitted TCP-Octets

|  | Statistic Name: | Number of Retransmitted TCP-Octets |
| --- | --- | --- |
|  | Statistic ID: | retxoct (0x0002) |
|  | Description: | This statistics logs the number of TCP-octets which are retransmitted due to the expiration of the acknowledgement timer. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### {Editor’s note (2012-09 meeting): other TCP related metrics should be used from H.248.84, packages  tcphp, tcptv, tcpccm}

## 9.5	Error Codes

None

## 9.6	Procedures

### 9.6.1	TCP endpoint creation

The MGC is responsible for creating a TCP connection control enabled SEP in the MG. This will be indicated on the H.248 interface through the use of the “m=” line as well as the “a=setup” attribute for the affected termination. The rules as specified in [ITU-T H.248.84], 13.4 and 13.5 apply.

{Editor’s note (2012-09 meeting): H.248.84 uses the term « TCP-proxy mode », it might be more appropriate to use the term « TCP connection control mode » instead and to define the « TCP-proxy mode » as an application of the TCP connection control mode.}

Whether this SEP will act as a TCP client or TCP server is indicated by the property Connection Setup Role (csr). In case the role is not yet determined at the time of the SEP’s creation, the value “notset” will be used, and any received TCP segments for this SEP are silently (?) dropped by the MG.

### 9.6.2	TCP connection establishment

In case the role is not yet determined at the time of the SEP’s creation, the value “notset” will be used for the property Connection Setup Role (csr). Any received TCP segments for this SEP are dropped by the MG in this case.

Once the role of the TCP endpoint is determined by the MGC the property Connection Setup Role (csr) will be set either to “client” or to “server”. The MG will initiate the TCP connection setup according to the indicated role and the TCP connection setup handshake is performed by the MG without any further involvement of the MGC.

The event TCP connection established (estab) will indicate the completion of the three-way-handshake. It corresponds to a state change of the TCP state machine to “ESTABLISHED” as defined in [IETF RFC 793].

## 9.6.3	TCP application data transfer

Once the application data may flow between the termination and the remote TCP-endpoint. The event TCP connection established (estab) does not provide any statement regarding the application data flow between the two SEPs within the same H.248 context.

Thus application data may be received at a SEP in time where the other stream endpoint of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent.

{Editor’s note (2012-09 meeting): Just two options:

- Acknowledge and buffer the received application data

- Drop the application data and rely on the TCP-retransmission until the other stream endpoint is able to accept and process the application data}

The stream mode property of the Local Control Descriptor affects the application data flow rather than the TCP control information used to setup or close the TCP connection.

{Editor’s note (2012-09 meeting): Consensus required on this behaviour.}

TCP provides the application the capability to affect the data application transfer through the PSH- and the URG-flag. The usage of these flags is application specific and therefore must be specified in the application specific procedures.

## 9.6.4	TCP connection release

The TCP connection release might require different behaviours in the MG, depending on any combinations of

- the application
E.g. TLS to non-TLS interworking; a TCP-release on termination Ta might result in a TLS-release procedure on termination Tb.

- the used transport protocol on the other SEP
E.g. TCP to UDP interworking; no connection release for UDP

- the involvement of the MGC
E.g. the MGC might be involved actively in the release sequence (MGC controlled behaviour)

- …
{Editor’s note (2012-09 meeting): other use cases to be added}

The required behaviour is controlled through the properties MG Autonomous Bearer Release Indicator (abri) and Duplex Closure Indicator (dci).

If the property MG Autonomous Bearer Release Indicator (abri) is set to True, a TCP closure condition that has been indicated by the remote TCP-endpoint or that has been detected locally by the SEP will be passed to the other SEP of the stream to release the bearer connection there as well.

If the property MG Autonomous Bearer Release Indicator (abri) is set to False, a detected release condition will not affect the bearer connection on the other SEP. In this case the bearer connection of the other SEP might be released explicitly by the MGC, by the corresponding remote bearer connection endpoint or by other means.

The property Duplex Closure Indicator (dci) controls whether a received half-closure indication from the remote TCP endpoint shall lead to the closure of the complete TCP connection or not. If set to True, a TCP-FIN received for an established TCP connection will lead to the generation of a TCP-FIN by the MG without any involvement of the other SEP.

In case the closure of the TCP connection results in the generation of a TCP-FIN, the MG is required to send the TCP-FIN only after all previous TCP segments for that termination have been sent and have been acknowledged by the remote TCP endpoint.

The closure of a TCP connection is completed according to the procedures as defined in [IETF RFC 793] if the state “CLOSED” is reached. The event TCP connection closed (closed) may inform the MGC about this state change.

## 9.6.4.1	TCP-FIN received from remote TCP endpoint

A received TCP-FIN is acknowledged autonomously by the MG. The MGC can be notified of this event by the use of the TCP hole punching package as defined in [ITU-T H.248.84], see event tcphp/rrel.

In case Duplex Closure Indicator (dci) is set to True, the MG will add a TCP-FIN for transmission to its send buffer. After the TCP-FIN is sent and acknowledged by the remote TCP-endpoint, the TCP connection is closed and the MGC is notified of the event closed if requested.

In case Duplex Closure Indicator (dci) is set to False, the connection will stay in the half-closed state, i.e. application data received from the other SEP will still be transmitted. In this case the connection must be closed by other means, e.g. by an explicit close signal from the MGC or by a received bearer connection release indicator from the other SEP.

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the received TCP-FIN indication will be passed to the other SEP to trigger the release of the bearer connection there as well. If MG Autonomous Bearer Release Indicator (abri) is set to False, the other SEP will not be affected by the TCP-FIN. In this case the other SEP must be released by other means, e.g. by a signal from the MGC or by a release indication received externally.

## 9.6.4.2	Close signal received from MGC

The MGC may send the TCP connection closed (closed) signal to trigger the closure of the TCP connection. If this signal is received while the connection is open in the send direction (i.e. either the connection is “established” or it was “established” and the remote TCP endpoint indicated a half-closure by having sent a TCP-FIN), then the MG will send a TCP-FIN. The signal close will not affect the other SEP of the same stream. If desired it is up to the MGC to release this SEP accordingly.

## 9.6.4.3	Close indication received from SEP

In case a bearer release indication is received from the other SEP (e.g. it is a TCP endpoint, configured with abri=True and a TCP-FIN has been received) while the TCP connection is open in the send direction, then the MG will send a TCP-FIN.

### 9.6.2	Path MTU Discovery

{Editor’s note (2012-09 meeting): Further study required.}

### 9.6.4	Interaction of TCP-retransmissions with traffic control functions

{Editor’s note (2012-09 meeting): Further study is required.}

Appendix I

Sample use-cases of TCP connection control

(This appendix does not form an integral part of this Recommendation.)

## I.1	Use case #1: WebRTC to NGN/IMS interworking function with termination of transport security by MG

This use case shows an MG which provides a transport layer interworking function between a WebRTC-based data application and a UE located in a NGN or IMS network.

Background:

Web-based Real-Time Communication Services (WebRTC) include non-media (data) applications, which are using a datagram connection for data transport (see [b-IETF rtcweb-data]). The selected protocol solution results in a hierarchical protocol layering (data-over-SCTP-over-DTLS-over-ICE/UDP), driven by NAT traversal complexity, multiplexing and security related aspects.

The considered data applications (by WebRTC) would use legacy TCP transport in non-WebRTC IP network environments. [ITU-T H.248.WebRTC] considers several variants of H.248 WebRTC gateways which are positioned for WebRTC interworking scenarios.

The setup of termination T2 is controlled by the TCP connection control package while the setup of termination T1 is out of scope of this Recommendation.

Figure I.1 – Use-case #1: WebRTC to IMS interworking function

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation.)

## II.1	Assignment of TCP mode of operation

### II.1.1	Mode type: application agnostic TCP-proxy

The MGC needs to assign the TCP mode of operation to an H.248 context. This is fundamentally based on SDP information elements according [ITU-T H.248.84], clause 13.5. An example is provided by clause II.1.1.1.

The MGC could use also additional explicit H.248 signalling elements for mode control as defined by this Recommendation, see example in clause II.1.1.2.

#### II.1.1.1	Mode assignment and MG autonomous TCP connection control

The call flow is based on the network configuration as shown in Figure II.1:

Figure II.1 – Configuration “application agnostic TCP-proxy”

The use case shows an MG with two SEPs (labelled as T1(S1) and T2(S1)) configured in an application agnostic TCP-proxy mode. The configuration and the call flow are characterized as follows:

- MG autonomous control of the TCP endpoints on both terminations

- Termination T1: TCP-server

- Termination T2: TCP-client

- TCP-connection for both terminations are setup in parallel

- TCP-connection released by remote TCP-endpoint of termination T1; application data may still flow from UE-2 to UE-1.

Figure II.2 illustrates a traffic flow example concerning the establishment of an end-to-end TCP connection between UE-1 and UE-2, which relates to two TCP connection segments from H.248 MG perspective (due to TCP proxy mode).

The example traffic flow indicates parts of application control signalling (here SIP/SDP with scope on SDP Offer/Answer information), gateway control signalling (H.248) and bearer plane traffic (here TCP packets for connection establishment).

Figure II.3 illustrates the traffic flow example for the release of the TCP connection without any involvement of the MGC. The release procedure on the signalling path is not shown.

Figure II.2 – Traffic flow example for an MG autonomous connection setup 
using the TCP-proxy mode

Figure II.3 – Traffic flow example for an MG autonomous release of the TCP connection using the TCP-proxy mode

#### II.1.1.2	Mode assignment via explicit TCP connection control means

This use case applies to the same configuration than the previous use case as depicted in Figure II.2. The configuration and the call flow are characterized as follows:

- MGC controls the TCP endpoints on both terminations

- Termination T1: TCP-server

- Termination T2: TCP-client

- TCP-connection for both terminations are setup in parallel

- TCP-connection released by remote TCP-endpoint of termination T1; application data may still flow from UE-2 to UE-1

Figure II.4 and Figure II.5 illustrate a traffic flow example where the MGC explicitly controls the setup and the release of the TCP endpoints at both terminations T1 and T2. The release procedure on the signalling path is not shown.

Figure II.4 – Traffic flow example for an MGC controlled connection setup using the TCP-proxy mode

Figure II.3 provides an example for release of the TCP connection.

Figure II.5 – Traffic flow example for an MGC controlled connection release 
using the TCP-proxy mode

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

________________________