|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 7 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | October 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 7 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP: Clause 3 – Terminology update proposals |  |  |  |

Summary

This contribution provides change proposals for progressing draft H.248.TCP. The proposed changes are marked up against the proposed input draft to this meeting TD 19 (WP 2/16).

# Discussion

This change request is based on two proposals:

- Move from two terms from H.248.TLS (see also C-0017);

- New terms related to H.248 control modes as discussed in C-0043, which might be also positioned in H.248.1 IG.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 19 (WP 2/16).

[Begin Proposal]

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	transport (TCP) proxy (translator) mode [ITU-T H.248.84], see clause I.4.1 in [b-ETSI TR 183 068]: mode (also known as Back-to-Back TCP Endpoint (B2BTE) mode): statefull forwarding of TCP packets in terms of full protocol termination. The end-to-end TCP connection is partitioned in two TCP connection legs by the BGF. Each H.248 Stream endpoint provides a statefull TCP connection state machine.

NOTE 3 – The term proxy mode is similar as used for HTTP proxy, FTP proxy, SIP proxy, etc.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.x0	Basic H.248 master-slave control mode: {see proposal from C-0043, clause 3; for discussion: term added to H.248.1 IG instead of H.248.TCP?}

3.2.x1	MG autonomous mode: {see proposal from C-0043, clause 3; for discussion: term added to H.248.1 IG instead of H.248.TCP?}

3.2.x2	MGC-strictly controlled mode: {see proposal from C-0043, clause 3; for discussion: term added to H.248.1 IG instead of H.248.TCP?}

3.2.x3	MGC-smartly controlled mode: {see proposal from C-0043, clause 3; for discussion: term added to H.248.1 IG instead of H.248.TCP?}

3.2.3	TCP proxy variants: the basic TCP proxy mode is defined in clause 3.1.3. The following TCP proxy variants are useful for the various TCP services as supported by H.248 MGs:

There are two orthogonal attributes:

- Discrimination with respect to TCP payload data awareness:

- Application-aware TCP proxy (the MG knows the carried protocol (stack) by TCP packets)

- Application-agnostic TCP proxy (the MG is unaware of the TCP payload content)

- Discrimination with respect to statefull TCP connection control:

- Statefull TCP proxy (by support of the complete TCP connection state machine, see Figure 6 in [IETF RFC 793])

- Lightweight TCP proxy (by support of simplified TCP connection state models)

Example: “application-agnostic, statefull TCP proxy” mode.

[End Proposed Correction]

_________________