|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 29 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 29 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.1v3 Implementors' Guide: addition of a new terms related to H.248 stream endpoint entities |  |  |  |

Summary

New terminology is proposed.

Background

This is a resubmission of AVD-4279 from the Brisbane meeting, which agreed to position these terms as H.248.1v3 IG item, rather than subject of H.248.1v4, which make sense because the proposed terms are protocol version independent.

AVD-4279 - H.248.1 V4 Living list: addition of a new term [Alcatel-Lucent]

This document was discussed and it was agreed that this could be an Implementors’ guide item. Contributions are solicited for the next meeting. It was thought that this could be included into H.248.1v3 and then other H.248.x Recommendation could then refer to this in the “Terms defined elsewhere” clause.

Discussion again:

Identifier perspective:

- Term stream is primarily a horizontal concept within a context: a stream with its associated identifier may affect a single, two or multiple terminations. The stream could be seen as termination-independent from that point of view.

- Term stream endpoint shall emphasize the binding of a stream to a specific termination; could be thus considered as a vertical concept.

The vertical view and the notion of stream endpoint (SEP) is in the meanwhile used in a couple of published and Draft H.248.x-Recommendations, but not yet explicitly defined. H.248.1 would be the appropriate place.

Further, a pair of SEPs and in general a tuple of SEPs are subsequent concept, based on the SEP. Correspondent terms are suggested.

Proposal

To H.248.1v3 IG:

[Begin Proposed Correction]

3.1.x	stream endpoint (SEP): relates to a stream associated to a termination. A SEP is identified by the tuple of StreamID and TerminationID values.

3.1.y	stream endpoint pair (SEPP): two associated Stream Endpoints (SEP) of two Terminations within the same Context. The two SEPs share thus the same StreamID value. There is always a point-to-point topology for Stream Endpoint Pair configurations (e.g., there shall be not any further associated SEP on a third termination etc).

3.1.z	stream endpoint tuple (SEPT): the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

[End Proposed Correction]

____________________