|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 43 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 43 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248 control models: “MGC strictly controlled mode” vs “MG autonomous mode” |  |  |  |

# Summary:

The H.248 decomposed gateway model of MGC/MG entities allows many different models concerning the distribution of control logic between the master and his slave entities. A coarse granular classification distinguishes two established models, often termed as “MGC strictly controlled mode” and “MG autonomous mode”.

These models are again in consideration with respect to H.248 controlled

- connection-oriented IP transport bearer connections (TCP, SCTP),

- IP session levels protocols for transport security (TLS, DTLS, DTLS-SRTP), and

- tunnelled IP (transport) protocols (e.g. WebRTC data with SCTP as tunnel).

Issue: both models are already explicitly described (in their application specific context), but not yet defined as an explicit term.

Discussion: in order to limit (firstly) the scope, H.248.TCP should provide correspondent terminology, either generic or TCP specific or both. This is still open.

Purpose of this contribution is to elaborate characteristics and differences of each control mode.

Table of contents:

Summary:	1

1	Background:	2

2	Discussion	3

2.1	Reminder: problem statement in case of TLSoTCP bearer types	3

2.2	Gateway decomposition model: state-of-the-art (2012)	4

2.2	Basic control models - Overview „MG autonomous“ vs „MGC-strictly controlled“	5

2.2.1	Overview	5

2.2.2	General model „MGC-strictly controlled“ in more detail	6

2.2.3	Special model „MG autonomous“ in more detail	7

2.3	Extended control models	8

2.3.1	Refined model „MGC-smart controlled“	8

2.3.2	Reminder: “SCP-smart controlled“ model from IN	9

2.4	Bearer type ‘TCP’: end-to-end TCP bearer connection control	9

2.4.1	Major control functions	9

2.4.2	End-to-end TCP bearer connection control vs TCP modes	10

2.4.3	Conclusions for TCP	11

3	Terminology proposals	11

4	Summary	12

Appendix I: Gateway decomposition options (BICC network architecture)	13

---------------

# 1	Background:

The two H.248 control models are already known and in use (“with some flavours”), for instance:

- H.248 Event-/Signal-coupled follow-up activities (“embedded descriptors”);

- Realtime fax over IP: H.248 controlled T.38 service (see T.38 Annex E);

- Compelled CAS: H.248 controlled signalling system R2 interfaces (H.248.29 Annex A vs B);

- Voiceband data over IP: MG autonomous V.152 VBDoIP service vs a MGC strictly controlled emulation;

- Cut-through behaviour: Q.1950/H.248 bnct package (bearer network connection cut-through);

- NAT traversal support: bearer-level ALG according H.248.78, which defines both modes in clauses 6.1 and 6.2;

- Bearer-level codec broker: codec related resource management entirely delegated from call control level down to MG;

- Bearer-level “SDP B2BUA”: SDP session description processing remains at SIP B2BUA level, but SDP media description handling (and all associated resource management functions) delegated to MG;

- others.

# 2	Discussion

## 2.1	Reminder: problem statement in case of TLSoTCP bearer types

See Figure 1: the two termination context (with a TLS/TCP/IP SEP (Stream endpoint) and TCP/IP SEP) is controlled by a “media & bearer control function” (MBCF), which is somehow distributed across MGC and MG entities in the general case.

Figure 1 – Bearer Connection Control: Problem Statement 
(here: TLSoTCP and TLS-to-non-TLS interworking)

Some control aspects may be noted:

- IP layer:

  - B2BIH (Back-to-Back IP Host) mode? or

  - IPR (IP Router) mode?

  - … IPv6-to-IPv6

- TCP layer:

  - TCP mode? (relay, merge, proxy)

  - TCP endpoint role (client|server)?

  - successful NAT-T … reuse of TCP connection …

- TLS layer

  - role (client|server)?

- MG location:

  - Anchor for possible access bearer transfer (like in an access transfer gateway (ATGW))?

  - Access point for legal interception?

Conclusions:

Package design (e.g., H.248.TCP) must serve/consider:

- different H.248 profiles (such as for access and peering)

- optional local NAT and optional local NAT-T

- MGC strategy

- …

We’d like to go consider the two H.248 control models in more detail.

NOTE: reader should be familiar with

Q.Sup31: Technical Report TRQ.2141.0 (12/2001): Signalling requirements for the support of narrow-band services over broadband transport technologies

- due to concept of functional-to-physical mapping

- due to principal gateway decomposition options as elaborated in Annex C

- due to introduced terminology

Appendix I to this contribution summarizes the main functional-to-physical mapping models from Q.Sup31.

## 2.2	Gateway decomposition model: state-of-the-art (2012)

The “two physical unit model” (from Q.Sup31/Figure C.3) could be considered as starting point of todays gateway decomposition models: Figure 2 provides an illustration with MGC and MG as physical units and high level generic functions (at the example of a SIP network).

Figure 2 – Gateway decomposition model: state-of-the-art (2012)

Primary scope of this contribution is the discussion about the MBCF and possible distribution models. Media Bearer Control Function (MBCF): what kind of information may basically trigger control decisions?

E.g., MGC-level MBCF part (MGCF):

- MGC external: call control signalling

- MGC internal: algorithms for MG selection, load control, QoS control, etc

- MG external: bearer interfaces

- MG internal: algorithms for …

Note: (3) & (4) relates to H.248 MG-to-MGC signalling.

## 2.2	Basic control models - Overview „MG autonomous“ vs „MGC-strictly controlled“

### 2.2.1	Overview

Figure 3 provides a high level comparison of both control modes. We need to focus on the MG-level MBCF part only (due to MG autonomous mode). MG autonomy implies the existence of a Local Control Function (LCF), able to make MG-local control decisions.

Figure 3 – Lineup „MG autonomous“ vs „MGC-strictly controlled“

### 2.2.2	General model „MGC-strictly controlled“ in more detail

This H.248 control mode represents the general case, expressed by the notion of master-slave relationship between MGC and MG. The very, very majority of control decisions (of the MBCF) is up to the master. Also due to the fact that the control logic for “media & bearer” is tightly coupled to call/service control logic

When such a control decision would require the consideration of MG level information, then such information must be signaled by the MG towards the MGC (see Figure 4). E.g., based on H.248 Notify or ServiceChange mechanisms.

Figure 4 – General model „MGC-strictly controlled“ in more detail

### 2.2.3	Special model „MG autonomous“ in more detail

Figure 5 depict the MG autonomous mode:

Figure 5 – Special model „MG autonomous“ in more detail

MG provides a local decision logic (“intelligence”), however, decisions are limited with scope on media & bearer aspects only (but not call). There are following main advantages:

- Offloading the MGC! (particularly from minor tasks)

- performance: control responsiveness (e.g., faster reactivity)

- less H.248 signalling

MG autonomous behaviour is already supported in H.248 by e.g. embedded descriptors (as explicit syntax) or as H.248 profile defined behaviour (implicit).

## 2.3	Extended control models

The basic control models are characterized by the fact that an H.248 Control Association is either using method A or B for all calls. E.g., in present H.248 profile specifications is T.38 FoIP either using the “MGC transitioning method“ or the „(MG) autonomous transitioning method” exclusively for all calls. More flexibility would be a model which allows the MGC to decide call-individually which control behaviour. Such a model may be called „MGC-smart controlled“.

### 2.3.1	Refined model „MGC-smart controlled“

Figure 6 outlines the basic difference to the basic models: the existence of “detection points” in the control logic, which decide finally whether a MG level decision is made or whether the MGC would be responsible for that.

Figure 6 – Refined model „ MGC-smart controlled“

To summarize the main idea:

- principal support of both control models

- MGC decides which control mode

- Decision at the level of root, context, … stream

- statefull control models …

- … with programmable state transitioning behaviour

- (re-)using concepts from IN (Intelligent Network) (see next clause)

### 2.3.2	Reminder: “SCP-smart controlled“ model from IN

Well-known control decomposition concept from „Intelligent Networks“, see Figure 7.

Figure 7 – “SCP-smart controlled“ model from IN

The core of such a control model is a defined state model, with the capability in “programming state transitioning behaviour”.

## 2.4	Bearer type ‘TCP’: end-to-end TCP bearer connection control

### 2.4.1	Major control functions

The end-to-end TCP bearer connection control could be abstracted by two (disjoint?) major functions:

- Bearer Control Protocol (for TCP) procedures, at the MG external bearer interface, divided in

  - ESTablishment function and

  - RELease function

- User Plane Connectivity (UPC) function, the MG internal interworking function concerning control of

- Cut-through function and

- Break function

At the example of Figure 8: there would be a “BCP for TCP for SEP T1(S1)”, a “BCP for TCP for SEP T2(S1)” and a common “UPC for TCP”. There is the fundamental question whether

- all three functions would be more isolated at MG level (hence “MGC-strictly controlled”); or

- interaction between all three functions would be allowed (i.e., “MG autonomous” behaviour).

Figure 8 – Major control functions

It may be noted that there are meaningful use cases for all combinations, e.g.:

- incoming BCP REL.requests leads to an autonomous Break, but not yet to an autonomous outgoing BCP REL.request at the partner SEP;

- MGC strictly controlled TCP bearer connection establishment, and MG autonomous cut-through, and MG autonomous TCP bearer connection release;

- etc etc

### 2.4.2	End-to-end TCP bearer connection control vs TCP modes

There’s a possible dependency between TCP modes (see H.248.84 and H.248.TCP) and end-to-end TCP bearer connection control. Table 1 provides an initial analysis:

Table 1 – Support of end-to-end TCP bearer connection control by TCP modes

| End-to-end TCP bearer connection control |  | Mode "TCP Relay" | Mode "TCP Proxy" | Mode "TCP Merge" |
| --- | --- | --- | --- | --- |
| 1. MGC strictly controlled mode: |  | (NOTE 1) | (NOTE 2) |  |
|  | 1.1: Establishment | No | Yes | No (NOTE 4) |
|  | 1.2: Release | No | Yes | N.A. (NOTE 5) |
| 2. MG autonomous mode: |  | (NOTE 1) | (NOTEs 2, 3) |  |
|  | 2.1: Establishment | Yes | Yes | Yes (NOTE 4) |
|  | 2.2: Release | Yes | Yes | N.A. (NOTE 5) |
| NOTE 1 – The relay mode provides only “MG autonomous” behaviour concerning end-to-end TCP bearer connection control according the definition (clause 3.1.2/[ITU-T H.248.84]). NOTE 2 – MGC decides which control mode via a package-property (e.g., tcpcc/abri in H.248.TCP 2012/09 package design; or tcpecc/sepplink in H.248.TCP 2013/01 package design proposal (see C-0011 & C-0013)). NOTE 3 – The “MG autonomous” behaviour is conditional, dependent on the “TCP proxy variant” (see clause 3.2.x/[ITU-T H.248.TLS¦.TCP]). E.g., a lightweight TCP proxy should at least support a minimum state model which allows to trigger establishment/release procedures with the H.248 TCP SEPP. NOTE 4 – The merge mode represents a “MG autonomous” behaviour concerning the self-contained resolution of the two separated TCP connection segment control procedures (“overall TCP simultaneous open scenario”) and MG internal cut-through (clause 3.2.2/[ITU-T H.248.84]). NOTE 5 – Not applicable because out of scope of the merge mode, which is merely used in the context of NAT traversal. And NAT-T is only relevant during connection establishment phases. |  |  |  |  |

### 2.4.3	Conclusions for TCP

Some observations:

- TCP connection phases: the establishment phase and release phase should be separated; they are different concerning 
a) realtime performance requirements, 
b) NAT-T assistance, 
c) H.248 control mode and 
d) TCP connection state machine (establishment: always twoway; release: oneway or twoway);

- User plane connectivity: cut-through / break of internal TCP packet flow forwarding (accompanied by the availability and usage of TCP protocol control information (“header flags and sequence numbers”) by the partner H.248 SEP)

- H.248 control modes: “MGC strictly controlled mode” and “MG autonomous mode” could be applied both for the same context, for different bearer connection control phases

# 3	Terminology proposals

Following definitions are proposed as working baseline:

| Mode | Definition: |
| --- | --- |
| Basic H.248 master-slave control mode: | An H.248 decomposed gateway following an hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship. With MGC as master and MG as slave entity due to considered distribution models on the decomposed gateway entities Note: [ITU-T Q.Sup31] Annexes C.2 or C.3 provides examples of functional-to-physical mapping models according basic H.248 master-slave control mode. |
| MG autonomous mode: | The MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited on decisions with MG available information only. |
| MGC-strictly controlled mode: | Synonym to the basic H.248 master-slave control mode. All control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| MGC-smartly controlled mode: | A MGC-MG tandem with support of both (MGC-strictly controlled mode and MG autonomous mode) on a service individual basis. The MGC decides the applied control mode. |

# 4	Summary

The contribution elaborates on H.248 control models, which are all known from the past, but which were never really documented at a single place.

The benefit of the MG autonomous mode for bearer connection control of connection oriented IP transport/session/tunnel connections is out of question (due to control performance aspects or offloading the MGC, till to the fact that this mode is inherent to all pure SDP-based bearer connection controls).

However, we need an explicit definition about the H.248 control mode terminology due to different interpretations (but also expectations).

Purpose of this contribution was to start such a discussion, - and if there would be initial agreement, to move relevant terms to draft H.248.TCP (see also C-0007) or H.248.1 IG.

# Appendix I: Gateway decomposition options (BICC network architecture)

A collection of possible functional-to-physical mapping options according Q.Sup31: Technical Report TRQ.2141.0. The variation of models is primarily related to a) number of physical units and b) location of bearer control protocol.

Figure I.1 – Model “one physical unit” (= Fig. C.4/ ITU-T Q.Sup31)

Figure I.2 – Model “two physical units, MGC and MGU” (= Fig. C.3/ ITU-T Q.Sup31)

Figure I.3 – Model “two physical units, CCU and BIWN” (= Fig. C.2/ ITU-T Q.Sup31)

Figure I.4 – Model “three separate physical units” (= Fig. C.1/ ITU-T Q.Sup31)

It could be concluded that, - since the publication of the technical report eleven years ago -, the landscape of decomposition models consolidated (luckily) over the past decade, primarily thanks to

- consolidation in bearer plane (“all IP”)

- consolidation of link layer (“dominated by Ethernet”) and

- experiences with bearer control and signalling protocols (“amount of CPU resources”)

It may be also noted that we may limit the scope on “all IP” H.248 gateway types (from consideration of bearer types ‘TCP’ or ‘TLS’), such as given by IMS access and peering gateways.

_________________

Defined as (T.38) “MGC transitioning method“ and „(MG) autonomous transitioning method“.

Defined as “Explicit control“ (= (CSM) MGC controlled) and “Implicit control“ (= (BIWF) MG controlled).

For discussion: UPC function realized as explicit H.248 Property (such as Q.1950), or emulated via H.248 StreamMode settings?