| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4359 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | H.323 Associative SET (ASSET) Framework description and proposed call flows |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This document outlines the description and proposed call flows for a new H.323 SET (Simple Endpoint Type) which allows the instruction of H.325 like functions into H.323 systems. AVD-4360 and AVD-4361 contain the proposed work items derived from this document.

# Introduction

The proposal to add support to H.323 as a stepping stone to H.325 functionality was first proposed at the last rapporteur meeting in Brisbane. Out of that meeting the first draft of a H.460 based protocol document was put forward at the meeting in Geneva in January. It was noted at that meeting…

[TD-51]

It was suggested that perhaps what we need is a framework document for introducing "H.323 Application Entities" like file transfer applications or whiteboard applications.  This might be a "profile" of H.323 similar to the "Simple Endpoint Type" defined in H.323 Annex F.  Perhaps most important is the removal of the requirement that an H.323 Application Entity support G.711, since most such applications, would not need audio or video support, as the user's primary H.323 terminal would provide that capability.  Use of audio or video should not be precluded, though.  If the H.323 Application Entity is a video display, for example, it would need to utilize video.  The framework document would explain how to use call-independent supplementary service messages to allow association of applications and terminals, etc.  The framework would probably be an Annex to H.323.  Contributions are solicited.

It was decided not to add this as a work item to the work programme at this meeting, though there is interest in progressing this and the framework document once it is prepared. Further, definition of a protocol and application protocol entities are also solicited.

# Discussion

The author has taken note of the experts review and developed this contribution within in the framework of a H.323 “Simple Endpoint Type” as proposed. It includes an overall framework and call flow contribution to better outline the proposal.

# Conclusion

The following contribution outlines the description and proposed call flows for a new “Associative” H.323 simple endpoint type (ASSET) and the interaction with existing H.323 devices.

# H.323 Associative SET (ASSET) Framework description and proposed call flows

# Description

An H.323 Associative SET device (ASSET) is a device within a H.323 network that is able to provide additional capabilities beyond those of any regular H.323 terminal devices. For instance, an H.323 device capable of only audio may connect with an H.323 Associative SET (ASSET) on another device to share a H.239 presentation within the context of a call initiated/received on the audio device or a File transfer application maybe invoked on a separate device to send a file within the context of an on-going H.323 call.

Proposed Standardisation methodology

It is proposed to:

- Include a new annex in H.323 to describe Associative SET (ASSET).

- Draft a new H.46x.0 recommendation to describe the H.225.0 procedures involved with ASSETs and H.323 based applications.

- H.46x.y recommendations to define ASSET applications.

# Proposed Device Definitions and Requirements

Full device definitions and requirements for ASSET are proposed as a new Annex to H.323.

Presented below is a high level overview of that document.

## Definitions

### Application:  A real-time function which resides on an ASSET. These applications are identified by an Object Identifier (OID). A listing of standard H.323.323 application OIDs are defined in H.46x.0.

Pairing:  A unique association between an endpoint and an ASSET.

### AssociationToken: A Globally Unique Identifier used to validate/authenticate ASSET pairings.

InvokeToken: A Globally Unique Identifier used to identity an Application in the context of a call to invoke/terminate.

## Requirements

An ASSET shall operate with limited H.323 functionality and shall have the following minimum requirements.

H.323 Device to ASSET

- Full H.225.0 RAS Support

- H.225.0 Call Signalling support for Setup, Connect, Facility and ReleaseComplete.

- Advertise with EndpointType of SET (bit 2).

- conferenceGoal element set to callIndependentSupplementaryService.

- No H.245.

- H.460.18

- H.46x.0 as a required feature.

ASSET to ASSET

- Full H.225.0 RAS Support

- Full H.245 Support

- No mandatory capability support

- H.460.18/19 for Nat Traversal.

- H.46x.0 as a required feature.

ASSET Applications (H.46x.y)

- Regular H.245 Capability defined to be invoked in an ASSET Environment.

A regular H.323 Endpoint may also be capable of ASSET functionality and advertise an endpointType of SET (bit 2) in conjunction with its existing endpointType and when invoked shall adhere to ASSET requirements.

ASSETs shall be assigned and reachable by using the existing H.225.0 AliasAddress and defined H.225.0 Call Signalling procedures.

# Functional Overview

There are 2 functional components of an ASSET.

- Pairing and Application Discovery between itself and other H.323 devices.

- Preparing, Invoking and terminating Applications with other ASSET devices.

## Pairing and application discovery

Pairing between a H.323 device and an ASSET shall be carried over existing H.225.0 signalling using existing H.225.0 calling mechanisms. Each pairing shall be managed/controlled by an AssociationToken. A list of ApplicationID (OID registered in H.46x.0) is retrieved from the ASSET. All messages once call has been established shall be carried between the devices via a Facility Message with reason set to featureSetUpdate.

## Preparing, invoking and terminating Applications

At the start of a call the H.323 device shall call the ASSET with a list of applications it may want to invoke during that call. Each application shall be assigned an aliasAddress and a uniquely identified InvokeToken create by the ASSET and transmitted back to the H.323 device. The aliasAddress and ApplicationID shall be transmitted in the remote party to advise the Application is available and how it can be contacted. The InvokeToken shall be used locally to invoke the application on the local ASSET.

To invoke the application, the ASSET is instructed, using the InvokeToken, to place a call to the supplied aliasAddress.

At the end of the call, the H.323 device notifies the ASSET that the call is complete and all InvokeToken is released.

ASSETS or H.323 devices may poll each other from time to time for application/call status updates.

# Detailed Procedures

Full description of ASSET discovery is proposed in H.46x.0.

Presented below is a high level overview of that document.

## H.225.0 RAS Procedures

An ASSET shall support H.460.18 and be capable of registration via H.225.0 RAS to an existing Gatekeeper using endpointType of SET (bit 2).

## ASSET Discovery procedures

To associate, an endpoint places a call to an ASSET with conferenceGoal set to callIndependentSupplementaryService and H.46x.0 advertised in the featureSet.requiredFeature field. It shall contain a request to associate (H.46x.0 parameter). Once approved, a new AssociationToken is generated and transmitted back (via H.46x.0 parameter) to the H.323 device. The device may decline the request and that is transmitted back to the caller (H.46x.0 parameter).

Once created, the AssociationToken shall be used for all communication between the paired devices.

A list of applications identified by ApplicationID (OID registered in H.46x.0) may be requested by the H.323 device. Periodically, the ASSET may be called to update status information. (H.46x.0 parameter).

## ASSET Application procedures

The section refers to establishing media flows for applications.

All calls placed between ASSETs within the context of an existing call shall have the H225.0 Setup PDU conferenceGoal set to create or join. All calls shall also contain the CallLinkage field of the H323_UU_PDU set to the CallIdentifier of the associated call the message refers to.

### Making applications available for call

Prior to call establishment the endpoint shall place a call to the ASSET with the paired AssociationToken and an ordered sequence of application ID’s contained in H.46x.0 parameter of a H.46x.0 feature advertised in the featureSet.requiredFeature field.  The ASSET shall reply with an invoke list (H.46x.0 parameter) for the supplied application IDs in the Connect message containing the application ID, application’s contactable address (H.225.0 AliasAddress) and unique InvokeToken.

### Advertising ASSET support during a call

Endpoints that support ASSETs shall advertise to another device via H.46x.0 parameter advertised in the featureSet.supportedFeature field included in the Setup Message. The feature shall also include (via H.46x.0 parameter) then ordered merged application invoke list collected from its ASSETs. The called device, if supported, shall advertise the merged available and locally supported invoke list in the connect message, this notifies both devices of what applications are available for the call.

During a call the invocation list may be updated by sending updates via an H.46x.0 parameter included in a facility messages with reason set to featureSetUpdate.

### Invoking Applications

An endpoint may invoke an application at any time by sending an invoke instruction (via H.46x.0 parameter) to the ASSET containing the invokeToken and the remote aliasAddress.

The ASSET may notify that the application has or has not been successfully invoked via an invoke notification (H.46x.0 parameter) containing the application ID and indication the invocation was successful or not.

### Starting Applications

To start an application a call is place from the ASSET to the remote AliasAddress supplied in the invoke instruction. The call shall only contain the sequence of H.245 capabilities associated with that application. Upon receiving the call the ASSET may notify the endpoint (H.46x.0 parameter) with the associated InvokeToken.

### Terminating Applications

To terminate an application, an endpoint sends a terminate instruction (H.46x.0 parameter) containing the InvokeToken to the ASSET.

The ASSET may send a terminate notification (H.46x.0 parameter) containing the InvokeToken to notify the endpoint the application has terminated.

When the original call terminates, a terminate notification (H.46x.0 parameter) is sent to the ASSET containing the CallIdentifier to notify the ASSET that the call has terminated and all running applications shall be closed and associated InvokeToken released.

__________________