| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4361 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | H.323 Associative SET (ASSET) Feature |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This document proposes a new work item to define the implementation of new H.323 “Associative” SET (Simple Endpoint Type). This document should be read in conjunction with AVD4359 and AVD4360.

# Introduction

The proposal to add support to H.323 as a stepping stone to H.325 functionality was first proposed at the last rapporteur meeting in Brisbane. Out of that meeting the first draft of a H.460 based protocol document was put forward at the meeting in Geneva in January. It was noted at that meeting…

[TD-51]

It was suggested that perhaps what we need is a framework document for introducing "H.323 Application Entities" like file transfer applications or whiteboard applications.  This might be a "profile" of H.323 similar to the "Simple Endpoint Type" defined in H.323 Annex F.  Perhaps most important is the removal of the requirement that an H.323 Application Entity support G.711, since most such applications, would not need audio or video support, as the user's primary H.323 terminal would provide that capability.  Use of audio or video should not be precluded, though.  If the H.323 Application Entity is a video display, for example, it would need to utilize video.  The framework document would explain how to use call-independent supplementary service messages to allow association of applications and terminals, etc.  The framework would probably be an Annex to H.323.  Contributions are solicited.

It was decided not to add this as a work item to the work programme at this meeting, though there is interest in progressing this and the framework document once it is prepared. Further, definition of a protocol and application protocol entities are also solicited.

# Discussion

The author has taken note of the experts review and developed this contribution within in the framework of a H.323 “Simple Endpoint Type” as proposed.

# Conclusion

The following contribution proposes the call flows for a new “Associative” H.323 simple endpoint type (ASSET).

Proposed Draft New Recommendation H.46x.0

H.323 Associative Set (ASSET) Feature

# 1	Scope

This Recommendation describes the procedures for using H.225.0 Call Signalling as a method for the association of H.323 systems for the purpose of remote querying and invoking of applications to realise functionality beyond the capability of one single device.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.323]	Recommendation H.323 (2009), Packet-based multimedia communications systems.

[ITU-T H.225.0]	Recommendation H.225.0 (2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.235]	Recommendation H.235 (2003), Security and encryption for H-Series 
(H.323 and other H.245-based) multimedia terminals.

[ITU-T H.245]	           Recommendation H.245 (2011), Control protocol for Multimedia Communications.

[ITU-T H.460.1]	Recommendation H.460.1 (2002), Guidelines for the Use of the Generic Extensible Framework.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

### 3.1.1	Application:  A real-time function which resides on an Associative Simple Endpoint Type.

## 3.2	Terms defined in this Recommendation

3.2.1 Pairing:  A unique association between an endpoint and an ASSET.

### 3.2.2 AssociationToken: A Globally Unique Identifier used to validate/authenticate ASSET pairings.

3.2.3 InvokeToken: A Globally Unique Identifier used to identity an Application in the context of a call to invoke/terminate.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ASSET 		    Associative Simple Endpoint Type as defined in H.323 Annex x.

OID		   Object Identifier

| ASN.1 | Abstract Syntax Notation No. 1 |
| --- | --- |
| GEF | Generic Extensible Framework |

# 5	Feature description

This Recommendation defines a procedure wherein H.323 devices may associate with an ASSET for the purpose of discovering and utilising functionality on that device that otherwise would not be available to the H.323 device.

# 6	Informative Description

This Recommendation outlines the mechanism to use the H.225.0 call signalling connection as a method for a H.323 device to associate with an ASSET for the purpose of discovering applications on that ASSET and invoking then during the context of a call on the H.323 device.

# 7	Capability advertisement

Endpoints and ASSETs capable of supporting this Recommendation shall advertise this capability via the generic extensibility framework (GEF) defined in [ITU-T H.323] and [ITU-T H.460.1].

An Endpoint calling an ASSET shall advertise the ASSET feature via the featureSet.requiredFeatures field in the Setup Message with the conferenceGoal set to callIndependentSupplementaryService. An ASSET shall respond via the featureSet.requiredFeatures field in the Connect Message.

An endpoint calling another endpoint shall advertise the ASSET feature via the featureSet.supportedFeatures field in the Setup Message. An endpoint that supports this recommendation shall respond via the featureSet.supportedFeatures field in the Connect Message.

An ASSET calling another ASSET shall advertise the ASSET feature via the featureSet.requiredFeatures field in the Setup Message. An endpoint that supports this recommendation shall respond via the featureSet.requiredFeatures field in the Connect Message.

Table 1 defines the ASSET feature in this Recommendation.

Table 1 – Indication of ASSET feature

| Feature name: | ASSET Feature |
| --- | --- |
| Feature description: | H.323 Associative Simple Endpoint (ASSET) Feature |
| Feature identifier type: | oid |
| Feature identifier value: | 0.0.8.46x.0 |

Table 2 – ASSETPDU parameter

| Parameter name: | ASSETMessage Feature |
| --- | --- |
| Parameter description: | ASSET Message |
| Parameter identifier type: | oid |
| Parameter identifier value: | 0.0.8.46x.0.0 |
| Parameter type: | octetString ASN.1 Encoded ASSETPDU (SEQUENCE OF ASSETMessage) (Refer Annex A) |
| Parameter cardinality: | One and only one |

# 8	Pairing an endpoint with an ASSET

Endpoint wishing to pair with an ASSET shall include the request AssociateIE in an ASSETMessage in the Setup PDU to indicate that the device is wishing to pair. The associateToken  of the ASSETMessage shall be omitted to indicate that this is a new request.

Upon receiving the request AssociateIE the device shall establish the callIndependentSupplementaryService call by responding via a Connect Message. The ASSET may then prompt to approve the request. All subsequent messages shall be ignored until the pairing has been confirmed or rejected.

Once the pairing has been approved, an AssociationToken (Globally Unique Identifier) shall be generated and transmitted back to the Endpoint in a Facility Message with reason set to featureSetUpdate. It shall include the confirm AssociateIE containing the AssociationToken. The associateToken  of the ASSETMessage may be omitted.

The AssociationToken shall be presented in the associateToken  field for all further interactions between the endpoint and the ASSET.

Where pairing is rejected, the reject AssociateIE shall be included in the genericData field of a ReleaseComplete message with reason set to genericDataReason.

Once the pairing has been approved has been approved, the endpoint may request an application listing. This is achieved by sending a Facility message containing the statusRequest  AssociateIE set to complete.

Upon receiving a statusRequest,  the ASSET shall reply with a Facility message including the statusResponse  AssociateIE. The statusResponse shall contain a SEQUENCE of Application.

Each Application sequence shall include:-

applicationId: Unique object identifier (OID) for the application. (Refer Annex B).

Each Application may also include:-

display: An ApplicationDisplay sequence contain human readable name of the application and the language.

avatar:  A URL path to an image to display for the application.

status: An ApplicationStatus choice indicating the current availability of the application.

From time to time the endpoint may request status updates from the ASSET. This is done by placing a call to the ASSET with the statusRequest  AssociateIE set to status in the Setup Message and the ASSET shall respond with the statusResponse  AssociateIE in a ReleaseComplete with reason genericDataReason containing a SEQUENCE of Application containing the applicationId and the status field. The ASSET may from time to time send an unsolicited statusResponse AssociateIE in a Setup Message to the endpoint to notify any status updates.

# 9	Invoking Application

## 9.1  Preparing Applications

Prior to sending the SETUP message or after receiving a SETUP message, the endpoint shall send to the ASSET an ASSETMessage containing the associationToken and the application field.  The application field shall contain the callIdentifer of the proposed/received call and a message field set to listRequest. The listRequest shall contain the SEQUENCE of Application containing only the applicationId that the endpoint may wish to invoke. The invokeToken element of the application field shall be omitted.

The ASSET shall respond with an ASSETMessage with the associationToken and the application field containing the message field set to listResponse.  The listResponse shall contain the SEQUENCE of Application requested.

Each Application sequence shall include:-

applicationId: Unique object identifier (OID) for the application (Refer Annex B)

aliasAddress: The unique H.225.0 AliasAddress the application can be reached on.

invokeToken: The generated Globally Unique Identifier of the application for the call.

The  application field shall also contain the callIdentifer and the invokeToken element omitted.

## 9.2	Advertising and Negotiating ASSET support during a call

The SETUP message shall contain the ASSETMessage containing the application field (other fields shall be omitted). The application field shall contain the callIdentifer of the call and the message field set to listRequest.  The invokeToken element shall be omitted. The listRequest shall contain a SEQUENCE of Application that are available for the call.

Each Application sequence shall include:-

applicationId: Unique object identifier (OID) for the application (Refer Annex B)

aliasAddress: The unique H.225.0 AliasAddress the application can be reached on.

invokeToken: The Identifier for the remote to use to invoke the application.

The remote endpoint shall respond with an ASSETMessage in the Connect Message. The ASSETMessage shall contain the application field (other fields shall be omitted). The application field shall contain the callIdentifer of the call and the message field set to listResponse.  The invokeToken element shall be omitted. The listResponse shall contain a SEQUENCE of Application that is common between the two endpoints.

Each Application sequence shall include:-

applicationId: Unique object identifier (OID) for the application (Refer Annex B)

aliasAddress: The unique H.225.0 AliasAddress the application can be reached on.

invokeToken: The Identifier for the remote to use to invoke the application.

## 9.3	Invoking an application during a call

An application shall be invoked by sending an ASSETMessage to the ASSET. The ASSETMessage shall contain the associationToken and the application field. The application field shall contain the callIdentifer and local invokeToken of the application to invoke. The message field shall be set to startRequest and Application to start.

The Application sequence shall include:-

applicationId: Unique object identifier (OID) for the application to invoke

aliasAddress: AliasAddress of the application to call.

invokeToken: The remote application InvokeToken.

## 9.4   Starting an application

To start an application, A SETUP message is sent to the aliasAddress supplied in the startRequest from the endpoint. The SETUP message shall include an ASSETMessage containing the application field (other fields are omitted). The application field shall contain the callIdentifer, the invokeToken field shall be omitted. The message field shall be set to startRequest and contain the Application to start.

The Application sequence shall include:-

applicationId: Unique object identifier (OID) for the application to invoke

invokeToken: The local application InvokeToken.

### 9.4.1  Notifying an Application has started

Upon successful/unsuccessful call establishment the calling ASSET shall signal back to the endpoint with an ASSETMessage containing the associationToken and the application field. The application field shall contain the callIdentifier and the invokeToken. The message field shall be set to startConfirm. Where the call is unsuccessful the message field shall be set to startReject and contain the H.225.0 ReleaseCompleteReason returned from the failed Application call. Where an ASSET rejects the startRequest without attempting to place the call due to the application being in use or unavailable the ReleaseCompleteReason shall be set to gatewayResources.

Upon receiving the call to start an application, the ASSET shall signal the endpoint with the matching callIdentifer via an ASSETMessage containing the associationToken and the application field. The application field shall contain the callIdentifier and the invokeToken. The message field shall be set to startConfirm. Where the call is unsuccessful the message field shall be set to startReject and contain the H.225.0 ReleaseCompleteReason returned to the remote ASSET in the application start. Where an ASSET rejects the application start due to the application being in use or unavailable the ReleaseCompleteReason shall be set to gatewayResources.

### 9.4.2  Application status updates

An endpoint may query, from time to time, the current status of a running application. An ASSETMessage is sent to the ASSET containing the associationToken and the application field. The application field shall contain the callIdentifier and the invokeToken. The message field shall be set to statusRequest.

The asset shall respond or from time to time send unsolicited status updates. An ASSETMessage is sent containing the associationToken and the application field. The application field shall contain the callIdentifier and the invokeToken. The message field shall be set to statusResponse.  The statusResponse shall be set the current status of the application.

## 9.5  Terminating an Application

To terminate an application an ASSETMessage is sent from the endpoint to the ASSET and shall contain the associationToken and the application field. The application field shall contain the callIdentifier and the invokeToken. The message field shall be set to stopRequest.

The asset shall respond or whenever the Application has terminated via an ASSETMessage. The ASSETMessage contains the associationToken and the application field. The application field shall contain the callIdentifier and the invokeToken. The message field shall be set to stopConfirm.

## 9.6  Hanging up a call

When the original call terminates, all invokeTokens and any running applications associated with that call shall also terminate and resources released.

An ASSETMessage is sent to the ASSET and shall contain the associationToken and the application field. The application field shall contain the callIdentifier. The invokeToken shall be omitted. The message field shall be set to callRelease.

# 10  Security Considerations

For further study.

Annex A

ASN.1 Code for H.46x.0

H46x.0 {itu-t(0) recommendation(0) h(8) 46x (0) } DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

AliasAddress,

GenericIdentifier,

GloballyUniqueID,

CallIdentifier,

ReleaseCompleteReason

FROM H323-MESSAGES -- See Rec. ITU-T H.225;

--

-- ApplicationDisplay

--

ApplicationDisplay ::= SEQUENCE

{

language      IA5String                 OPTIONAL,    -- RFC4646 language tag

display       BMPString(SIZE(1..128))

}

--

-- ApplicationStatus

--

ApplicationStatus ::= CHOICE

{

available      NULL,

unavailable    NULL,

inuse          NULL

}

--

-- Application

--

Application ::= SEQUENCE

{

applicationId      GenericIdentifier,

display            ApplicationDisplay   OPTIONAL,

avatar             IA5String            OPTIONAL,

status             ApplicationStatus    OPTIONAL,

aliasAddress       AliasAddress         OPTIONAL,

invokeToken        GloballyUniqueID     OPTIONAL,

...

}

--

-- Application IE

--

ApplicationIE ::= CHOICE

{

listRequest         SEQUENCE OF Application,

listResponse        SEQUENCE OF Application,

startRequest        Application,

startConfirm        NULL,

startReject         ReleaseCompleteReason,

stopRequest         NULL,

stopConfirm         NULL,

statusRequest       NULL,

statusResponse      ApplicationStatus,

callRelease         NULL,

...

}

--

-- Application PDU

--

ApplicationPDU ::= CHOICE

{

message               ApplicationIE,

callIdentifier        CallIdentifier      OPTIONAL,

invokeToken           GloballyUniqueID    OPTIONAL,

...

}

--

-- Request Type

--

RequestType ::= CHOICE

{

complete            NULL,

status              NULL,

...

}

--

-- Associate IE

--

AssociateIE ::= CHOICE

{

request              NULL,

confirm              GloballyUniqueID,

reject               NULL,

statusRequest        RequestType,

statusResponse       SEQUENCE OF Application,

...

}

--

-- ASSETMessage

--

ASSETMessage ::= SEQUENCE

{

associateToken   GloballyUniqueID               OPTIONAL,

associate        AssociateIE                    OPTIONAL,

application      ApplicationPDU                 OPTIONAL,

...

}

--

-- ASSETPDU

--

ASSETPDU ::= SEQUENCE OF ASSETMessage

END

Annex B

Table B.1 – OIDs Defined for this Recommendation

| OID | Clause Reference |
| --- | --- |
| { itu-t (0) recommendation (0) h (8) 46x application(1) } |  |
| { itu-t (0) recommendation (0) h (8) 46x application(1) apps(0) } |  |

__________________