| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4364 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.TLS |  |  |  |
| Title: | H.248.TLS "Gateway control protocol: H.248 packages for control of transport security using TLS" (New): Input draft Ed. 0.8 |  |  |  |
| Purpose: | Meeting input (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's input draft for draft new H.248.TLS "H.248 packages for control of transport security using TLS" (Ed. 0.8).

# Changes versus Ed. 0.7 (TD 20 R1 (WP 1/16) (2013-01)):

- Modify Rec title (dependent on agreement for H.248.DTLS)

- Modify “MG TLS profile” to “TLS MG profile” due to consistency with the two other TLS profile names.

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.TLS Ed. 0.1 | Initial draft (TD-23) from WP 2/16 Geneva meeting, Feb. 2012 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-23.zip |
| H.248.TLS Ed. 0.2 | Input (TD 749 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749 |
| H.248.TLS Ed. 0.3 | Output (TD 749R1/WP2) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749R1 |
| H.248.TLS Ed. 0.4 | Input (AVD-4232) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4232.zip |
| H.248.TLS Ed. 0.5 | Output (TD-23) from WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-23.zip |
| H.248.TLS Ed. 0.6 | Input (TD 20 (WP2/16)) from ITU-T SG16 Geneva meeting, Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0020 |
| H.248.TLS Ed. 0.7 | Output (TD 20 R1 (WP2/16)) from ITU-T SG16 Geneva meet., Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0020 |
| H.248.TLS Ed. 0.8 | Input (AVD-4364) to WP 1/16 Oslo meeting (2013-01) Location: |
| H.248.TLS Ed. 0.9 | Output (TD-##) from WP 1/16 Oslo meeting (2013-01) Location: |
| H.248.TLS Ed. 0.10 |  |

CONTENTS

Page

1 Scope 8 1.1 Applicability statements 9 2 References 9 3 Definitions 10 3.1 Terms defined elsewhere 10 3.2 Terms defined in this Recommendation 10 4 Abbreviations and acronyms 10 5 Conventions 11 6 Use case descriptions 11 6.1 Use cases related to TLS transport modes 11 6.2 Bearer connection network use cases with H.248 IP-IP gateways 12 6.2.1 High-level use case categories 12 6.2.2 Use case variations 13 6.3 Bearer connection network use cases with H.248 TCP-to-Non-TCP gateways 14 6.4 Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway) 14 7 Models 14 7.1 Network model from H.248 entity point of view 14 7.2 Bearer connection model 15 8 TLS session control package 16 8.1 Properties 16 8.2 Events 16 8.2.1 TLS session state change (“BNC change”) 16 8.3 Signals 18 8.3.1 Establish (BNC) 18 8.3.2 Release (BNC) 18 8.3.2 Closure Alert 19 8.4 Statistics 20 8.5 Error Codes 20 8.6 Procedures 20 8.6.1 Determining TLS Related Capabilities 20 8.6.2 Mode of Operation 20 8.6.3 TLS-Endpoint Role 21 8.6.4 TLS-Profile Handling 21 8.6.5 Completion of the initial TLS handshake 24 8.6.6 Security Context Renegotiation 24 8.6.7 TLS-Session Resumption 24 8.6.8 Statistics 25 8.6.9 TLS-Session Termination 25 8.7 Example for the TLS profile concept 26 9 TLS capability negotiation package 28 9.1 Properties 28 9.1.1 TLS Versions 28 9.1.2 Cipher Suites 29 9.1.3 Compression Methods 29 9.1.4 Support for Session Resumption 30 9.1.5 Support for Renegotiation of the Security Context 30 9.1.6 Renegotiation Period 30 9.1.7 TLS Domain Profile Identifier 31 9.1.8 Client Authentication Required 31 9.2 Events 31 9.3 Signals 31 9.4 Statistics 31 9.5 Error codes 32 9.6 Procedures 32 10 TLS session maintenance package 32 10.2 Events 32 10.2.1 TLS Session Warning Alert 32 10.2.2 Lower Layer Abort 33 10.3 Signals 34 10.3.1 Warning Alert 34 10.4 Statistics 35 10.5 Error codes 35 10.6 Procedures 35 11 TLS traffic volume metrics package 35 11.1 Properties 35 11.2 Events 35 11.3 Signals 35 11.4 Statistics 35 11.4.1 Number of Received TLS-Fragments 35 11.4.2 Number of Sent TLS-Fragments 35 11.4.3 Number of Received Application Data Octets 36 11.4.4 Number of Sent Application Data Octets 36 11.5 Error codes 36 11.6 Procedures 36 12 Package-less TLS control 36 13 Security considerations 37 Appendix I Sample use-cases of TLS bearer encryption 37 I.1 Use-case #1: Terminal-to-MG TLS session, provisioned TLS service negotiation 37 I.2 Use case #2: Terminal-to-MG TLS session, MGC-controlled TLS service negotiation 38 I.3 Use case #3: WebRTC to NGN/IMS interworking function with DTLS-to-TLS support 38 Appendix II Example call flows 39 II.1 TLS to non-TLS interworking with TCP as example bearer type 39 II.1.1 Initial session setup using SDP security descriptions in application control signalling 39 II.1.2 Call flow for communication establishment phase (in MG autonomous mode) 40 II.1.3 Call flow for communication release phase (in MG autonomous mode) 42 II.1.4 Call flow for communication establishment phase (in MGC strictly control mode) 44 II.1.5 Call flow for communication release phase (in MGC strictly control mode) 46 Appendix III Example TLS profiles 47 III.1 Typical Internet TLS profile 47 III.2 3GPP TLS domain profile 48 III.3 OMA TLS domain profiles 49 III.3.1 OMA public key TLS domain profile 50 III.3.2 OMA pre-shared key TLS domain profile 51 III.4 IETF Minimum TLS domain profile 51 Appendix IV Living List related to DTLS and DTLS-SRTP 52 IV.1 DTLS protocol 52 IV.2 DTLS related services 52 IV.3 DTLS in scope of H.248.TLS? 52 IV.4 DTLS use cases 52 IV.4.1 Use cases from TLS-to-DTLS interworking perspective 53 IV.4.2 Use cases from SRTP “key exchange” perspective 53 IV.4.3 Two interpretations for „DTLS-SRTP“? 54 IV.4.4 Use cases from WEBRTC perspective 55 IV.4.5 Use cases from Telepresence perspective 55 Bibliography 56

List of Tables

Page

Table 1 – Principal TLS-based interfaces of H.248 entities and their relevance for this Recommendation 9 Table 2 – Example of a MG TLS profile 27 Table 3 – Example of an intersection between a TLS domain profile and a MG TLS profile 27 Table III.1.a – Example of a TLS domain profile – Main part 47 Table III.1.b – Example of a TLS domain profile – Cipher Suites 48 Table III.2.a – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Main part 48 Table III.2.b – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Cipher Suites 49 Table III.3.a – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Main part 50 Table III.3.b – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites 51

List of Figures

Page

Figure 1 – TLS transport modes 12 Figure 2 – Bearer connection network use cases with H.248 IP-IP gateways 13 Figure 3 – Network model from H.248 entity point of view (“half call/bearer connection model”) 14 Figure 4 – TCP/IP protocol stack with TLS protocol 15 Figure 5 – Two-termination context with a TLS termination 15 Figure 6 - Intersection of the TLS domain profile 1 and the MG TLS profile 22 Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG 38 Figure II.1 – Configuration for the example call flow 39 Figure II.2 – Example call flow, setup of a TLS to non-TLS IWF 41 Figure II.3 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-1 42 Figure II.4 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-2 44 Figure II.5 – Example call flow for a strictly MGC controlled bearer connection setup 45 Figure II.6 – Example call flow, strictly MGC controlled release of a TLS to non-TLS IWF, release initiated by UE-2 46 Figure IV.1 – TLS/DTLS protocol historic 52 Figure IV.2 – Example TLS-to-DTLS interworking 53 Figure IV.3 – Media-path decoupled key exchange using SDP (via SIP & H.248) 53 Figure IV.4 – Media-path coupled key exchange using DTLS 54 Figure IV.5 – Interpretation „DTLS only used for SRTP key exchange“ 54 Figure IV.6 – Interpretation „Secure RTP traffic using DTLS“ 55 Figure IV.7 – DTLS use cases: „WebRTC data“ & „WebRTC SRTP key exchange“ 55

Draft new ITU-T Recommendation H.248.TLS

Gateway control protocol: 
H.248 packages for control of transport security using TLS

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

Recommendation ITU-T H.248.TLS defines new ITU-T H.248 packages for support of secured IP transport bearer traffic according the Transport Layer Security (TLS) technology. In addition, this Recommendation identifies typical network use cases with different ITU-T H.248 media gateway variants, and provides example signalling flows.

# 1	Scope

{Editor’s note (2013-01 meeting): this scope section is still not complete and should be updated as soon as the Recommendation text becomes more mature. Contributions are solicited.}

Transport Layer Security (TLS) is a cryptographic protocol that provides secure communication between two IP transport connection endpoints. This Recommendation defines in general H.248 signalling elements for support of TLS in various options.

This Recommendation provides in more detail information about

- basic exchange architecture of keying information between control plane (incl. H.248 interface) and bearer plane;

- negotiation aspects: indication and determination of cryptographic capabilities between TLS endpoints;

- minimum amount of information carried by H.248 for establishing H.248 TLS/L4 terminations (NOTE 1);

NOTE 1 – Some information could be provisioned via management and there is also TLS information exchanged via the (L4) bearer interface with the remote TLS endpoint.

- TLS procedures in detail at the various TLS sublayers, i.e. for support of the

  - TLS record protocol

  - TLS handshake protocol

  - TLS change cipher spec protocol

  - TLS alert protocol

  - TLS application data protocol

- profiling of TLS services;

- specify set of cipher suites;

- consideration of H.248 MG modes of operation and connection models;

- SDP- versus Property-based TLS endpoint control

- {list to be updated before approval/consent …}

- The scope of this Recommendation is limited to {… any limitations?  …}

## 1.1	Applicability statements

Table 1 summarizes all possible TLS-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TLS-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TLS-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TLS-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)

[IETF RFC 4975]	IETF RFC 4975 (2007), The Message Session Relay Protocol (MSRP)

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (03/2009), Gateway control protocol: Packages for interworking between MSRP and H.248

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services

[ITU-T H.248.TCP]	Draft Recommendation ITU-T H.248.TCP (201X), TCP support packages

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	TLS-client [IETF RFC 5246]: The application entity that initiates a TLS connection to a server.  This may or may not imply that the client initiated the underlying transport connection.  The primary operational difference between the server and client is that the server is generally authenticated, while the client is only optionally authenticated.

3.1.2	TLS-server [IETF RFC 5246]: The server is the application entity that responds to requests for connections from clients.  See also "client".

3.1.x	TLS protocol profile: … see e.g. ANNEX E in [3GPP 33.310] ….

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	TLS MG profile: The TLS profile supported by the MG.

3.2.2	TLS Domain: A set of entities in a network domain which use the same set of TLS related parameters, procedures and resources.

3.2.3	TLS domain profile: The TLS profile defined for a TLS domain.

3.2.4	TLS endpoint: Either a TLS-client or a TLS-server.

3.2.5	TLS endpoint profile: The TLS profile used by a TLS endpoint for the setup of a TLS session.

3.2.6	TLS-profile: A selection of options from a set of TLS related parameters and procedures.

{Editor’s note (2012-09): the definition proposals in clause 3.2 are still under discussion .}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2B | Back to Back |
| --- | --- |
| B2BIH | Back to Back IP Host (see also [b-ITU-T H.248.64]) |
| B2BTE | Back-to-Back TCP Endpoint |
| B2BUA | Back to Back User Agent (SIP) |
| BGF | Border Gateway Function |
| FTP | File Transfer Protocol |
| HTTP | Hypertext Transfer Protocol |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.1) or IP-to-non-IP gateway (see clause 6.2) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.3).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to TLS transport modes

The TLS protocol is independent of the underlying transport protocol stack. Multiple TLS transport modes are basically useful (Figure 1).

Figure 1 – TLS transport modes

This Recommendation is not dependent on any TLS transport mode, but uses TLS-over-TCP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion “IP-to-IP” (briefly “IP-IP”) indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 2):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as “non-TLS to non-TLS”);

- Use case #1.2: an end-to-end TCP connection traverses two network domains with different transport security policies (“TLS”, “no TLS”). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as “TLS to non-TLS”);

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as “TLS to TLS”);

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as “TLS to TLS*”);

Figure 2 – Bearer connection network use cases with H.248 IP-IP gateways

{Editor’s note 2 (2012-02): we need to summarize the different requirements for the H.248 interfaces and the IP bearer interface.}

### 6.2.2	Use case variations

#### 6.2.2.1	“TLS-to-TLS transparent forwarding”

FIXTHIS

#### 6.2.2.2	“TLS-to-TLS application aware and decryption-encryption based forwarding”

The MG may be enforced for application awareness in case of e.g. bearer-level ALG support (according [ITU-T H.248.78]). The MG is located within a single security domain, therefore not any change of the TLS profile. However, the MG is located at the edge of two network address domain, thus requested for NAT-T and NAT support.

Example use case:

MSRP bearer-level ALG with MSRP-over-TLS transport and so-called media anchoring in H.248 IP-IP media gateways. See [IETF RFC 6714]; the “TLS-to-TLS” interworking function is termed as “TLS B2BUA”. It may be noted that this use case does not belong to the “TLS-to-TLS*” category. The TLS decryption-encryption process is indicated in clauses 6.5 and 7.4 in [IETF RFC 6714].

## 6.3	Bearer connection network use cases with H.248 TCP-to-Non-TCP gateways

Theoretical scenarios with TCP endpoint location in H.248 MG, but forwarding of application data via a non-TCP bearer.

For further studies.

## 6.4	Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway)

{Editor’s note (2012-02): see also discussion of clause 13.3 in H.248.NATTP2P.}

For further studies.

# 7	Models

## 7.1	Network model from H.248 entity point of view

Following network model (Figure 3) may illustrate the relevant areas covered by this Recommendation. The H.248 MG is basically peering a TLS-capable IP host as remote endpoint. Both TLS endpoints spanning a network transport security domain (here TLS domain).

Figure 3 – Network model from H.248 entity point of view 
(“half call/bearer connection model”)

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of TCP mode(s) of operation in the H.248 MG; and

- configuration and procedures of the TLS/TCP/IP protocol stack for H.248-controlled bearers.

{Editor’s note (2012-02): there might be an additional network interface in case of dedicated, centralized network servers in the TLS domain with respect to key distribution. This may imply a modified Fig. 2, as well as an extend list above.}

## 7.2	Bearer connection model

All protocol elements and procedures described in this Recommendation are limited to the extent of a single ITU-T H.248 termination. In addition, no assumptions are made regarding either the lower layer protocols beneath the TLS level or the upper layer protocols being carried by the TLS/TCP transport. This allows the use of the Recommendation's procedures in various connection models and use-cases.

Figure 4 provides the TCP/IP protocol stack with the suite of TLS protocols and their sub-layer organization.

Figure 4 – TCP/IP protocol stack with TLS protocol

Figure 5 details the generic connection-model where a TLS-enabled termination is connected to a single other termination (either TLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 5 – Two-termination context with a TLS termination

# 8	TLS session control package

{Editor’s note (2013-01): package redesign according C.14 and alignment with H.248.TCP “TCP basic connection control package”.}

|  | Package Name: | TLS session control package |
| --- | --- | --- |
|  | Package ID: | tlsc (0x####) |
|  | Description: | This package provides …{Editor’s note (2013-01): package description text is not yet updated. Needs to be done if package partitioning proposal is agreed.} |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

## 8.2	Events

### 8.2.1	TLS session state change (“BNC change”)

|  | Event Name: | TLS session state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TLS) bearer network connection occurs. For example a bearer has been established or a bearer has been released. This event notifies the MGC that the TLS session setup is finished, or when the TLS session is closed again. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Enumeration |
|  | Optional: | Yes?? |
|  | Possible values: | Est, [0x01] Bearer Established Mod, [0x02] Bearer Modified Cut, [0x03] Bearer Cut through Mfail, [0x04] Bearer Modification Failure Rel, [0x05] Bearer Release NOTE 1 – For release indication see: General Package E.1.2/H.248.1 Cause Event. |
|  | Default: | ??? |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Enumeration |
|  | Optional: | Yes?? |
|  | Possible values: | Est, [0x01] Bearer Established Mod, [0x02] Bearer Modified Cut, [0x03] Bearer Cut through Mfail, [0x04] Bearer Modification Failure Rel, [0x05] Bearer Release NOTE 1 – For release indication see: General Package E.1.2/H.248.1 Cause Event. |
|  | Default: | ??? |

##### 8.2.1.2.2	Session Termination Initiator

|  | Parameter Name: | Session Termination Initiator |
| --- | --- | --- |
|  | Parameter ID: | sti (0x0001) |
|  | Description: | Indicates which side initiated the termination of the TLS session. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Session termination was initiated by the MG “remote” (0x0002) Session termination was initiated by the remote TLS-endpoint |
|  | Default: | NA |

##### 8.2.1.2.3	Session Termination Alert Description

|  | Parameter Name: | Session Termination Alert Description |
| --- | --- | --- |
|  | Parameter ID: | stad (0x0002) |
|  | Description: | Indicates the alert description for the termination of the TLS session. The values are as assigned by IANA for the TLS Alert Registry. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | NA? {Editor’s note (2012-02): seems to be contradicting with above “IANA value assignment” indication. Hm?. Contributions are solicited.} |
|  | Default: | NA |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”):

Package-less mode NOT possible. Any notification mechanism implies the explicit usage (and definition) of an H.248 event.

Conclusion: event as such should be kept …).}

## 8.3	Signals

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to send bearer establishment (i.e., this signal is used to establish the TCP connection). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |
|  | Additional Parameters: | Not applicable |

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TCP connection). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |
|  | Additional Parameters: | ?? (see gb package) |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”) concerning close/release:

Option 1:

The trigger for bearer release would be tight to the H.248 Command request SUB.req for stream or termination.

Other options?

None!?

Conclusion: a packageless semantic could be defined.}

### 8.3.2	Closure Alert

{Editor’s note: signal closure alert needs yet to be merged with above Release BNC signal.}

|  | Signal Name: | Closure Alert |
| --- | --- | --- |
|  | Signal ID: | cal (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates “fatal”, or if the ad parameter is set to “close_notify(0)” the session will be terminated. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

8.3.1.1	Additional parameters

8.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

8.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertDescription”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

## 8.4	Statistics

None.

## 8.5	Error Codes

FIXTHIS …

## 8.6	Procedures

{Editor’s note (2013-01): description of procedures needs to be updated …}

### 8.6.1	Determining TLS Related Capabilities

The MGC can determine the properties of the TLS MG profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), Supported Compression Methods (scm) and Support for Session Resumption (ssr). Normally the audit will be performed on the Root termination.

The Supported TLS Versions property provides the TLS-versions supported by the MG without any specific sequence.

The Supported Cipher Suite property may include the cipher suite TLS_NULL_WITH_NULL_NULL (“0000”), which is used as the initial cipher suite during the first handshake for the TLS-session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

TLS session resumption may be used to avoid the need for a negotiation of the new security context. The Support for Session Resumption property provides the MGC with an indication whether the MG supports this feature.

{Editor’s note: TLS-resumption use cases to be identified.}

### 8.6.2	Mode of Operation

The instantiation of a TLS endpoint is indicated to the MG by the “m=” line in the Local and Remote Descriptor. The protocol being used is defined in [IETF RFC 4572] for the media agnostic case or in the media related specification (e.g. [IETF RFC 4975] for MSRP).

With the creation of a TLS endpoint in the MG the underlying TCP endpoint is created as well implicitly. The SDP attribute “a=setup” must be set to “actpass” in the Local and Remote Descriptor to indicate the TCP-proxy mode, refer to [ITU-T H.248.84] and to [ITU-T H.248.TCP].

### 8.6.3	TLS-Endpoint Role

The TLS-Endpoint Role (er) property controls whether the MG shall act as a TLS client or a TLS server. Usually per TLS endpoint this role is coupled to role of the TCP layer, however use cases might exist where the role in the TLS layer and the TCP layer may differ.

At creation time of a TLS endpoint in the MG the TCP connection setup role and the TLS-Endpoint Role (er) might not be defined yet, as normally the SEP is created in the MG before the client/server role negotiation is completed on the call control signalling plane (SDP offer/answer). In such a case the TCP connection setup role “notset” will be used (refer to [ITU-T H.248.TCP]) and the TLS-Endpoint Role (er) will be set to “tcp-alike”.

Once the TCP connection setup role is set to a value different from “notset” (via TCP connection control package property tcpcc/csr) in the MG, the TCP connection setup will be triggered in the MG, and at completion the TLS session will be established autonomously in the MG according to the value of the property TLS-Endpoint Role (er).

### 8.6.4	TLS-Profile Handling

A MG may be requested to support more than one TLS domain. For each such TLS domain different behaviour of the TLS implementation may be required. For each TLS domain those requirements are defined by a “TLS domain profile”. Generally the TLS domain profile must be provisioned on the MG. The property TLS Domain Profile Identifier (dpid) is used to refer to a TLS domain profile. This property must be aligned between the MG and the MGC, as the MGC will use it to indicate to the MG which TLS domain profile will be applicable for the creation of a TLS endpoint in the MG.

The TLS related parameters and procedures supported by the TLS implementation in the MG are referred to as “TLS MG profile”.

On creation of a TLS endpoint in the MG the intersection of the applicable TLS domain profile and the TLS MG profile will be build. This intersection is called the TLS endpoint profile. It defines the TLS related parameters and procedures the MG will use to establish the TLS session on the endpoint.

Figure 6 illustrates the relation between the profiles.

Figure 6 - Intersection of the TLS domain profile 1 and the TLS MG profile

{Editor’s note (2012-09 meeting): two clarifications are requested:

- notion of “overall TLS profle”( what exactly? required?)

- relation of “TLS MG profile” to “TLS domain profile(s)”: subset, equal, superset, - all options possible?

Contributions are solicited.}

8.6.4.1	TLS-profile handling: TLS version

The list of TLS versions used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. This list is supposed to be provisioned per dpid on the MG.

In case the MGC provides the property TLS Versions (tlsv), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG is acting as a TLS-client, the highest TLS versions as defined by the TLS-endpoint profile are included into the initial TLS-handshake message (ClientHello). In case the MG acts as a TLS-server, the MG shall select the highest TLS-version that is present in the list of TLS-versions offered from the TLS-client and in the TLS Versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS-handshake is regarded as failed.

{Editor’s note (2012-09 meeting): Error cases need to be worked out, e.g. in case not all TLS-versions received from the MGC are supported by the MG}

8.6.4.2	TLS-profile handling: Cipher suites

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account. It is supposed that the client and the server list of cipher suites are provisioned per dpid on the MG.

In case the MGC provides the property Chipher Suites (cs), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint. In case the MG is acting as a TLS-client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS-server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS-version.

8.6.4.3	TLS-profile handling: Compression methods

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account. It is supposed that the client and the server list of compression methods are provisioned per dpid on the MG.

In case the MGC provides the property Compression Methods (cm), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle than the negotiation for the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS-version.

8.6.4.4	TLS-profile handling: Client authentication

The MG’s TLS server implementation must support the client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). It is supposed that this setting is provisioned per dpid on the MG.

In case the MGC provides the property Client Authentication Required (car), the MG will use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

8.6.4.5	TLS-profile handling: Self-signed client certificates

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute a=fingerprint. If present in the Remote Descriptor, the client certificate will be self-signed and will be verified against the fingerprint.

8.6.4.6	TLS-profile handling: signed client certificates

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). This list of root certificates is supposed to be configured per dpid on the MG.

8.6.4.7	TLS-profile handling: Server authentication

In case the TLS endpoint in the MG is configured as a TLS server, the MG must provide the server certificate to the client as defined by the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). Different TLS domains may require different server certificates, because the list of accepted root certificates may differ between TLS domains.

8.6.4.8	TLS-profile handling: Session resumption

FIX THIS

8.6.4.9	TLS-profile handling: Renegotiation support

The TLS implementation in the MG must provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). It is supposed that this value is provisioned per dpid on the MG.

In case the MGC provides the property Support of Renegotiation of the Security Context (srsc), the MG will use this value instead.

### 8.6.5	Completion of the initial TLS handshake

The TLS handshake will be performed autonomously by the MG with the properties as defined by the TLS endpoint profile.

The initial TLS handshake is completed when the following both conditions are true:

- The TLS-Finish message has been sent, using the negotiated set of security parameters

- The TLS-Finish message has been received and has been successfully verified

At this point in time TLS application data may be sent and received using the negotiated security parameters. The event TLS session established (estab) can be used to notify the MGC accordingly.

The event estab does not provide any statement about the state of the other SEP of the Context.

### 8.6.6	Security Context Renegotiation

The behaviour of the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, and the Renegotiation Period (rp) property. In case the TLS-endpoint profile disables renegotiation, the MG will never initiate a renegotiation. If receiving a renegotiation request from the remote TLS-endpoint, the MG will respond with a no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS-version.

If renegotiation shall be supported, the MG will accept and process a renegotiation request from the remote TLS-endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to 0, no renegotiation will be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS-handshake completed successfully. On expiration of the timer the MG will trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS-version. In case the MG receives a trigger for the renegotiation from the remote TLS-endpoint while this timer is running, the MG will cancel and restart this timer.

{Editor’s note (2012-09 meeting): Current assumption is that no explicit MGC control of the renegotiation is required, e.g. trigger renegotiation through a signal.}

### 8.6.7	TLS-Session Resumption

FIXTHIS …

### 8.6.8	Statistics

FIXTHIS …

### 8.6.9	TLS-Session Termination

The trigger for a TLS session termination might come from different sources:

- The TLS session is terminated by the remote TLS endpoint by sending a close_notify alert or a fatal alert;

- The underlying TCP connection is terminated

- A non-recoverable error condition has been detected in the TLS function, e.g. the authentication of a received TLS segment has failed;

- The MGC indicated the closure of the TLS session by the signal Alert;

- The release of the bearer connection of the other SEP triggers the release autonomously in the MG;

- In case the MG acts in an application aware mode, the application part in the MG may indicated the release of the TLS session;

8.6.9.1		TLS alert

In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour of the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a close_notify was received the TLS endpoint will generate a close_notify on its own and the TLS session is closed. The MG will autonomously initiate the release of the TCP connection as well. If requested the MGC will be notified by the event TLS-Session Termination Alert (sta). In this case the parameter Session Termination Initiator (sti) will be set to “remote” and the parameter Session Termination Alert Description (stad) will contain the value from the AlertDescription as received in the TLS alert message.

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.2		TCP connection closed

In case the underlying TCP connection is closed, e.g. due to a received TCP-FIN or TCP-RST the MG will autonomously complete the closure of the TCP session if needed.

If requested the MGC will be notified by the event Lower Layer Abort (lla).

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.3		Non recoverable error condition detected

In case an error condition is detected locally in the TLS layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert will be sent to the remote TLS endpoint with the AlertLevel set to “fatal” and the AlertDescription is provided accordingly.

If requested the MGC will be notified by the event TLS-Session Termination Alert (sta). The parameters Session Termination Alert Description (stad) will contain the same values as sent to the remote TLS endpoint, and the parameter Session Termination Initiator (sti) will be set to “local”.

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.4		Signal alert received from MGC

In case the signal alert is received and the parameter Alert Level (al) is set to “fatal” or if the parameter Alert Description (ad) is set to “close_notify”, the TLS session closure is initiated by the MG by sending a TLS alert with the same parameter as received from the MGC.

The alert signal does not generate the notification TLS-Session Termination Alert (sta) in return.

Independent of the value of the property MG Autonomous Bearer Release Indicator (abri) the other SEP will not be affected by the alert signal. If desired the MGC needs to release the bearer connection on that SEP separately as well.

8.6.9.5		MG autonomous release initiated on related SEP

The TLS enabled SEP might receive the indication that the bearer connection of the related SEP is released. If such an indication is detected the termination of the TLS session is initiated by sending a TLS alert (close_notify) to the remote TLS endpoint. The closure of the underlying TCP connection is initiated by the MG autonomously.

The event TLS-Session Termination Alert (sta) is not reported to the MGC in this case, as the MGC has the option to be notified by the related SEP about the release of the bearer connection.

8.6.9.6		Release indication received from TLS application

In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session will be closed autonomously by sending a TLS alert (close notify) and the underlying TCP connection is closed autonomously by the MG as well. The property MG Autonomous Bearer Release Indicator (abri) will have no effect, as it will depend on the application if the bearer connection of the related SEP needs to be released as well.

{Editor’s Note: In this case the behaviour of the application may be controlled by a similar property than tlsn/abri. This would need to be clarified in the application related specification.}

{Editor’s Note:

Observation: There are a lot of commonalities to H.248.TCP:

- Bearer connection state “established” and “closed” are reported by event notifications

- MGC may trigger the release of the bearer connection by a signal (“close”)

- The property MG Autonomous Bearer Release Indicator (abri) allows a “MGC-strictly controlled” behaviour as well as a”MG-autonomous” behaviour. The property  is present in both packages, indeed this indicator is bearer type independent

- Release procedure differs only in minor protocol dependent details (TCP: half closure; TLS: “duplex” closure)

Thus defining a bearer protocol independent package to control the bearer connection setup and release might be an option. Such a packet could be the base for other bearer connection protocols as well like SCTP, DTLS, etc. Such a package could simplify interworking functions between different bearer connection protocols.}

## 8.7	Example for the TLS profile concept

Appendix III provides examples for the TLS profile concept.

Table 2 depicts an example for a TLS MG profile. It also shows which properties can be audited on the Root termination by the MGC.

Table 2 – Example of a TLS MG profile

| Scope | Example | Corresponding H.248 signalling element for property audit by MGC |
| --- | --- | --- |
| TLS versions | TLS1.1 TLS1.2 | tlsn/tlsv |
| Cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA | tlsn/cs |
| Compression methods | NULL, DEFLATE, LZS | tlsn/cm |
| Client authentication required | yes | tlsn/car |
| Resumption support | no | tlsn/ssr |
| Extensions |  |  |

The following Table 3 shows an example how the intersection of the TLS domain profile and the TLS MG profile is built to determine the values for the TLS endpoint profile. The properties of the TLS endpoint profile will be used by the MG for the TLS handshake. This table does not indicate  which properties can be provided by the MGC.

Table 3 – Example of an intersection between a TLS domain profile and a TLS MG profile

| Property | TLS Domain profile | TLS MG profile | TLS endpoint property value |
| --- | --- | --- | --- |
| TLS version | Mandatory: TLS1.1 Optional: TLS1.2 | TLS1.1, TLS1.2 | TLS1.1, TLS1.2 |
| Cipher suite | Mandatory: TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA Optional: TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_DH_RSA_WITH_AES_256_CBC_SHA256 | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_RC4_128_MD5, TLS_RSA_WITH_DES_CBC_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA |
| Compression method | NULL, LZS | NULL | NULL |
| Client authentication required | Yes | Yes | Yes |
| Root certificates | List: A, B, C | List: X, A, B | List: A, B Provisioned per TLS domain profile |
| MG certificate |  | List of certificates, one certificate per TLS domain profile | Provisioned per TLS domain profile |
| Support for renegotiation | No | Yes | No |
| Support for resumption | Yes | No | No |

{Editor’s note (2012-09 meeting): A clear view on the MG certificate is still missing. It is used during the TLS handshake, thus it is a part of the “set of TLS related parameter and procedures” (current definition of a TLS profile), but it is neither defined for the TLS domain profile nor (currently) for the TLS MG profile. The MG certificate is network element specific and TLS-domain specific.}

# 9	TLS capability negotiation package

|  | Package Name: | TLS capability negotiation package |
| --- | --- | --- |
|  | Package ID: | tlscn (0x####) |
|  | Description: | ## When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. This package provides control tools for the MGC for influencing bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. {Editor’s note (2013-01): package description needs to be inserted….} |
|  | Version: | 1 |
|  | Extends: | None. |

## 9.1	Properties

### 9.1.1	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0001) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS-RFC (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the “0x”, and if needed, a leading “0” is added for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl, Root {Editor’s note (2012-02): the scope needs to be clarified, i.e., either Root or LocalControl level per H.248 profile, or both. Contributions are solicited.} |
|  | Characteristics: | Read/Write |

### 9.1.2	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0002) |
|  | Description: | This property indicates the cipher suites and their precedence for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sublist of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA. The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA will be coded into the first two characters, and the second value will be coded into the last two characters of the string. Thus each value in its hexadecimal representation will be converted into the double-hexdigit string. The “0x” will be omitted and if needed, a “0” is used for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). {Refer to http://www.iana.org/assignments/tls-parameters/tls-parameters.xml, other contributions are welcome. } |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write, Root |

### 9.1.3	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0003) |
|  | Description: | This property declares the list of compression methods and their preference for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write, Root |

### 9.1.4	Support for Session Resumption

|  | Property Name: | Support for Session Resumption |
| --- | --- | --- |
|  | Property ID: | ssr (0x0004) |
|  | Description: | This property declares the support for the TLS session resumption in the MG |
|  | Type: | Boolean |
|  | Possible values: | False Session Resumption shall not be supported by the MG True Session Resumption shall be supported by the MG |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write, Root |

### 9.1.5	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0005) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported by the MG |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported by the MG True Renegotiation shall be supported by the MG |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 9.1.6	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0006) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. {Editor’s note: Do we need other criteria for a renegotiation trigger, like nbr fragments sent/rec., nbr octets sent/rec., etc?} |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 9.1.7	TLS Domain Profile Identifier

|  | Property Name: | TLS Domain Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0008) |
|  | Description: | This property identifies the TLS domain profile that shall be used by the MG to setup the TLS session. |
|  | Type: | String |
|  | Possible values: | Any string as agreed between MGC and MG. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 9.1.8	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0009) |
|  | Description: | This property declares if the MG – if acting as a TLS-server – shall authenticate the client |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS-server, the client is not requested to be authenticated. True In case the MG acts as a TLS-server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.4	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

FIXTHIS.

# 10	TLS session maintenance package

|  | Package Name: | TLS session maintenance package |
| --- | --- | --- |
|  | Package ID: | tlsm (0x####) |
|  | Description: | {Editor’s note (2013-01): package description needs to be inserted ….} |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

None.

## 10.2	Events

### 10.2.1	TLS Session Warning Alert

|  | Event Name: | TLS-Session Warning Alert |
| --- | --- | --- |
|  | Event ID: | swa (0x0003) |
|  | Description: | This event indicates an abnormal condition for the TLS layer that does not lead to the termination of the session. The abnormal condition may be detected on the local or on the remote TLS endpoint. The event shall be set at stream level. |

#### 10.2.1.1	EventsDescriptor parameters

None

#### 10.2.1.2	ObservedEventsDescriptor parameters

##### 10.2.1.2.1	Session Warning Initiator

|  | Parameter Name: | Session Warning Initiator |
| --- | --- | --- |
|  | Parameter ID: | swi (0x0001) |
|  | Description: | Indicates on which TLS endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Abnormal condition detected by MG “remote” (0x0002) Abnormal condition detected by the remote TLS endpoint |
|  | Default: | NA |

##### 10.2.1.2.2	Session Warning Alert Description

|  | Parameter Name: | Session Warning Alert Description |
| --- | --- | --- |
|  | Parameter ID: | swad (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition. The values are as assigned by IANA for the TLS Alert Registry. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | NA |
|  | Default: | NA {Editor’s note (2012-02): seems to be contradicting with above “IANA value assignment” indication. Hm?. Contributions are solicited.} |

### 10.2.2	Lower Layer Abort

{Editor’s note (2013-01): this Event might be redundant and could be probably deleted again. The underlying use case:

- there’s an active TLS session;

- the underlying IP transport connection (e.g., TCP or SCTP transport connection) is released from the remote side, but without any TLS closure alert indication;

- thus, there’s still an established TLS session endpoint, but a released L4 bearer connection from a local MG perspective

- the TLS session is interrupted, not any TLS alert could be sent to the remote side

This use case could be addressed by generic Event “BNCChange” (or specific events “TCP connection state change” or “SCTP connection state change”).

Further:

- the communication path might be also interrupted below layer 4, then error codes 530, 531, … interactions to be considered.

- there might be also layer Lx specific keep alive mechanisms enabled, then again possible interactions to be considered.

Contributions are solicited.}

|  | Event Name: | Lower layer abort |
| --- | --- | --- |
|  | Event ID: | lla (0x0004) |
|  | Description: | This event indicates that the layer below TLS has become unavailable, thus indicates the termination of the TLS session. The event shall be set at stream level. |

#### 10.2.2.1	EventsDescriptor parameters

None

#### 10.2.2.2	ObservedEventsDescriptor parameters

##### None

## 10.3	Signals

### 10.3.1	Warning Alert

|  | Signal Name: | Warning Alert |
| --- | --- | --- |
|  | Signal ID: | wal (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates “fatal”, or if the ad parameter is set to “close_notify(0)” the session will be terminated. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

10.3.1.1	Additional parameters

10.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

10.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertDescription”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

## 10.4	Statistics

None.

## 10.5	Error codes

None.

## 10.6	Procedures

FIXTHIS.

# 11	TLS traffic volume metrics package

|  | Package Name: | TLS traffic volume metrics package |
| --- | --- | --- |
|  | Package ID: | tlstv (0x####) |
|  | Description: | {Editor’s note (2013-01): package description needs to be inserted ….} |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

None.

## 11.3	Signals

None.

## 11.4	Statistics

### 11.4.1	Number of Received TLS-Fragments

|  | Statistic Name: | Number of Received TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recfrag (0x0001) |
|  | Description: | This statistic logs the number of TLS-fragments received by the MG. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 11.4.2	Number of Sent TLS-Fragments

|  | Statistic Name: | Number of Sent TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentfrag (0x0002) |
|  | Description: | This statistic logs the number of TLS-fragments sent by the MG. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 11.4.3	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0003) |
|  | Description: | This statistic logs the number of application data octets that are delivered to the layer above TLS. Received application data messages from the remote TLS endpoint are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 11.4.4	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0004) |
|  | Description: | This statistic logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

## 11.5	Error codes

None.

## 11.6	Procedures

FIXTHIS.

# 12	Package-less TLS control

{Editor’s note (2013-01 meeting): dedicated chapter on SDP based control required, addressing the following

Purpose of this clause is to describe the SDP-based semantics for TLS control, which needs the consideration of

- SDP usage in H.248.69, H.248.84

- how the package-based elements/procedures would be realized via SDP?

- which functionality would be out of scope of SDP control?

- assumption of mixed Property- & SDP-based control

First comments (from C-20):

To requirement of “Originating/Terminating Bearer control protocol procedures”:

[other package elements]

Contributions are solicited.}

# 13	Security considerations

…

Appendix I

Sample use-cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use-case #1:   Terminal-to-TLS MG session, provisioned TLS service negotiation

This use case illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain; the MGC could be outside (as shown) or inside this TLS-domain, but the MGC is in any case aware of the applied TLS domain profile for the secured bearer traffic.

- IP transport bearer type: ‘TCP’ in this example (but might be also e.g. SCTP).

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by transport options SIP/SCTP/IPsec, SIP/DTLS/UDP/IP, SIP/TLS/TCP, etc.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS

- Bearer path protocol layering: Application/TLS/TCP/IP (i.e., not any tunnelled transport).

- UE: TCPclient and TLS client, MG: TCP server and TLS server (NOTE: this assumption could be satisfied with SDP “a=setup:” attribute)

- H.248 MG TCP-mode: TCP-proxy.
{Editor’s note: Any other TCP-modes and potential transitions between different TCP-modes are to be studied. See also work on H.248.TCP }

- Mutual TLS-authentication via certificates, MG’s certificate is signed, UE’s certificate is self-signed

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG

Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG

## I.2	Use case #2: Terminal-to-TLS MG session, MGC-controlled TLS service negotiation

This use case is an extension of the first use case (clause I.1) by allowing the MGC to influence the TLS service negotiation at bearer level. Use case #1 is based on preconditions which are partially subject of network provisioning. Use case #2 is motivated by the fact that there are a number of reasons which demand for a MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

{Editor’s note: this clause will be updated when the design and definition of H.248 packages for TLS will be more mature …}

## I.3	Use case #3: WebRTC to NGN/IMS interworking function with DTLS-to-TLS support

To be inserted.

{Editor’s note (2013-01): this use case is also considered from TCP perspective, thus covered by Appendix I.1/H.248.TCP. I.e., there might be a reference added later on in this clause …}

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation)

{Editor’s note (2013-01): entire Appendix needs to be updated as soon as H.248.TLS (and H.248.TCP) packages are stable…}

## II.1	TLS to non-TLS interworking with TCP as example bearer type

### II.1.1	Initial session setup using SDP security descriptions in application control signalling

This use case is presented in Appendix I.1. Figure II.1 outlines the network configuration as used in the message flow.

Figure II.1 – Configuration for the example call flow

The scenario shows a simplified SIP session establishment. The TCP-client/server role is defined by the setup attribute (based on [IETF RFC 4145]), and the fingerprint of the UE’s certificate is included in the SDP (based on [IETF RFC 4572]).

The H.248 MG is triggered by the MGC to create two terminations, whereby the bearer for the first SEP/termination is secured by the use of TLS. In the RD of this termination the MG also receives the fingerprint of the UE’s certificate.

### II.1.2	Call flow for communication establishment phase (in MG autonomous mode)

The call flow (Figure II.2) shows the setup of the TCP-connection as well as the TLS handshake on the first termination. The self-signed certificate received from the UE is verified by the use of the fingerprint. After the TLS handshake is finished, application data may flow between the UE and the remote call session endpoint. In the message flow presented here the MG will forward these data in the TCP-proxy mode.

Figure II.2 – Example call flow, setup of a TLS to non-TLS IWF

### II.1.3	Call flow for communication release phase (in MG autonomous mode)

#### II.1.3.1	Communication release initiated by remote TLS endpoint

An example for bearer connection release is depicted by Figure II.3. UE-1 initiates the release by sending a TLS-alert (close_notify).

Figure II.3 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-1

{Editor’s note (2013-01): above message flow indicates a scenario with a tightly coupled TLS/TCP bearer. Questions;

- how would be such a bearer type indicated? E.g., would be SDP proto = TCP/TLS imply such a semantic, but if yes, then this parameter value couldn’t be used for decoupled bearer indications …? Still open …}

#### II.1.3.2	Communication release initiated by remote non-TLS endpoint

Figure II.4 shows the traffic call flow in case the release of the bearer connection is initiated on the non-TLS connection. In the shown example UE-2 initiates the closure of the TCP connection by sending a TCP-FIN. The example assumes that the connection has been setup as shown in Figure II.2.

Figure II.4 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-2

### II.1.4	Call flow for communication establishment phase (in MGC strictly control mode)

This scenario is based on the same network configuration as shown in Figure II.1. It shows a simplified session establishment using SDP offer/answer. The H.248 MG is triggered by the MGC to create two terminations, whereby termination T1 is secured by TLS. The setup of the TCP connection on termination T2 will start after the setup of the TLS session on T1 is completed.

The call flow (Figure II.5) shows the setup of the two terminations strictly controlled by the MGC.

Figure II.5 – Example call flow for a strictly MGC controlled bearer connection setup

### II.1.5	Call flow for communication release phase (in MGC strictly control mode)

The following call flow (Figure II.6) shows the release of the bearer connection strictly controlled by the MGC. The release is initiated by UE-2.

Figure II.6 – Example call flow, strictly MGC controlled release of a TLS to non-TLS IWF, release initiated by UE-2

Appendix III

Example TLS profiles

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some exemplary TLS profiles.

## III.1	Typical Internet TLS profile

Table III.1.a – Example of a TLS domain profile – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 1.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 1.4 | Extensions: | None |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 TLS 1.2 | - | - | - |
| 2.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | None |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | - |  |  |  |
| 3.2 | Client authentication required | Yes |  |  |  |
| 3.3 | Certificate types (client): | signed, self-signed |  |  |  |
| 3.4 | Renegotiation: | Yes |  |  |  |
| 3.5 | Resumption: | No |  |  |  |
| 3.6 | Extensions: | - |  |  |  |
| 3.7 | Capability negotiation: | Yes |  |  |  |

Table III.1.b – Example of a TLS domain profile – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | - | - |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | - |
|  | SHOULD NOT | - | - |

## III.2	3GPP TLS domain profile

Network domain security is defined in [ETSI TS 133.310] with the normative TLS domain profile in Annex O. Table III.2 summarizes this profile with status of 3GPP Release 11 (2012-12).

Table III.2.a – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | SSL 3.0 | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 1.3 | Compression methods: | Compression Method.null |  | methods of [IETF RFC 3749] |  |
| 1.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | see client | see client | see client | - |
| 2.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | see client |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | Client authentication required | ? |  |  |  |
| 3.3 | Certificate types (client): | ? |  |  |  |
| 3.4 | Renegotiation: | NOTE 2 |  |  |  |
| 3.5 | Resumption: | NOTE 2 |  |  |  |
| 3.6 | Extensions: |  |  |  |  |
| 3.7 | Capability negotiation: | YES (NOTE 1) |  |  |  |
| NOTE 1 – Target: negotiation up to highest TLS protocol version. NOTE 2 – Not explicitly specified, thus TLS V1.1 [IETF RFC 4346] applies: YES |  |  |  |  |  |

Table III.2.b – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA (NOTE 1) TLS_RSA_WITH_3DES_EDE_CBC_SHA (NOTE 2) TLS_RSA_WITH_NULL_SHA | see client |
|  | SHALL NOT | a) Cipher suites with NULL integrity protection or HASH b) “TLS_DH_anon_WITH_” (NOTE 3) | see client |
| O | SHOULD | TLS_RSA_WITH_AES_128_CBC_SHA256 TLS_RSA_WITH_NULL_SHA256 | see client |
|  | SHOULD NOT | Cipher suites with RC4 | see client |
| NOTE 1 – The only mandatory cipher suite from TLS V1.2 [IETF RFC 5246]. NOTE 2 – The only mandatory cipher suite from TLS V1.1 [IETF RFC 4346]. NOTE 3 – Because the “the key exchange method shall not be anonymous”. |  |  |  |

{Editor’s note (2013-01): Preferred cipher suites (ordered list): negotiation aspect?}

## III.3	OMA TLS domain profiles

The Open Mobile Alliance (OMA) defines two TLS domain profiles [OMA-TS-TLS-V1_0-20080902-A], a “public key TLS domain profile” and a “pre-shared key TLS domain profile”.

FIXTHIS

### III.3.1	OMA public key TLS domain profile

Table III.3.a – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 1.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 1.3 | Compression methods: | methods of [IETF RFC 3749] |  |  |  |
| 1.4 | Extensions: |  |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 2.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 2.3 | Compression methods: | methods of [IETF RFC 3749] |  |  | - |
| 2.4 | Extensions: |  |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | (Server ¦ Client) authentication required | YES (Server authentication both) |  | YES (Client authentication by server) |  |
| 3.3 | Certificate types (client): |  |  |  |  |
| 3.4 | Renegotiation: |  |  |  |  |
| 3.5 | Resumption: | YES (NOTE z1) |  |  |  |
| 3.6 | Extensions: | [IETF RFC 4366] |  | TLS tunneling |  |
| 3.7 | Capability negotiation: |  |  |  |  |
| NOTE 1 – #. NOTE z1 – The longer session life (e.g., 12 hours) SHOULD be used. |  |  |  |  |  |

Table III.3.b – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_3DES_EDE_CBC_SHA TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | None specified. | see client |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | None specified. |
|  | SHOULD NOT | All others | see client |
| NOTE 1 – # |  |  |  |

### III.3.2	OMA pre-shared key TLS domain profile

see [OMA-TS-TLS-V1_0-20080902-A].

## III.4	IETF Minimum TLS domain profile

The IETF noted the value of TLS protocol profiling. The IETF itself does not (yet) define any specific TLS profile itself, rather provides guidelines and decision criteria for TLS profiling, see [b-IETF tls-minimal].

Appendix IV
Living List related to DTLS and DTLS-SRTP

(This appendix does not form an integral part of this Recommendation)

{Editor’s note (2013-01 meeting): This Appendix is related to “DTLS” and “DTLS related topics”

## IV.1	DTLS protocol

DTLS is derived from TLS, could be thus considered as a single security protocol family. DTLS versioning numbers are (somehow) linked to TLS protocol versions.

Figure IV.1 – TLS/DTLS protocol historic

## IV.2	DTLS related services

There are two slightly different applications:

- DTLS as “transport security” means for UDP/IP bearer traffic

- DTLS as “key exchange” means for RTP/UDP/IP bearer traffic using media security according SRTP

## IV.3	DTLS in scope of H.248.TLS?

There are basically following options:

- DTLS covered by H.248.TLS?

- DTLS in a self-contained, new H.248.DTLS?

- DTLS added to H.248.77 due to link with DTLS-SRTP?

- DTLS covered by H.248.WEBRTC due usage by WebRTC service.

Option #2 (or #1) seems to be preferable.

## IV.4	DTLS use cases

Initial list of possible use cases:

### IV.4.1	Use cases from TLS-to-DTLS interworking perspective

Figure IV.2 provides an interworking example (here: DTLS based secure WebRTC data service).

Figure IV.2 – Example TLS-to-DTLS interworking

### IV.4.2	Use cases from SRTP “key exchange” perspective

SRTP-based media encryption, - there are many key exchange options for SRTP. For example the two outlined in below subclauses.

#### IV.4.2.1	Media-path decoupled key exchange using SDP (via SIP & H.248)

Figure IV.3 recalls again the protocol stack for SDES-based key exchange for SRTP, as in scope of H.248.77 or as part of the 3GPP H.248 Iq profile (29.334):

Figure IV.3 – Media-path decoupled key exchange using SDP (via SIP & H.248)

#### IV.4.2.2	Media-path coupled key exchange using DTLS

Figure IV.4 illustrates the alternative of a DTLS-based key exchange for SRTP:

Figure IV.4 – Media-path coupled key exchange using DTLS

### IV.4.3	Two interpretations for „DTLS-SRTP“?

RFC 5764:

This document describes DTLS-SRTP, a SRTP extension for DTLS that

combines the performance and encryption flexibility benefits of SRTP

with the flexibility and convenience of DTLS-integrated key and

association management.  DTLS-SRTP can be viewed in two equivalent

ways: as a new key management method for SRTP, and a new RTP-specific

data format for DTLS.

I.e., there are two possible interpretations from protocol stack point of view, see Figures IV.5 and IV.6.

Figure IV.5 – Interpretation „DTLS only used for SRTP key exchange“

Figure IV.6 – Interpretation „Secure RTP traffic using DTLS“

It has to be noted that these interpretations might be incorrect. To be confirmed.

### IV.4.4	Use cases from WEBRTC perspective

DTLS use cases from WebRTC perspective:

Figure IV.7 – DTLS use cases: „WebRTC data“ & „WebRTC SRTP key exchange“

In Figure IV.7:

1) WebRTC data traffic

2) WebRTC DTLS-based key exchange for SRTP

### IV.4.5	Use cases from Telepresence perspective

Are there any? TBD

Bibliography

[b-IETF rtcweb-data]	IETF draft-ietf-rtcweb-data-channel (2012), RTCWeb Datagram Connection.
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-data-channel/

[b-IETF tls-minimal]	IETF draft-tschofenig-lwig-tls-minimal (2012), A Minimal (Datagram) Transport Layer Security Implementation.
URL: http://tools.ietf.org/html/draft-tschofenig-lwig-tls-minimal-01

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

____________________

Defined as: “A network element that terminates security associations (SAs) from endpoints and establishes separate SAs between itself and each endpoint.“ ([IETF RFC 6714])