| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4370 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.39 (Rev): New SDP wildcard proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

A new wildcard “~” is proposed.

Discussion

Ref.: this contribution relates to the list announcement 2013-04-15:

[Megaco] 2013-06 Meeting: New wildcard proposal "~"

http://www.ietf.org/mail-archive/web/megaco/current/msg08648.html

The motivation for a new wildcard is primarily related to

- the purpose of MG resource preservations and

- signalling savings of redundant information at the H.248 interface (which may again lead to increased signalling robustness).

We've identified so far at least two application areas (below the respective SDP elements with proposed new wildcard):

a) NAT-T

e.g., a=ssrc:~ cname:~

b) TLS and DTLS basec Transport security:

e.g., a=fingerprint:~ ~

The specification of wildcard „~“ is contained in the change proposal section for H.248.39, see below.

The wildcard “~”

- is (in our understanding) limited on H.248-embedded SDP only;

- not a new general H.248 wildcard according clause 6.3/H.248.1;

- thus, similar as wildcard “-“, as introduced by ETSI TISPAN via Ia profile;

- solves a well-known issue: instead of repeating redundant information, the MGC may simple provide a short indication;

- therefore: existing H.248 signalling solution could be denoted as “long notation” syntax and the new wildcard as “short notation” alternative.

Usage of this wildcard may be of interest for all SDP elements used at H.248 interfaces.

An initial example is provided by AVD-4371.

Conclusions

H.248 wildcarding options are normally subject of the core standard, i.e., H.248.1. However, we are suggesting to use H.248.39 due to similarities of new wildcard “~” with “-“. We propose to

- add the lacking specification of the old wildcard “-“ (new clause 5.2.2.2);

- add thes specification of additional wildcard" “~” under new clause 5.2.2; and

- update affected sections: clauses 1, 2.2 and 6..

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 48 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Summary

This Recommendation describes the principles used to identify a single SDP sub-field and how to apply wildcarding to that sub-field. The sub-field may be fully specified, wildcarded (CHOOSE "$" or ALL "*") or not significant "-". Being able to identify a single sub-field is of use when requesting the MG to choose a parameter, specifying a range or for the purposes of auditing. Further specification of how SDP can be wildcarded will assist in interoperation between a Media Gateway Controller and a Media Gateway.

This revision incorporates guidelines for the wildcarding of source-specific media attributes.

# 1	Scope

This Recommendation describes the principles to be used to identify a single SDP sub-field and how to apply wildcarding to that sub-field. The sub-field may be fully specified, wildcarded (CHOOSE "$" or ALL "*") or not significant "-" (see clause 5.2.2.2). Being able to identify a single sub-field is of use when requesting the MG to choose a parameter, specifying a range or for the purposes of auditing.

Furthermore a new wildcard “~” is introduced (see clause 5.2.2.3), which could be considered as a short notation syntax (versus fully specified SDP information) and allows performance optimization as well as the reduction of syntax errors (due to avoidance of the repetition of redundant information).

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

## 2.2	Informative references

…

[ITU-T H.248.49]	Recommendation ITU-T H.248.49 (08/2007), Gateway control protocol: Session description protocol RFC and capabilities packages.

[ETSI ES 283 018]	ETSI ES 283 018 (2007), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification.
Note: so called “H.248 Ia profile version 1”.

[ETSI TR 183 068]	ETSI TR 183 068 (2009), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

...

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 5	H.248 wildcarding guiding principles

## 5.1	General wildcarding principles by H.248

See clause 6.3/[ITU-T H.248.1]: The overriding guidance provided by [ITU-T H.248.1] on the use of CHOOSE in SDP is: "the use of CHOOSE is allowed in place of a single parameter value".

Subsequent principles are:

1)	In any SDP scheme each mandatory element SHALL therefore be represented by a single CHOOSE "$".

2)	Any part that contain text tokens (e.g. those in " ") that are mandatory SHALL also be included in the wildcard command.

3)	Optional parts of SDP scheme should not be included unless the wildcard or value applies to them.

4)	When an item specifies a wildcard for a single sub-field, this applies to the nested ABNF in that parameter.

## 5.2	Specific wildcarding principles for H.248 embedded SDP

## 5.2.1	Background: IETF SDP

The core SDP [IETF RFC 2327], as well as successor [IETF RFC 4566], does not specify any wildcards. SDP lines are therefore always fully specified. (TBC)

It may be reminded that [ITU-T H.248.49] provides, inter alia, complementary information concerning relation and usage of H.248 and the various IETF RFCs on SDP.

### 5.2.2	H.248 introduced SDP wildcards

### 5.2.2.1	Purpose

Wildcards are used (by the MGC) for resource management or/and the indication of particular modes of operation at MG level. The scope of SDP wildcards in H.248 messages is therefore limited on SDP media descriptions (see clause 7.1.8.1.1.2/[ITU-T H.248.1]), within the LD and RD.

#### 5.2.2.2	Wildcard “-“

Wildcard “-“, with the semantic of “not significant” (or “agnostic”, “unaware”), was introduced (in 2005) by the so-called “H.248 Ia profile version 1” (see clause 5.15/[ETSI ES 283 018]), motivated by the fact for the control of dedicated “IP-to-IP interworking modes” in border gateways (see clause G.2.2/[ETSI TR 183 068]).

#### 5.2.2.3	Wildcard “~“

Purpose of this wildcard is the preservation of MG-level resources or the configuration data or resources. The wildcard is typically used in subsequent H.248 command request, i.e., this wildcard is not used in the first H.248 command request, when initially creating a stream endpoint or termination. However, there are also use case with the first H.248 command request, e.g., when correspondent resources already prepared via management actions.

Semantic of wildcard “~”:

maintain unchanged already signaled (or previously wildcarded) attribute values.

Purpose/motivation:

Not using this wildcard implies the complete replication of all SDP information (i.e., fully specified SDP lines) by the MGC, in order not to release previously requested resources at the MG. The MG must process entirely such SDP data and compare it with past received SDP. That’s a burdensome and unnecessary task, at MGC and MG side, which might be abbreviated by this wildcard.

# 6	Identification of single parameters

The subclauses below define which parts of SDP are classified as "single sub-fields (values)" for the purposes of H.248.1 wildcarding. Each of these sub-fields may be fully specified, designated as CHOOSE "$", designated as ALL "*" or designated as not significant "-". The sub-fields should not be partially wildcarded.

Each of the examples indicates what type of wildcarding is not valid. It also highlights some valid types of wildcarding and what should be returned.

NOTE 1 – For the purposes of the examples below, "?" is used to denote the potential wildcards:

•	"$" 		for Choose

•	"*" 		for All,

•	"–" 		"Not Significant" (NOTE 2), and

•	"~" 		"Maintain unchanged" (or "Preserve") (see clause 5.2.2.3)

NOTE 2 – Not to be confused with Annex B.2/H.248.1 encoding for NULL Context (see also clause 5.2.2.2).

ABNF grammar in subsequent clauses is copied from Appendix A (SDP Grammar) in [IETF RFC 2327] (which is replaced by clause 9/[IETF RFC 4566]) and other IETF RFCs concerning SDP extensions.

[End Proposed Correction]

____________________