| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4381a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Proposal for new Appendix on DPI rule emulation |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to add a new Appendix on DPI policy rule emulation.

Discussion and conclusions

This contribution is based on the discussion of following contributions at previous meetings:

- Meeting 2012-02 – AVD-4168 “H.248.DPI: Alignment with other policy control protocols” and

- Meeting 2012-09 – AVD-4245 “H.248.DPI: DPI Policy Rule Emulation Package”.

Purpose of these contributions was to show, how the capabilities of H.248.DPI could be applied on a policy control interface with DPI support, which is not H.248 based. Thus, a correspondent H.248 profile would provide a “DPI policy rule emulation service”, in case of H.248 usage for such policy control interfaces.

The purpose of the new proposed Appendix to H.248.DPI is to provide some guideline for H.248 profile specifications concerning such a DPI policy rule emulation service.

Thus, we are proposing to get rid of the idea in adding normative text to the main body of H.248.DPI, such as a new DPI policy rule emulation package (as discussed by AVD-4245). Such an H.248 package would be e.g. policy control technology specific.

The alternative approach could be the description of some high-level guidelines, how existing H.248.x capabilities plus the new proposed H.248 DPI packages (see AVD-4375) could be used to emulate the aimed DPI policy control behaviour.

We are suggesting to add an correspondent Appendix to H.248.DPI.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix V
Emulation of DPI policy rule control interfaces

(This appendix does not form an integral part of this Recommendation)

## V.1	Purpose

Appendix IV provides an overview of known policy specification languages (PSL). The behaviour of some of such PSLs could be principally emulated by H.248 based policy control interfaces. Such an emulation of DPI policy rule control interfaces would be based on H.248 capabilities as defined by this Recommendation and other H.248.x-series of Recommendation, dependent on the aimed set of DPI policy rules.

The purpose of this Appendix is to provide principal indications and high-level information how such a DPI policy rule emulation could work. The concrete specification of such H.248 interface behaviour is out of scope of this Recommendatio, rather considered to be subject of H.248 profile definitions.

Two examples are discussed.

## V.2	Guidelines for H.248 profile specifications at the example of native SNORT interfaces

### V.2.1	Motivation

Benefit from the huge base of existing SNORT based DPI policy rules.

### V.2.2	Scope

[b-SNORT] defines the syntax for SNORT DPI policy rules, following a common structure of rule header and rule options, which again comprises the rule conditions and rule actions.

### V.2.3	Possible emulation by H.248

Table V.1 provides an example approach for emulating the behaviour by H.248:

Table V.1 – Emulation approach of SNORT policy rules by H.248

| SNORT | Generic information element | Possible H.248 solution |
| --- | --- | --- |
| SNORT rule | entire DPI policy rule object | Either reference based or container based emulation, i.e., using property “Pointer to rule conditions” (irb/ptr), see clause 6.1.1, or property “Container for rule” (irb/cfr), see clause 6.1.2). |

## V.3	Guidelines for H.248 profile specifications at the example of 3GPP Diameter interfaces

### V.3.1	Motivation

There are e.g. following reasons:

- There are multiple protocol options (H.248, COPS and Diameter) for support of the ITU-T Rw policy control interface [ITU-T Y.2111]. Prerequisite (for protocol selection) is the functional parity between the different protocol alternatives. I.e., using H.248 implies the same service as supported by Diameter.

- The policy decision entity in the ITU-T RACF architecture needs to map policy control information from the northbound Rs to the southbound Rw interface (see Figure 5/[ITU-T Y.2111]). Diameter may be used for Rs, whereas an H.248 based Rw interface is very common at policy enforcement level. Hence, H.248 emulates in some way Diameter in such a mapping scenario.

### V.3.2	Scope

[b-ETSI TS 129 212] defines a Diameter object with support of DPI policy rule signalling, called Application Detection and Control (ADC) rule. It contains primarily a set of possible rule actions, without any support of explicit rule conditions, which are referred by a pointer mechanism (see clause 4.3b/[b-ETSI TS 129 212]).

### V.3.3	Possible emulation by H.248

Table V.2 provides an example approach for emulating the behaviour by H.248:

Table V.2 – Emulation approach of Diameter ADC policy rule by H.248

| ADC-Rule-Definition AVP | Generic information element | Possible H.248 solution |
| --- | --- | --- |
| ADC-Rule-Name | Name of DPI policy rule | No support required (NOTE 1). |
| TDF-Application-Identifier | Pointer to actual rule conditions | Property “Pointer to rule conditions” (irb/ptr), see clause 6.1.1. |
| Flow-Status | Policy rule action related to gate control (NOTE 2) | Either implicit in the policy rule specification itself, or/and (complemented) by H.248 gate control means (e.g., [ITU-T H.248.43]). |
| QoS-Information | Policy rule action related to traffic policing (NOTE 3) | Either implicit in the policy rule specification itself, or/and (complemented) by H.248 traffic policing tools (e.g., [ITU-T H.248.53]). |
| Monitoring-Key | Identifier for policy rule actions, related to usage parameter control across an aggregated traffic structure | Such an aggregated traffic structure could be emulated by e.g. a multiple-stream-to-termination mapping in H.248. |
| Redirect-Information | Policy rule action related to traffic redirection | Either implicit in the policy rule specification itself, or/and by H.248 support for explicit IP destination /source (transport) address setting capabilities (e.g., [ITU-T H.248.43]). |
| NOTE 1 – Only name of the specific “application detection service”. |  |  |

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# Bibliography

…

[b-ETSI TS 129 212]	ETSI TS 129 212 (2013), Universal Mobile Telecommunications System (UMTS); LTE; Policy and Charging Control (PCC); Reference points (3GPP TS 29.212 version 10.10.0 Release 10)

[End Proposed Correction]

____________________