| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4382 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.78 (Addendum 1 proposal): New MG located Bearer Level ALG package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes an additional package for H.248.78 in order to serve a third use case with regards to H.248 controlled B-ALG support.

1	Introduction

Existing H.248.78 describes two types of bearer-level ALG entities:

6.1	MGC strictly controlled MG embedded bearer-level ALG

6.2	MG autonomous embedded bearer-level ALG

The first option is specified in very detail, the second option is only pointed out as alternative.

2	Discussion

The first option, using package mcbalg, defines an event-based logic for the detection and forwarding of packets as “input” for the ALG service. The actually ALG function is located at MGC level. The pros of this variant are related to the fact, that the MGC has all relevant information inherently available for the ALG service, because typically resulting from application control protocol signalling (such as SIP-embbeded SDP Offer/Answer).

However, the downside of this approach is MGC involvement in bearer plane packet processing, which is normally not acceptable in case of high packet rates.

Thus, applications with high ALG traffic loads should be better processed by the MG itself. Such a mode is subject of clause 6.2/H.248.78.

However, H.248.78 is lacking infromation how this option could be used in detail, particularily the use case where the MGC would be involved as the source of translated address information for the B-ALG function.

A new package is proposed which is characterized by:

- B-ALG function located at MG level (B-ALGMG);

- MGC is able to switch on/off the B-ALGMG (at the level of individual streams or the entire MG);

- MGC is able to indicate individual L4+ protocols, which are subject of the B-ALGMG (at the level of individual streams or the entire MG); and

- Replaced address information in L4+ protocols may be based on three different sources of information.

3	Proposal

Amendment to H.248.78:.

Change proposal #1:

[Begin Proposed Correction]

# X	MG located Bearer Level ALG

Package name:	MG located Bearer Level ALG

PackageID:	mgbalg (0x0###)

Description:		The actual ALG function with respect to the replacement of address information at application layer is executed by the MGC when using package mcbalg (clause 7). This package moves that function down to the MG itself.
The MGC is only involved in enabling that function and (if required) providing address information to be inserted.	.

Version:	1

Extends:	None

## X.1	Properties

### X.1.1	Protocol type bearer level ALG

Property name:		Protocol type bearer level ALG

Property ID:		ptbalg (0x0001)

Description:		This property enables/disables the MG located Bearer Level ALG based on the protocol type.
{Contributor’s note: property design derived from 9.1.1/H.248.43.}

Type:				Boolean

Possible values:		ON to enable MG located bearer level ALG

OFF to disable MG located bearer level ALG

Default:			OFF

Defined in:			LocalControl on an individual stream or 
TerminationState on the root termination.

Characteristics:		Read/write

### X.1.2	Upper layer protocol filter

Property name:		Upper layer protocol filter

Property ID:		ulpf (0x0002)

Description:		This property specifies the stream's upper layer protocols type that the filtering is applied to. The term "upper layer" denotes the protocol layer above the layer 4 transport protocol, i.e., an "upper layer" protocol is a "L4+" protocol, e.g., HTTP, MSRP,  or RTSP.

{Contributor’s note: property design derived from 9.1.3/H.248.43.}

Type:				Sub-list of Integer

Possible values:		A single value or a sub-list. A value representing an application protocol, using the well known or registered port number assigned to that protocol by IANA (http://www.iana.org/assignments/port-numbers). 
For example:

80	World Wide Web HTTP

554	Real Time Streaming Protocol (RTSP)

2855	MSRP

Default:	0, indicating that the protocol is determined via the local and remote descriptors.

Defined in:			LocalControl on an individual stream or 
TerminationState on the root termination.

Characteristics:		Read/write

## X.2	Events

None.

## X.3	Signals

None.

## X.4	Statistics

None.

## X.5	Error codes

None.

## X.6	Procedures

### X.6.1	Enabling the MG located bearer level ALG service

When a MG located bearer level ALG service is required for incoming packet traffic, the MGC sets the property protocol type bearer level ALG (mgbalg/ptbalg) to "ON".

### X.6.2	Indication of affected application protocol(s)

The bearer level ALG function is normally application specific, i.e., enabled for selected application(s). The MGC should use property upper layer protocol filter (mgbalg/ulpf) to indicate the application(s).

### X.6.3	Indication of replaced address information

The MG needs to know which address information has to be used for the ALG service. There are basically three options:

Table 1 – Principle options for the indication of replaced address information

| Variant | Source of translated address information | Indicated by MGC via: |
| --- | --- | --- |
| 1 | Contained in H.248 Stream Descriptor. E.g., as part of the SDP media description, other than “c=” and/or “m=”line address information. Such as related to (general or application specific) SDP attributes. | Explicitly by MGC. Example: The SDP “a=path:” attribute (clause 8.2/[IETF RFC 4975]), used by application protocol ‘MSRP’. |
| 2 | Provisioned in MG. | Not indicated by MGC. The MG configured for that variant via management plane actions. |
| 3 | IP L3 (and L4) source (transport) address information of incoming IP packet. NOTE – I.e., numerical IP address information. | Default option, i.e., when neither variant 2 nor 3 is used. |

### X.6.4	B-ALG: L4+ protocol specific translation details

The translation of address information at L4+ level may be not defined in a generic manner because dependent on the particular L4+ protocol. The protocol dependent application details are therefore out of scope of this Recommendation (as in case of the mcbalg package, see clause 7.6.3). Such information could be e.g. part of an H.248 profile specification.

### X.6.5	Examples

{To be inserted}

[End Proposed Correction]

____________________