| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4394 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Clause 10 – Update of TCP extended connection control package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates according the agreements of the 2013-01 meeting.

1	Introduction

Contributions C-4 and C-13 from last meeting (2013-01) resulted in an agreement in drafting three individual H.248 packages for TCP bearer control.

Copy of COM 16 – C 13/Figure 1 – H.248.TCP – Package redesign proposal – Overview

This contribution continues the update process as initiated by C-11 of the TCP extended connection control package.

2	Background

The TCP extended connection control package defines two properties with support of

- TCP-intrinsic connection termination protocol, which is comprised of two independent oneway closure procedures. Independent in the sense that one TCP half-closure does not necessarily trigger the closure of the reverse TCP data path.
The MGC may enable such TCP connection closure behaviour by property Oneway Release Indicator.

- End-to-end view of connection-oriented (IP transport) connections by coupling of two connected (paired) SEPs within a context, i.e., interlinkage behaviour for a particular Stream Endpoint Pair (SEPP).

3	Discussion of protocol semantics

## 3.1	Semantical scope limitations in general

Semantical scope limitations:

- only the two phases of TCP bearer connection establishment and release (but not any bearer modifications during active communication phase);

- only incoming direction (i.e., any outgoing TCP bearer connection control request from a particular SEP does not affect the partner SEP within a SEPP);

- only the first stimuli of each TCP bearer connection control procedure to be considered (i.e., first incoming TCP SYN of three-way handshake; first incoming TCP FIN of half-closure);

Further, we got SEP-related properties, i.e., there are four value combinations (per Property) from SEPP perspective (in order to address network asymmetries)

## 3.2	TCP bearer connection establishment: impact of sepplink property

We’d like to start with discussion of the sepplink property, which is related to establishment and release phases. Figure 2 illustrates the major use case combinations (four combinations: sepplink usage at the SEPP (T1(S1) and T2(S1)), and boolean values):

Figure 2 – TCP bearer connection establishment – Impact of sepplink property

It may be noted that the event notification in scenario (E.1) is optional. Whether the event is used or not depends on the MGC level call control logic (“which reflects a network service scenario”). Anyway, the H.248 NOTIFY.req command is just submitted after a successful state transitioning, i.e., state transitioning from TCP CLOSED to ESTABLISHED after 3-way-handshake.

It may be also noted that use case (E.2) and (E.4) are redundant, because of the scope limitations as outlined in clause 3.1 (here: only first TCP-SYN forwarded).

## 3.3	TCP bearer connection release: impact of sepplink property

Figure 3 illustrates release scenarios versus sepplink setting. The ori property value is supposed to be FALSE, which relates to a TCP bearer connection release behaviour according the base package, i.e., a complete release of both TCP traffic directions.

Figure 3 – TCP bearer connection release – Impact of sepplink property

Use case (R.1) recalls again the base package behaviour (i.e., usage of tcpbcc package without tcpecc package) because both tcpecc package properties are set to FALSE:

An end-to-end TCP bearer connection (between X and Y) is effectively divided in two separate TCP bearer connection segments, from perspective of the H.248 gateway!

The MGC is explicitly triggering TCP bearer connection release (also establishment) in outgoing direction via signal tcpbcc/RelBNC (or tcpbcc/EstBNC respectively).

The signal is sent (by the MGC) either due to a received event notification from the other SEP (R.1.a) or due to call control signaling/logic (R.1.b).

Use case (R.3) demonstrates again the purpose of the sepplink property: the reception of a TCP-FIN leads to an immediate outoging TCP-FIN at the partner SEP.

## 3.4	TCP bearer connection release: impact of ori property

This property is irrelevant during establishment, only affects connection release: Figure 4 illustrates the two release scenario options due to ori settings:

Figure 4 – TCP bearer connection release – Impact of ori property

Use case (O.1) reflects again the base package behaviour of a bidirectional TCP connection release.

Use case (O.2): when ori is equal to TRUE, then only a TCP half-closure will processed, but the 2nd TCP half-closure is not triggered.

## 3.5	TCP bearer connection release: impact of both properties

Figure 5 summarizes the impact of both properties (note: only sepplink = TRUE depicted; value FALSE does not provide any new information).

Figure 5 – TCP bearer connection release – Impact of both properties

Use case (C.1):  we got again the base package behaviour of a bidirectional TCP connection release, but the incoming release request from X is propagated on the other side to Y.

Use case (C.2):  ori enforces now TCP half-closure in the direction to X, and sepplink the start of TCP release towards Y.

4	Proposal

The proposed semantical update of the two properties from the tcpecc package will lead to straightforward and unambiguous rules for the MG concerning the handling of TCP connection establishment and release. The MGC will get a flexible and comprehensive tool, which satisfies all required use cases beyond the base package capabilities.

5	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

References & abbreviations needs to be updated according agreed changes

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 9	TCP extended connection control package

|  | Package Name: | TCP extended connection control package |
| --- | --- | --- |
|  | Package ID: | tcpecc (0x####) |
|  | Description: | This extension package augments the base package behaviour by additional support of - TCP bearer connection control behaviour across a Stream Endpoint Pair (SEPP), rather instead of SEP-only scope; and - unidirectional TCP bearer connection release. |
|  | Version: | 1 |
|  | Extends: | tcpbcc |

## 9.1	Properties

### 9.1.1	Stream Endpoint Pair Interlinkage

|  | Property Name: | Stream Endpoint Pair Interlinkage |
| --- | --- | --- |
|  | Property ID: | sepplink (0x0005) |
|  | Description: | The semantical scope of this property relates to a Stream Endpoint Pair (SEPP; see definition in (clause 3.2.11) [ITU-T H.248.1]). This property defines if a detected condition leading to the establishment or closure of the TCP connection will trigger an MG-autonomous (see clause 3.2.2) establishment or release of the bearer connection of the partner SEP as well. Only TCP protocol control information (e.g., typically TCP header flag type of information or sequence/acknowledgement numbers) is part of such conditions. Normal TCP data traffic is not affected by this property. Semantical scope limitations: - only the two phases of TCP bearer connection establishment and release; - only incoming direction; - only the first stimuli of each TCP bearer connection control procedure (i.e., first incoming TCP SYN of three-way handshake; first incoming TCP FIN of half-closure); - SEP-related properties, i.e., there are four value combinations (per Property) from SEPP perspective (in order to address network asymmetries) |
|  | Type: | sublist of Boolean (for separated support of establishment and release phase) |
|  | Possible values: | Table X defines the protocol semantic per value and dependent on connection control phase. See model of Figure 5 concerning the used SEP names. Table X – Semantic of property sepplink Value; MG semantic for TCP bearer connection. Establishment; Release. True; a) Incoming: An incoming TCP bearer connection establishment request at SEP T1(S1) shall lead to an autonomous outgoing TCP bearer connection establishment request at the other related SEP T2(S1). b) Outgoing: An MGC-initiated outgoing TCP bearer connection establishment at SEP T1(S1), shall not impact the TCP connection state of SEP T2(S1).; a) Incoming: An incoming TCP bearer connection release request at SEP T1(S1) shall lead to an autonomous outgoing TCP bearer connection release request at the other related SEP T2(S1). b) Outgoing: Any action (e.g. by MGC) in closing the outgoing TCP bearer connection at SEP T1(S1), shall not impact the TCP connection state of SEP T2(S1).. False; The state of TCP bearer connection endpoint SEP T1(S1) shall not impact the state of TCP bearer connection endpoint SEP T2(S1) (and vice versa).. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tcpbcc), see clause 8). |
|  | Defined in: | LocalControl {Editor’s note (2012-09): Its unclear how this property relates to the other Terminations/Streams in the Context. i.e. if a release is triggered as a result of this property are other terminations affected if they don't have this property set? Are events triggerred on these other terminations. Might be also a Context-level property? For further study. Comments (2013-06): the semantic is a) limited to a SEPP instance only; and b) would trigger event notification(s), if SEPP interlinkage would lead to valid event detection conditions at the paired SEP. Such events are primarily those in scope of the base package. } |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

“False” semantic”, because the two SEPs within a SEPP are normally decoupled in H.248. Usually an explicit H.248 signalling request (such as a SUB.req for a SEP or termination) would trigger outgoing bearer control procedures at the MG bearer interface.

Option 2:

“True” semantic”, but justification?

Conclusion: a package-less semantic could be defined, any interaction with H.248.84 needs not to be considered because “H.248.84 NAT-T” is only for TCP connection establishment.
}

### 9.1.2	Oneway Release Indicator

|  | Property Name: | Oneway Release Indicator |
| --- | --- | --- |
|  | Property ID: | ori (0x0006) |
|  | Description: | This property defines the behaviour in the MG in case a TCP-FIN is received for an established TCP connection. It instructs the MG whether or not to send a TCP-FIN. |
|  | Type: | Boolean |
|  | Possible values: | Table X – Semantic of property ori Value; MG semantic for TCP bearer connection. False; Indicate the duplex closure of the TCP bearer connection by sending a TCP-FIN autonomously.. True; Keep the TCP bearer connection in the half-closed state, i.e. the MG will not generate the TCP-FIN autonomously. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tcpbcc), see clause 9). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

“True” semantic”, with the justification that the H.248 media stream concept is fundamentally bidirectional. Such a semantic would lead to a single solution for …

Option 2:

“False” semantic” in order to support this protocol specific characteristic of TCP.

Conclusion: a package-less semantic could be defined, but …. The semantic may be delegated to a profile specification (e.g., when all communication services across TCP would be of type bidirectional).
}

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.3	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

### 9.6.1	TCP bearer connection establishment

Only relevant for incoming TCP bearer connection establishment requests from MG external bearer interface. Outgoing procedures are not affected.

#### 9.6.1.1	Stream endpoint pair interlinkage disabled

If the property sepplink is set to False, then the incoming TCP bearer connection establishment procedure shall be identical to base package tcpbcc procedures.

NOTE – Appendix III.3.1 provides signalling examples.

#### 9.6.1.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TCP bearer connection establishment request shall be propagated to the partner SEP, triggering there an outgoing TCP-SYN.

NOTE – Appendix III.3.2 provides signalling examples.

### 9.6.2	TCP bearer connection release

Only relevant for incoming TCP bearer connection release requests from MG external bearer interface. Outgoing procedures are not affected.

#### 9.6.2.1	Stream endpoint pair interlinkage disabled and Oneway release indicator disabled

If the property sepplink is set to False, then the incoming TCP bearer connection release procedure shall be not propagated to the partner SEP.

If the property ori is set to False, then the incoming TCP bearer connection release procedure shall lead to a bidirectional TCP bearer connection release, hence identical to base package tcpbcc procedures.

#### 9.6.2.2	Stream endpoint pair interlinkage enabled and Oneway release indicator disabled

If the property sepplink is set to True, then incoming TCP bearer connection release request shall be propagated to the partner SEP, triggering there an outgoing TCP-FIN.

If the property ori is set to False, then see clause 9.6.2.1.

NOTE – Appendix III.5.3 provides signalling examples.

#### 9.6.2.3	Stream endpoint pair interlinkage disabled and Oneway release indicator enabled

If the property sepplink is set to False, then see clause 9.6.2.1.

If the property ori is set to True, then the incoming TCP bearer connection release procedure shall lead to a one TCP bearer connection release (so called TCP half closure).

#### 9.6.2.4	Stream endpoint pair interlinkage enabled and Oneway release indicator enabled

If the property sepplink is set to True, then see clause 9.6.2.2.

If the property ori is set to True, then see clause 9.6.2.3.

NOTE – Appendix III.5.3 provides signalling examples.

...

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix III

Illustration of protocol semantics 
of the TCP extended connection control package

(This appendix does not form an integral part of this Recommendation.)

## III.1	Overview

The tcpecc package (clause 9) defines an augmented TCP bearer connection control as the base package (tcpbcc, clause 8). The effect of each individual package property is illustrated in clauses III.3 (property tcpecc/sepplink) and III.4 (property tcpecc/ori). Clause III.5 describes examples of combined usage of both properties.

The illustrated use cases are some examples. An exhaustive consideration of all possible combinations is out of scope.

## III.2	Conventions

An H.248 context with a single SEPP, given by SEPs T1(S1) and T2(S1), is considered. The two tcpecc package properties are related to individual SEPs (as opposed to Context-level properties); however, the scope is on SEP T1(S1) property settings only.

Further, both properties provide only an extended behaviour (in contrast to the base package) for incoming TCP bearer connection control requests (i.e. the terminating side). The originating side is therefore out of scope of this Appendix.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) are not discussed, but could be for instance related to state changes of the local TCP bearer connection endpoint(s) (as defined by the base package).

## III.3	Usage of SEPP Interlinkage property

Property tcpecc/sepplink is defined in clause 10.1.1. The property affects TCP bearer connection establishment and release procedures.

### III.3.1	Property „sepplink = FALSE“

See Figure III.1. Semantic FALSE implies decoupled SEPs, i.e., reflect the semantic of the base package. There are two separate TCP bearer connection segments, - the two segments related to TCP connections “X  MG (SEPT1(S1))” and “Y  MG (SEPT2(S1))” -, from perspective of the MG.

Figure III.1 – Property „sepplink = FALSE“

Here: an incoming TCP bearer connection establishment request (via TCP-SYN) or release request (via TCP-FIN) at SEP T1(S1) does not affect SEP T2(S1).

Figure III.2 illustrates correspondent signalling scenarios at the example of TCP bearer establishment:

Figure III.2 – Signalling example for TCP bearer connection establishment 
– Property „sepplink = FALSE“

It may be emphasized that the sepplink setting at SEP T2(S1) does not have any impact in these scenarios.

If an incoming TCP bearer connection establishment request from remote X should also lead to establishment procedures towards remote TCP endpoint Y, then the MGC needs to be involved (see E.1 example).

### III.3.2	Property „sepplink = TRUE“

See Figure III.3. Semantic TRUE leads to a forwarding of the TCP bearer connection control request within a SEPP.

Figure III.3 – Property „sepplink = TRUE“

The TCP open and closure procedures, when initiated by remote TCP endpoints (X or Y), could now have an end-to-end scope. E.g., the three-way handshake for TCP bearer connection establishment or a TCP half-closure for oneway TCP bearer connection release would be leaded by remote TCP endpoints (X, Y) and forwarded via the SEPP, under the condition that both sepplink properties (of SEPT1(S1) and SEPT2(S1)) would be set to TRUE.

Figure III.4 illustrates correspondent signalling scenarios at the example of TCP bearer establishment:

Figure III.4 – Signalling example for TCP bearer connection establishment 
– Property „sepplink = TRUE“

It may be emphasized that the sepplink setting at SEP T2(S1) does not have again any impact in these scenarios (because only considering incoming procedures at SEP T1(S1)).

## III.4	Usage of Oneway Release Indicator property

Property tcpecc/ori is defined in clause 9.1.2. The property affects only TCP bearer connection release procedures.

### III.4.1	Property „ori = FALSE“

See Figure III.5. Semantic FALSE implies base package behaviour, i.e., a complete TCP bearer connection release.

Figure III.5 – Property „ori = FALSE“

Here: an incoming TCP bearer connection release request (via TCP-FIN) at SEPT1(S1) triggers an immediate TCP half-closure of the reverse direction towards X, but does not affect SEPT2(S1).

### III.4.2	Property „ori = TRUE“

Semantic TRUE results in a oneway TCP bearer connection release, see Figure III.6..

Figure III.6 – Property „ori = TRUE“

## III.5	Combined consideration of both properties

Only TCP bearer connection release (due to ori property) needs to be considered.

### III.5.1	Property settings: “ori = FALSE“ and “sepplink = TRUE“

See Figure III.7. The ori property codepoint leads to a complete TCP bearer connection release between MG and remote TCP endpoint X, and the sepplink property codepoint results in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure III.7 – Property settings: “ori = FALSE“ and “sepplink = TRUE“

### III.5.2	Property settings: “ori = TRUE“ and “sepplink = TRUE“

See Figure III.8. The ori property codepoint leads to an oneway TCP bearer connection release only between MG and remote TCP endpoint X, and the sepplink property codepoint results again in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure III.8 – Property settings: “ori = TRUE“ and “sepplink = TRUE“

### III.5.3	Signalling examples

Figure III.9 summarizes the impact of both properties (Note: only sepplink = TRUE depicted; value FALSE does not provide any new information).

Figure III.9 – Signalling example for TCP bearer connection release and example value combinations for both properties

Use case (C.1): the incoming TCP bearer connection release request from remote TCP endpoint X is propagated on the other side to remote TCP endpoint Y, and the base package behaviour of a bidirectional TCP connection release towards X.

Use case (C.2): Property ori enforces now a TCP half-closure in the direction to X, and sepplink the start of TCP bearer connection release towards Y.

[End Proposed Correction]

____________________