| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4395 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Clause 11 – Update of TCP retransmission metrics package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates according the agreements of the 2013-01 meeting.

1	Introduction

Contributions C-4 and C-13 from last meeting (2013-01) resulted in an agreement in drafting three individual H.248 packages for TCP bearer control.

Copy of COM 16 – C 13/Figure 1 – H.248.TCP – Package redesign proposal – Overview

This contribution continues the update process as initiated by C-11 of the TCP retransmission metrics package.

2	Discussion and conclusions

Some observations:

- TCP performance metrics concerning retransmitted TCP traffic may only generated at sending side, because the TCP receiver is not able to discriminate between new or retransmitted traffic (tbc);

- A link to H.248.84 defined TCP metrics should be indicated.

- An example of a derived statistic is added.

- There are SNMP-defined managed objects for TCP traffic. Relationships are indicated, being aware that the underyling TCP traffic instance may be principally different (H.248 = TCP Stream/Termination; SNMP = TCP/IP interface of a host entity).

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

References & abbreviations needs to be updated according agreed changes.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 11	TCP retransmission metrics package

|  | Package Name: | TCP retransmission metrics package |
| --- | --- | --- |
|  | Package ID: | tcprm (0x####) |
|  | Description: | This package is used to support explicit octet and packet count statistics for the TCP bearer protocol, related to TCP retransmission traffic. The statistics are applicable for all TCP modes of operation, as long as the MG is aware that the H.248 Stream carries TCP traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

None.

## 11.3	Signals

None.

## 11.4	Statistics

### 11.4.1	Retransmitted TCP Octets Sent

|  | Statistic Name: | Retransmitted TCP Octets Sent |
| --- | --- | --- |
|  | Statistic ID: | tcpros (0x0001) |
|  | Description: | This Statistic provides the number of retrasnmitted octets sent on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of octets (i.e., including TCP header) received in TCP packets. At the Termination level, it is equal to the sum of the ingress TCP flows over all Streams. NOTE – The semantic of this TCP metric is similar with the managed object tcpEStatsPerfOctetsRetrans of the TCP MIB [IETF RFC 4898]. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 11.4.2	Retransmitted TCP Octets Received

|  | Statistic Name: | Retransmitted TCP Octets Received |
| --- | --- | --- |
|  | Statistic ID: | tcpror (0x0002) |
|  | Description: | This Statistic provides the number of retrasnmitted octets received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of octets (i.e., including TCP header) received in TCP packets. At the Termination level, it is equal to the sum of the ingress TCP flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 11.4.3	Retransmitted TCP Packets Sent

|  | Statistic Name: | Retransmitted TCP Packets Sent |
| --- | --- | --- |
|  | Statistic ID: | tcprps (0x0003) |
|  | Description: | Provides the number of retransmitted packets sent on the Termination or Stream since the statistic has been set. The packets represent the egress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of TCP packets, inclusive of TCP connection establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the ingress TCP flows over all Streams. NOTE – The semantic of this TCP metric is similar with the managed object tcpRetransSegs of the TCP MIB [IETF RFC 4022]. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 11.4.4	Retransmitted TCP Packets Received

|  | Statistic Name: | Retransmitted TCP Packets Received |
| --- | --- | --- |
|  | Statistic ID: | tcprpr (0x0004) |
|  | Description: | Provides the number of retransmitted packets received on the Termination or Stream since the statistic has been set. The packets represent the ingress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of TCP packets, inclusive of TCP connection establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the ingress TCP flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

## 11.5	Error codes

None.

## 11.6	Procedures

### 11.6.1	Ingress TCP traffic – Statistic "Retransmitted TCP Packets Received"

Every incoming, retransmitted TCP packet successfully delivered to its H.248 Context and H.248 TCP Stream/Termination is counted by Statistic tcprm/tcprpr.

### 11.6.2	Ingress TCP traffic – Statistic "Retransmitted TCP Octets Received"

The measurement represents the volume of TCP retransmission of all TCP flows of an H.248 Stream, i.e. across all received, retransmitted TCP packets according Statistic tcprm/tcpror.

### 11.6.3	Egress TCP traffic – Statistic "Retransmitted TCP Packets Sent"

Every outgoing, retransmitted TCP packet, sent from a H.248 TCP Stream/Termination, is counted by Statistic tcprm/tcprps.

### 11.6.4	Egress TCP traffic – Statistic "Retransmitted TCP Octets Sent"

The measurement represents the volume of TCP retransmission of all TCP flows of an H.248 Stream, i.e. across all outgoing, retransmitted TCP packets according to Statistic tcprm/tcpros.

### 11.6.5	Derived statistics

The H.248 statistics of this package together with other TCP related statistics (such as from [ITU-T H.248.84] allow the derivation of other TCP performance metric. This clause provides one example.

#### 11.6.5.1	Performance metric “TCP efficiency”

This metric is defined in clause 4.2 in [IETF RFC 6349] according following equation:

TCP Efficiency % =Transmitted Bytes-Retransmitted BytesTransmitted Bytes×100

Metric TCP efficiency for an H.248 stream/termination could be calculated (by the MGC), using the two H.248 statistics TCP Octets Sent (tcptv/tcpos, see clause 10.4.1/[ITU-T H.248.84]) and Retransmitted TCP Octets Sent (tcprm/tcpros).

[End Proposed Correction]

____________________