| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4396 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Clauses 3 to 5 – Update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution collects a couple of update proposals for clauses 3 to 5.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.x	TCP packet [ITU-T H.248.84]: IP datagram (also known as IP packet) carrying a (single) TCP segment in the payload.

NOTE – Such a L4/L3 PDU is also known as TCP/IP packet (briefly TCP packet).

3.1.1	transport (TCP) proxy (translator) mode [ITU-T H.248.84], see clause I.4.1 in [b-ETSI TR 183 068]: mode (also known as Back-to-Back TCP Endpoint (B2BTE) mode): statefull forwarding of TCP packets in terms of full protocol termination. The end-to-end TCP connection is partitioned in two TCP connection legs by the BGF. Each H.248 Stream endpoint provides a statefull TCP connection state machine.

NOTE 1 – The term proxy mode is similar as used for HTTP proxy, FTP proxy, SIP proxy, etc.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

{Edittor’s note (2013-01): it is yet undecided whether terms 3.2.1, 3.2.2 and 3.2.3 remain in H.248.TCP due to the fact that they are more“H.248 generic”, may be thus rather subject of H.248.1. Examples: applicable as well for H.248.78, H.248.TLS, H.248 “T.38 control” (= T.38 Annex E.).}

3.2.1	Basic H.248 master-slave control mode: An H.248 decomposed gateway following an hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship. With MGC as master and MG as slave entity due to considered distribution models on the decomposed gateway entities

NOTE 1: [ITU-T Q.Sup31] Annexes C.2 or C.3 provides examples of functional-to-physical mapping models according basic H.248 master-slave control mode.

3.2.2	MG autonomous mode: For a particular function (NOTE 2), the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited on decisions with MG available information only.

NOTE 2: Examples: a) state transitioning between multiple media configurations (e.g., [b-ITU-T V.152], b) bearer-level ALG [b-ITU-T H.248.78].

3.2.3	MGC-strictly controlled mode: Synonym to the basic H.248 master-slave control mode. All control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit.

3.2.4	TCP proxy variants: the basic TCP proxy mode is defined in clause 3.1.1. The following TCP proxy variants are useful for the various TCP services as supported by H.248 MGs:

There are two orthogonal attributes:

- Discrimination with respect to TCP payload data awareness:

- Application-aware TCP proxy (the MG knows the carried protocol (stack) by TCP packets)

- Application-agnostic TCP proxy (the MG is unaware of the TCP payload content)

- Discrimination with respect to statefull TCP connection control:

- Statefull TCP proxy (by support of the complete TCP connection state machine, see Figure 6 in [IETF RFC 793])

- Lightweight TCP proxy (by support of simplified TCP connection state models)

Example: “application-agnostic, statefull TCP proxy” mode.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2BTE | Back-to-Back TCP Endpoint |
| --- | --- |
| BCP | Bearer Control Protocol |
| BGF | Border Gateway Function |
| BNC | Backbone Network Connection |
| DTLS | Datagram Transport Layer Security |
| FTP | File Transfer Protocol |
| GCP | Gateway Control Protocol |
| HTTP | Hypertext Transfer Protocol???? |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| MPTCP | Multipath TCP |
| MSRP | Message Session Relay Protocol |
| NAT | Network address translation |
| NAT-T | NAT traversal |
| NGN | Next generation network |
| RD | Remote descriptor |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | SEP Pair |
| SIP | Session initiation protocol |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| UDP | User datagram protocol |
| UE | User equipment |
| W3C | World Wide Web Consortium |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 5	Conventions

## 5.1	Naming of stream endpoints and terminations

The notations “Tx” and “Tx(Sy)” are used for naming of terminations and stream endpoints (SEP) respectively, with

- “Tx” as TerminationID value;

- “Sy” as StreamID value and

- numerical variables x an y.

## 5.2	TCP bearer

The notation of TCP bearer connection is synonym to a TCP connection in the network user plane. There might be also a control plane TCP connection (between MGC and MG) in case of a TCP-based H.248 transport mode [ITU-T H.248.67].

NOTE 1 – This relates to the so called backbone network connection (BNC) concept in [ITU-T Q.1950], which is referred by clause 8.

## 5.3	TCP bearer control

There are connection control information flows for TCP bearer connections in the network user plane (e.g., IP media/data/bearer plane) and the control plane (here H.248). Figure 2 outlines the used conventions for the differentiation of both control flows:

Figure 2 – Conventions for TCP bearer control

[End Proposed Correction]

____________________