| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4397 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Clause 6 – Update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes two change requests for clause 6.

1	Introduction

The evaluation of example interworking functions, and their possible interaction with TCP, is not yet fully complete. The completion of this task is purpose of this contribution.

2	Discussion

Following open items are tried to be clarified:

## 2.1	L3 IWF “IPR mode” vs “TCP modes”

The basic question is how the three main TCP modes (TCP relay, TCP merge, TCP proxy) align with the two main IP modes (B2BIH and IPR), - from H.248 MG perspective.

Our analysis:

- B2BIH: all three TCP modes could be realized on top of B2BIH. It may be reminded again that the B2BIH mode would be enforced by applying local NA(P)T.

- IPR: the TCP relay & merge mode could be enabled together with IPR mode; however, the definition of TCP proxy mode is only consistent with B2BIH mode.

See correspondent change proposal in Table 3.

## 2.2	L4+ IWF “MSRP relay” vs “MSRP switch”

There’s an error in Table 3 with regards to a confused usage of MSRP switch and MSRP relay. The MSRP relay is introduced by RFC 4976 whereas MSRP switch denotes the H.248.69 entity:

H.248.69, § 3.2.3	MSRP switch: A media level entity that receives MSRP messages and delivers them to the other conference participants. An MSRP switch has a similar role to a conference mixer with the exception that an MSRP switch does not actually "mix" together different input media streams; it merely relays the messages between participants.

RFC 4976	 MSRP relay: an MSRP node that forwards messages and delivery status and may provide policy enforcement.  Relays can fragment and reassemble portions of messages.

Side comment to H.248.69:

There’s a possible IG item: H.248.69 contains RFC 4976 in the (clause 3) list of References, however, the RFC is not referred from the main body of the Recommendation!

We believe that RFC 4976 should be deleted because the MSRP protocol extensions (as defined by this RFC) are not supported by H.248.69.

Alternatively, H.248.69 could be revised for RFC 4976 support …

The TCP perspective:

- MSRP switch: The H.248.69 MSRP message handling model concludes an underlying TCP proxy mode. We think it’s even correct to state an application-aware, statefull TCP proxy mode because the MG is aware about MSRP as application traffic and the particular MSRP message processing and forwarding implies the full protocol termination of TCP.

- MSRP relay: primary purpose of RFC 4976 is NAT traversal support in our understanding. The MG would need to support local NAT-T besides a local NAT function for TCP.
Whether the NAT-T function is limited to L3/L4 only, or whether also a B-ALG support (H.248.78) may be needed, is dependent on the prime H.248 entity (only MGC or also MG?) concerning MSRP message processing. (TBC)

- See correspondent change proposal in Table 3.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

References & abbreviations needs to be updated according agreed changes.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

### 6.2.3	Model with focus on context-internal interworking function

Figure 6 depicts a two termination model with scope on example interworking functions.

Figure 6 – Two-termination context with context-internal interworking function

The IWF is principally a context-level function, associated to a particular SEPP (here: SEPs T1(S1) and T2(S1)). Table 3 lists some example IWFs, which are relevant in scope of this Recommendation and TCP processing, classified from a layered protocol perspective:

Table 3 – Example IWFs

| IWF: | Example functions: | Comments from TCP perspective: |
| --- | --- | --- |
| L3 IWF | - Back-to-back IP host (B2BIH) | Conditional useful for all kind of TCP interworking (NOTE 1). |
|  | - B2BIH with: - Network address translation (NAT) - Network prefix translation (NPT in case of IPv6) |  |
|  | - IP protocol version interworking (IPv4-to-IPv6 or vice versa) |  |
|  | - NAT-T according [ITU-T H.248.37] | No impact (NOTE 2). |
|  | - IP router (IPR) [ITU-T H.248.64] - IPR with NAT | Conditional for all kind of TCP interworking: TCP relay and TCP merge mode would be inline with IPR mode, whereas the TCP proxy mode would be rather B2BIH (NOTE 4). |
| L4 IWF | - this Recommendation | There are two crucial aspects to be noted: - Cut-through characteristic (“when the two SEPs are interconnected for enabled traffic flow”); - BCP (for TCP) information of one SEP as possible stimuli for procedures at partner SEP, or not? (NOTE 3) |
|  | - NAT-T according [ITU-T H.248.84] | Impact, see clause 7.4. |
| L4+ IWF | - MSRP switch [ITU-T H.248.69] | For MSRP-over-TCP, bearer type indication is not explicitly specified, but supposed to be part of SDP “m=” line (see clause 6.2/[ITU-T H.248.69]). (TBC) For MSRP-over-TLS: unspecified, indicated as FFS by [ITU-T H.248.69]. |
|  | - MSRP relay [IETF RFC 4976] | MSRP relays facilitate NAT-T. The MSRP relay function could be provided by an H.248 gateway, e.g. using a similar mapping model as [ITU-T H.248.69]. However, [ITU-T H.248.69] does not (yet) support a “MGC controlled MSRP relay”. (TBC) |
|  | - Bearer-level ALG [ITU-T H.248.78] | Interactions to be considered. |
|  | - Transport level security [ITU-T H.248.TLS] | There are impacts, dependent on TLS-to-TLS or TLS-to-non-TLS type of interworking. |
| NOTE 1 – Exception: attention to ICMP (see clause 6.7/[ITU-T H.248.79], such as TCP import ICMP information with end-to-end significance. NOTE 2 – This is rather a SEP related function than SEPP scope. NOTE 3 – Example: an incoming TCP connection release triggers an outgoing TCP connection release, or not. NOTE 4 – The “full protocol termination” as part of TCP proxy mode implies two separate, addressable IP transport endpoints, which are only subject of B2BIH mode. |  |  |

[End Proposed Correction]

____________________