| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4402 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Clause 10 – Update of TLS session maintenance package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates according the agreements of the 2013-01 meeting.

1	Introduction

Contributions C-4 and C-14 from last meeting (2013-01) resulted in an agreement in drafting three individual H.248 packages for TLS bearer control.

Copy of COM 16 – C 14/Figure 1 – H.248.TLS – Package redesign proposal – Overview

This contribution continues the update process as initiated by C-20 of the TLS session connection control package.

2	Discussion

The TLS session maintenance package is related to the bearer path service concerning TLS alerts. The TLS alert protocol defines two fundamental alert categories:

- Closure alert; used in normal service scenarios (“TLS session release”), thus covered by the TLS basic session control package; and

- Error alerts, used in abnormal situations, subject of the TLS session maintenance package (this contribution)-

There are arround 25 alert types within the error alert category. The TLS alert protocol defines two severity levels for further qualification of each alert type:

- Warning level: alert provides only a warning indication, the communication should/could continue; and

- Fatal level: continuation of communication not possible, the TLS session shall be release.

Thus, we got so far three dimensions in structuring TLS alert activities: a) alert category, b) alert type and c) alert severity.

The next item of discussion would be to evaluation the likely location of the served user instance, responsible for processing the particular TLS alert information. There are three options in case of TLS protocol handling by a distributed architecture such as H.248 gateways:

- MG: the MG itself processes and terminates an incoming TLS alert or sents a TLS alert to the remote TLS endpoint, without any involvement of other remote control/management plane elements. Thus, scenario with MG local TLS alert processing.

- MGC: either, the MGC may trigger the MG to sent a specific TLS alert to the remote TLS endpoint, e.g. due to security aspects, application logic, etc.; or the MGC would be notified by the MG by incoming TLS alert information (Note: which again may be mapped to an H.248.8 error code).

- NMS (element management system of MG): e.g., in case of particular TLS alerts resulting in fault or security management actions.

We’ve outlined now a framework for TLS alert handling by H.248 gateways. The next step would be to evaluate the TLS alert protocol in more detail, which relates to an analysis of the TLS error alerts as defined by clause 7.2.2/RFC 5246. We could make following observations:

- roughly 70 % of the (ca. 25) error alerts are of fatal severity, i.e., shall trigger the release of the TLS session;

- there are two error alerts which may appear only as warnings; and

- a few error alerts are related to “older TLS versions”.

3	Conclusions

The individual handling of specific TLS error alerts should be implementation specific and out of scope of [H.248.TLS]. However, a general mechanism should be support to notify the MGC about received TLS alert information by the MG, - for instance the MGC must be notified about the fatal severity in order to initiate the release of a TLS session -, and the MGC should be able to trigger the MG in sending specific TLS alerts (e.g., due to application specific requirements).

The existing package draft for tlsm is still satisfying above H.248 requirements, primarily by an event (swa) and a signal (wal) related to TLS session warning alerts. We still need to add the general procedures for these H.248 elements.

Furthermore, there are two open aspects:

- the link of TLS fatal serverity to “TLS session release” procedures, as they are already adressed by the TLS basic and extended session control packages;

- ### lower layer ###

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 10	TLS session maintenance package

|  | Package Name: | TLS session maintenance package |
| --- | --- | --- |
|  | Package ID: | tlsm (0x####) |
|  | Description: | This package defines capabilities for support of TLS alerts, which are exchanged between the remote and MG local TLS endpoint, following the TLS alert protocol. This package version is limited to the so-called TLS error alerts, as defined by clause 7.2.2/[IETF RFC 5246]. The package provides capabilities for MGC triggered sending of TLS alerty by the MG, as well as a reporting mechanism to notify the MGC about received TLS alerts by the MG. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

None.

## 10.2	Events

### 10.2.1	TLS Error Alert

|  | Event Name: | TLS -error alert |
| --- | --- | --- |
|  | Event ID: | mgea (0x0003) |
|  | Description: | This event indicates an abnormal condition for the TLS layer that does not lead to the termination of the TLS session. The abnormal condition may be detected on the local or on the remote TLS endpoint. The event is limited on TLS error alerts and excludes the TLS closure alert event. The event shall be set at stream level. |

#### 10.2.1.1	EventsDescriptor parameters

None

#### 10.2.1.2	ObservedEventsDescriptor parameters

##### 10.2.1.2.1	Bearer level Alert Initiator

|  | Parameter Name: | Bearer level alert initiator |
| --- | --- | --- |
|  | Parameter ID: | blai (0x0001) |
|  | Description: | Indicates on which TLS endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Abnormal condition detected by the MG local TLS endpoint “remote” (0x0002) Abnormal condition detected by the remote TLS endpoint |
|  | Default: | NA |

##### 10.2.1.2.2	Error Alert Type

|  | Parameter Name: | Error alert type |
| --- | --- | --- |
|  | Parameter ID: | eat (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition by referencing the TLS error alert type. The codepoints are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml ). |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml ). |
|  | Default: | NA |

## 10.3	Signals

### 10.3.1	MGC triggered TLS error alert

|  | Signal Name: | MGC triggered TLS error alert |
| --- | --- | --- |
|  | Signal ID: | mgcea (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates “fatal” the session will be terminated. NOTE: the TLS “close_notify(0)” alert is excluded because it does not belong to the TLS error alert category. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

10.3.1.1	Additional parameters

10.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246], i.e., either a warning or fatal alert severity. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

10.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, whichs are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml ). |
|  | Default: | NA |

## 10.4	Statistics

None.

## 10.5	Error codes

None.

## 10.6	Procedures

### 10.6.1	TLS warning error alerts

#### 10.6.1.1	General behaviour

The TLS session should keep active (and not released) when TLS alerts with warning severity are exchanged between local and remote TLS endpoints.

#### 10.6.1.2	MG received TLS error alerts from remote TLS endpoint

The MGC may subscribe to be notified about TLS warning error alerts via event mgea. Whenever the MG receives an incoming TLS error alert at warning level, it shall be notified towards MGC by

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to “IANA registered codepoint”; and

- the tagging mgea/blaw equal to “remote”.

#### 10.6.1.3	MG locally generated TLS error alerts and sent to remote TLS endpoint

Same as previous clause 10.6.1.1, but a tagging mgea/blaw equal to “local”.

#### 10.6.1.4	MGC triggered TLS error alert, sent by the MG to remote TLS endpoint

When the MGC provides signal mgcea, then the MG shall generate a TLS alert message based on the information as contained in signal parameters al and ad, and sent the TLS alerts towards the remote TLS endpoint.

### 10.6.2	TLS fatal error alerts

#### 10.6.2.1	General behaviour

Whenever a TLS fatal error alert is exchanged between local and remote TLS endpoints, there will be always a subsequent, immediate release procedure of the TLS session.

#### 10.6.2.2	MG received TLS fatal error alerts from remote TLS endpoint

As in clause 10.6.1.2. The subsequently expected TLS closure_notify alert indicates a TLS session release request.

[…FIXTHIS]

{Editor’s note (2013-06): the procedure overlapps with the TLS basic session control package procedures of clause 8.6.5.1. The text is not yet aligned. Need to be fixed.}

#### 10.6.2.3	MG locally generated TLS fatal error alerts and sent to remote TLS endpoint

As in clause 10.6.1.3. The MG shall sent a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure is either triggered by the MGC (see clause x.y.z) or autonomously started by the MG itself (see clause x.y.z).

[…FIXTHIS]

{Editor’s note (2013-06): the procedure overlapps with the TLS basic and extended session control package procedures. The text is not yet aligned. Need to be fixed.}

#### 10.6.2.4	MGC triggered TLS fatal error alert, sent by the MG to remote TLS endpoint

As in clause 10.6.1.4. The MG shall sent a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure is either triggered by the MGC (see clause x.y.z) or autonomously started by the MG itself (see clause x.y.z).

[…FIXTHIS]

{Editor’s note (2013-06): the procedure overlapps with the TLS basic and extended session control package procedures. The text is not yet aligned. Need to be fixed.}

### 10.6.3	Lower layer issues

There might be issues in the protocol layers below the TLS layer.

Example use case:

- Established IP bearer path, i.e. there is an active TLS session;

- The underlying IP transport connection (e.g., TCP or SCTP transport connection) is released from the remote side, but without any TLS closure alert indication;

- Hence, there would be still an established TLS session endpoint, but a released L4 bearer connection from a local MG perspective;

- And therefore the TLS session would be interrupted, not any TLS alert could be sent to the remote side.

Procedures for the maintenance of lower layer issues are out of scope of this package, rather subject of exsting H.248 error handling capabilities.

[End Proposed Correction]