| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4403 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Clause 11 – Update of TLS traffic volume metrics package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates according the agreements of the 2013-01 meeting.

1	Introduction

Contributions C-4 and C-14 from last meeting (2013-01) resulted in an agreement in drafting three individual H.248 packages for TLS bearer control.

Copy of COM 16 – C 14/Figure 1 – H.248.TLS – Package redesign proposal – Overview

This contribution continues the update process as initiated by C-20 of the TLS session connection control package.

2	Discussion

The semantics could be based on the TCP traffic volume metrics package (clause 10/H.248.84).

It was noted that we need to distinguish TLS-to-TLS and TLS-to-non-TLS interworking modes with respect to “application data” related statistics.

Thus, we’d like to extend the package by two additional statistics with support of “octet-based TLS traffic volume”, which are applicable for all TLS interworking modes.

It was also noticed that the terminology used is not fully consistent, reflected by a new editor’s note in order to solve this issue at the next meeting.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 11	TLS traffic volume metrics package

|  | Package Name: | TLS traffic volume metrics package |
| --- | --- | --- |
|  | Package ID: | tlstv (0x####) |
|  | Description: | This package is used to support explicit octet and packet count statistics for the TLS protocol. The statistics are applicable as long as the MG is aware that the H.248 Stream carries TLS traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

None.

## 11.3	Signals

None.

## 11.4	Statistics

{Editor’s note (2013-06 meeting): the terminology (and hence semantics) is not yet consistent across all statistics defined by this package. E.g., terms “TLS fragment”, “TLS message”, “TLS application data”, “TLS-over-L4 packet”. We need a data model which is tightly linked to the data structures of RFC 5246 (“the TLS RFC is unfortunately lacking precise definitions”). The statistic definitions should be then updated again.}

### 11.4.1	Number of Received TLS-Fragments

|  | Statistic Name: | Number of Received TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recfrag (0x0001) |
|  | Description: | Provides the number of packets received on the Termination or Stream since the statistic has been set. The packets represent the ingress TLS-fragments of all TLS flows of an H.248 Stream. It is the total number of TLS-over-L4 packets, inclusive of TLS session establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 11.4.2	Number of Sent TLS-Fragments

|  | Statistic Name: | Number of Sent TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentfrag (0x0002) |
|  | Description: | Provides the number of packets sent from the Termination or Stream since the statistic has been set. The packets represent the egress TLS-fragments of all TLS flows of an H.248 Stream. It is the total number of TLS-over-L4 packets, inclusive of TLS session establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 11.4.3	TLS Octets Sent

|  | Statistic Name: | TLS Octets Sent |
| --- | --- | --- |
|  | Statistic ID: | tlsos (0x0003) |
|  | Description: | This Statistic provides the number of octets sent from the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress TLS messages of all TLS flows of an H.248 Stream, excluding all lower layer overhead. It is the total number of octets (i.e., including TLS header) transmitted in TLS messages. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 11.4.4	TLS Octets Received

|  | Statistic Name: | TLS Octets Received |
| --- | --- | --- |
|  | Statistic ID: | tlsor (0x0004) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of octets received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress TLS messages of all TLS flows of an H.248 Stream excluding all lower layer overhead.. It is the total number of octets (i.e., including TLS header) received in TLS messages. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 11.4.5	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0005) |
|  | Description: | This statistic logs the number of application data octets that are delivered to the layer above TLS. Received application data messages from the remote TLS endpoint are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 11.4.6	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0006) |
|  | Description: | This statistic logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

## 11.5	Error codes

None.

## 11.6	Procedures

### 11.6.1	Ingress TLS traffic – Statistic "TLS Packets Received"

Every incoming L4 packet, carrying a TLS fragment, which is successfully delivered to its H.248 Context and H.248 TLS Stream/Termination is counted by Statistic tlstv/recfrag.

### 11.6.2	Ingress TLS traffic – Statistic "TLS Octets Received"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all received TLS-over-L4 packets according Statistic tlstv/tlsor.

### 11.6.3	Ingress TLS traffic – Statistic "TLS Application Data Octets Received"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all received TLS-over-L4 packets according Statistic tlstv/recado.

### 11.6.4	Egress TLS traffic – Statistic "TLS Packets Sent"

Every outgoing TLS-over-L4 packet, sent from a H.248 TLS Stream/Termination, is counted by Statistic tlstv/sentado.

### 11.6.5	Egress TLS traffic – Statistic "TLS Octets Sent"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all sent TLS-over-L4 according to Statistic tlstv/tlsos.

### 11.6.6	Ingress TLS traffic – Statistic "TLS Application Data Octets Sent"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all sent TLS-over-L4 packets according Statistic tlstv/sentado.

[End Proposed Correction]