| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4404 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Clause 12 – Update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes initial input to clause 12.

1	Introduction

The purpose of clause 12 is to describe applicability and non-applicability of package-less TLS control options.

The underlying idea is to reuse SDP whenever possible. Usage of SDP is typically impossible when either SDP is missing or when SDP is already semantically overloaded.

NOTE: AVD-4421 = correspondent contribution for H.248.TCP.

2	Discussion

Figure 1 makes an attempt in indicating all possible control steps (from H.248 perspective) concerning the establishment of TLS security sessions plus underlying IP transport connections in environments demanding for NAT traversal support (typically L4/L3 NAT-T and L4+ NAT-T):

Figure 1 – Principal H.248 control steps for establishing the IP bearer path for “TLS-over-L4” inclusive all optional local NAT and NAT traversal support functions

Prerequisite for the start of TLS security session establishment (7) is the availability of a successfully established L4 connection (6). Thus, end-to-end L4/L3 connectivity (across the TLS network domain) is required, which implies the possible usage of NAT-T tools (5). It may be noted that later on a bearer-level ALG (i.e., a L4+ NAT-T tool) might be also needed, at the point in time when application layer PDUs transported across the TLS session (see step (10)).

The goal of package-less TLS control is to identify possible SDP elements for reuse at H.248 interface (if possible at all), without their semantical modification as defined by IETF RFCs.

In case of TLS control, at least two SDP elements would be in scope:

- SDP “m=” line <proto> element, because we don’t want to extend the H.248 bearer network characteristics package for TLS bearer type (but the SDP element seems to be ambiguous);

- SDP “a=fingerprint:”, which binds the call signalling to the media.

The previous control steps are discussed and summarized by Table 1:

Table 1 – H.248 control steps and possible control elements

| H.248 control step: |  | H.248 property: | H.248/SDP element: | Comments: |
| --- | --- | --- | --- | --- |
| Establishment phase I: „IP transport connection establishment“ |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | “m=”-line <proto> (clause 13.4 / H.248.84, H.248.69) | SDP may be ambiguous due to “protocol stack” concept. 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property, e.g., TCP: tcpbcc/bceb | “a=setup:holdconn” (?) |  |
| 4 | Start IP Transport Connection Establishment Procedure (e.g. TCP) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to H.248.84 “a=setup:” semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | H.248.37 | “a=setup:” for H.248.84 NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State „Established“) | Event, e.g., TCP: tcpbcc/BNCChange | — |  |
| Establishment phase II: „IP security session establishment“ |  |  |  |  |
| 7 | Bearer type (L4+) indication to SEP | Indirect: package elements used from TLS-related H.248 packages | “m=”-line <proto> | NOTE 1. SDP may be ambiguous due to “protocol stack” concept. |
| 8 | Block / Unblock L4+ Bearer Control Procedures at SEP | Property: tlsbsc/bceb | None (?) | i.e., handling of - TLS handshake protocol - TLS cipher change protocol - TLS alert protocol |
| 9 | Start IP Security Session Establishment Procedure („TLS negotiations“) | Event: tlsbsc/BNCChange Signal: tlsbsc/EstBNC | None (?) | above TLS protocols are enabled |
| 10 | Local L4+ NAT traversal support | H.248.78 | e.g., MSRP: “a=msrp-cema” (?) | NOTE: L4+ NAT-T may be also requested for non-TLS. |
| 11 | Successfully established IP Transport Security Session | Event: tlsbsc/BNCChange | — |  |
| Communication phase: |  |  |  |  |
| 12 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – Step 7 may be merged with step 1 in case of tightly coupled “TLS/L4” control. |  |  |  |  |

3	Conclusions

The discussion about (non-)applicability of SDP is only relevant for steps 2, 3, 4, 5 and 7. The SDP method for step 2 may work for native TCP bearers (e.g., non-TLS scenario). Most critical are steps 3, 4 and 5 due to the single SDP attribute referred (“semantically overloaded”), which implies possible interactions between these control steps.

More detailed information about steps 3 to 5, which are dependent on the TCP mode:

- TCP relay mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP merge mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP proxy mode: steps 3 to 5 see above Table 1.

4	Proposal

It is suggested to add above preliminary analysis to clause 12.

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 12	Package-less TLS control

{Editor’s note (2013-01 meeting): dedicated chapter on SDP based control required, addressing the following

Purpose of this clause is to describe the SDP-based semantics for TLS control, which needs the consideration of

- SDP usage in H.248.69, H.248.84

- how the package-based elements/procedures would be realized via SDP?

- which functionality would be out of scope of SDP control?

- assumption of mixed Property- & SDP-based control

First comments (from C-20):

To requirement of “Originating/Terminating Bearer control protocol procedures”:

[other package elements]

Contributions are solicited.}

## 12.1	Related to TLS bearer establishment

### 12.1.1	Overview – principal H.248 control steps

{Editor’s note (2013-06 meeting): the text is this clause is still subject of discussion … }

Figure x1 makes an attempt in indicating all possible control steps (from H.248 perspective) concerning the establishment of TLS-over-L4/IP bearer connections in environments demanding for NAT traversal support. The dashed boxes are L4-dependent (e.g., in case of TCP see [ITU-T H.248.TCP, H.248.69, H.248.84]):

Figure x1 – Principal H.248 control steps for establishing a TLS-over-L4 bearer

It is apparent that some steps might be combined (e.g., 1, 2 and 7) and that specific steps might be optional (e.g., 3, 5, 8, 10).

Table y1 provides a preliminary inventory of possible control elements from H.248 perspective (at the example of TCP as L4 protocol). The column “H.248/SDP element” refers to “package-less TLS control”.

Table y1 – H.248 control steps and possible control elements

| H.248 control step: |  | H.248 property: | H.248/SDP element: | Comments: |
| --- | --- | --- | --- | --- |
| Establishment phase I: „IP transport connection establishment“ |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | “m=”-line <proto> (clause 13.4 / H.248.84, H.248.69) | SDP may be ambiguous due to “protocol stack” concept. 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property, e.g., TCP: tcpbcc/bceb | “a=setup:holdconn” (?) |  |
| 4 | Start IP Transport Connection Establishment Procedure (e.g. TCP active or passive OPEN) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to H.248.84 “a=setup:” semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | H.248.37 | “a=setup:” for H.248.84 NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State „Established“) | Event, e.g., TCP: tcpbcc/BNCChange | — |  |
| Establishment phase II: „IP security session establishment“ |  |  |  |  |
| 7 | Bearer type (L4+ = ‘TLS’) indication to SEP | Indirect: package elements used from TLS-related H.248 packages | “m=”-line <proto> | NOTE 1. SDP may be ambiguous due to “protocol stack” concept. |
| 8 | Block / Unblock L4+ Bearer Control Procedures at SEP | Property: tlsbsc/bceb | None (?) | i.e., handling of - TLS handshake protocol - TLS cipher change protocol - TLS alert protocol |
| 9 | Start IP Security Session Establishment Procedure („TLS negotiations“) | Event: tlsbsc/BNCChange Signal: tlsbsc/EstBNC | None (?) | above TLS protocols are enabled |
| 10 | Local L4+ NAT traversal support | H.248.78 | e.g., MSRP: “a=msrp-cema” (?) | NOTE: L4+ NAT-T may be also requested for non-TLS. |
| 11 | Successfully established IP Transport Security Session | Event: tlsbsc/BNCChange | — |  |
| Communication phase: |  |  |  |  |
| 12 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – Step 7 may be merged with step 1 in case of tightly coupled “TLS/L4” control. |  |  |  |  |

{Editor’s note (2013-06 meeting): possible conclusions

The discussion about (non-)applicability of SDP is only relevant for steps 2, 3, 4, 5 and 7. The SDP method for step 2 may work for native TCP bearers (e.g., non-TLS scenario). Most critical are steps 3, 4 and 5 due to the single SDP attribute referred (“semantically overloaded”), which implies possible interactions between these control steps.

Steps 3 to 5 in more detail, dependent on TCP mode:

- TCP relay mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP merge mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP proxy mode: steps 3 to 5 see above Table y1.

}

### 12.1.2	Step 1: Bearer endpoint creation

Not SDP related, based on H.248 Command request.

### 12.1.3	Step 2: Bearer type indication for IP transport “L4/L3”

Starting point should be the SDP-based approach by using the SDP “m=” line <proto> element as transport indicator. The SDP method shall work for all required TLS transport modes (see clause 6.1/[ITU-T H.248.TLSPROF]).

Conditional approach, dependent on:

C3.1: unambiguous indication of transport protocol “L4” (Issue: “m=”-line <proto> relates to a protocol stack based indicator, rather than a distinct identifier to a specific L4 protocol)

C3.2: consistent LD/RD signalling (due to the fact of a LCD level characteristic)

### 12.1.4	Step 3: Block / Unblock L4 bearer control procedures at SEP

Out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 12.1.5	Step 4: Start IP transport connection establishment procedure

Relevant for connection-oriented IP transport protocols, which are used as TLS transport modes (see clause 6.1/[ITU-T H.248.TLSPROF]), but out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 12.1.6	Step 5: Local L4/L3 NAT traversal support

Crucial step because TLS will fail without any successfully established IP transport connection. However, out of scope of this Recommendation. There are dedicated L4/L3 NAT-T support methods, such as [ITU-T H.248.37], [ITU-T H.248.50] or [ITU-T H.248.84].

It should be noted that above example NAT-T mechanisms are primarily enabled using H.248 packages (“and not via LD/RD-embedded SDP signalling”).

### 12.1.7	Step 6: Reporting successfully established IP transport connection

Not supported by SDP, but out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 12.1.8	Step 7: Bearer type indication for TLS

If SDP based, then only same method as in step 2.

Conditional approach, dependent on:

C7.1: unambiguous indication, see C3.1.

### 12.1.9	Step 8: Block / Unblock TLS security session control procedures at SEP

Not supported by SDP.

### 12.1.10	Step 9: Start TLS security session establishment procedure

There is not any SDP as explicit indicator for outgoing and incoming TLS “handshake procedures” as part of TLS session establishment.

### 12.1.11	Step 10: Local L4+ NAT traversal support

FIXTHIS

### 12.1.12	Step 11: Reporting successfully established TLS session

Not supported by SDP.

### 12.1.13	Step 12: Active communication phase

Control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TLS traffic) are out of scope of the SDP.

## 12.2	Related to TLS authentication

NOTE 1 – This clause satisfies requirement R-8.3.3.2.2/1//[ITU-T H.248.TLSPROF].

The SDP “a=fingerprint:” attribute (according [IETF RFC 4572]) shall be used under the condition of H.248 gateways involved in network architectures (given by trust model and SIP/SDP as application control protocol) as described by this RFC.

NOTE 2 – The SDP based method is exclusive, i.e., there are not any alternative H.248 signalling elements defined.

If the SDP “a=fingerprint:” element would be required by an H.248 profile, then

- the MGC would provide a fully specified SDP fingerprint attribute in the H.248 RD and

- the MGC would provide either a fully specified or wildcarded (see [ITU-T H.248.39]) SDP fingerprint attribute in the H.248 LD

to the MG.

Usage of the the SDP “a=fingerprint:” attribute at the H.248 interface validates the bearer-to-call binding between SIP call and TLS secured bearer (see e.g. Figure 6/[ITU-T H.248.TLSPROF]).

## 12.3	Related to TLS bearer release

FIXTHIS

# 13	Security considerations

…

[End Proposed Correction]

____________________