| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4405 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Appendix I – Update proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to Appendix I.

2	Discussion

Appendix I contains three application scenarios with TLS. They are all still valid. The 3rd one should be moved to H.248.DTLS. The use case numbering should be revised in order not to be confused with the bearer connection network use cases of clause 6.2.1.

Furthermore, some editorial updates are suggested.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix I

Sample use-cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use case “Terminal-to-TLS MG session, provisioned TLS service negotiation”

This use case illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain; the MGC could be outside (as shown) or inside this TLS-domain, but the MGC is in any case aware of the applied TLS network domain profile for the secured bearer traffic.

- IP transport bearer type: ‘TCP’ in this example (but might be also e.g. SCTP).

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by transport options SIP/SCTP/IPsec, SIP/DTLS/UDP/IP, SIP/TLS/TCP, etc.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS (see also clause 6.2.1)

- Bearer path protocol layering: Application/TLS/TCP/IP (i.e., not any tunnelled transport).

- UE: TCP client and TLS client, MG: TCP server and TLS server.

- H.248 MG TCP-mode: TCP-proxy.
NOTE – In more detail: L4/L3 NAT traversal support requirements during the establishment phase of TCP bearer connection may imply e.g. initially a TCP merge mode (see [ITU-T H.248.84]), which is later on transitioning to a TCP proxy mode (due to the TLS protocol termination here for TLS to non-TLS interworking). Another option is already the TCP proxy mode from the very beginning, e.g., in case of lacking NAT-T support.

- Mutual TLS-authentication via certificates, MG’s certificate is signed, UE’s certificate is self-signed.

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG (e.g., in case of an H.248 profile without support of the TLS capability negotiation package, see clause XY).

Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG

## I.2	Use case “Terminal-to-TLS MG session, MGC-controlled TLS service negotiation”

This use case is an extension of the first use case (clause I.1) by allowing the MGC to influence the TLS service negotiation at bearer level via usage of the the TLS capability negotiation package (see clause XY). The first use case is based on preconditions which are partially subject of network provisioning. This use case is motivated by the fact that there are a number of reasons which demand for a MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

## I.3	Use case #3: WebRTC to NGN/IMS interworking function with DTLS-to-TLS support

To be inserted.

{Editor’s note (2013-01): this use case is also considered from TCP perspective, thus covered by Appendix I.1/H.248.TCP. I.e., there might be a reference added later on in this clause …}

[End Proposed Correction]

____________________