| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4408 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: use case "T.38-over-UDPTL/DTLS/UDP" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes the addition of a further use case to Appendix I.

Discussion and conclusions

T.38 defines IP transport modes which are based on UDP and TCP. Transport level security itself is not specified by T.38, but may be principally applied. Such a security scenario becomes more and more interesting in case of so-called Internet-aware fax devices, i.e., when one T.38 endpoint is directly located in an IP terminal (the “T.38 IAF endpoint”).

This use case is special due to relevance for PSTN access/trunking gateways, whereas all our existing use cases are related to border gateways.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

clause 3: [ITU-T T.38] Recommendation ITU-T (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix I

Sample use-cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

...

## I.x	Use-case #X:   TLS-based transport security for facsimile packet relay service T.38

Background:

[ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected for IP networks with IP transport layer security requirements.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [ITU-T V.153];

- such a network configuration relates, - for both connection models -, to the “TLS to non-TLS” interworking case (see clause 6.2.1);

- UDP transport implies DTLS;

[End Proposed Correction]

____________________