| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4410 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLSPROF: Discussion - Security risk "if MG as TLS server … size of certificate …" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes add a further requirement driven by security aspects.

Motivation

The remote TLS bearer endpoint might be located in a UE, CPE or any kind of untrusted environment. There is then an unsure situation during the establishment of a TLS session, which might be subject of TLS-based security attacks. There are many well-known threat scenarios known by there the “TLS client” is attacking the TLS server (i.e., related to the very first TLS ClientHello message).

Such a security risk scenario is in particular fairly serious in case of H.248 MG when serving thousands of TLS sessions endpoints in parallel.

There are many solutions arround as countermeasure, such as

- specific TLS version

- TLS message padding proposals (e.g., http://tools.ietf.org/html/draft-pironti-tls-length-hiding-00)

- regulation of TLS client/server role assignments

- etc, etc

Conclusion

The MGC must be able to assign the TLS client role to the MG in order to prevent security attacks from a remote “TLS client” against the MG as TLS server. The MGC assignment is basically conditional, dependent on security policy rules, call origination (from trusted/untrusted network zones), etc.

A correspondent requirement should be added to clause 8.2.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

## 8.2	Client/Server Mode

R-8.2/1: The MG is required to support both the TLS server and TLS client mode.

The TLS standards do not make any correlation between the initiation of the underlying transport connection and the TLS server/client mode. Thus this mode is decoupled from the (TCP) SDP “a=setup:” attribute, as used for TLS-based SDP indications (see clause 4/[IETF RFC 4572]).

R-8.2/2: The MGC is required to indicate the TLS server/client role to the MG.

NOTE 1 – There is not any existing TLS related SDP defined which represents TLS client/server semantic. [IETF RFC 4572] intentionally delegates such control information up to the application (layer).

R-8.2/3: The MG is required to select the TLS client or TLS server role based on the MGC request.

R-8.2/4: The MGC is required to enforce the TLS client role to the MG in case of correspondent security policies.

NOTE 2 – This requirement is related to threat scenarios where a “TLS client” is trying to attack TLS server. Such a security risk could be mitigated by enforcing a TLS session establishment in direction from the MG to the remote bearer endpoint.

[End Proposed Correction]

____________________