| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4417 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: additional TLS extended session control package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates by consideration of an additional package for TLS session control.

1	Introduction

Contributions C-4 and C-14 from last meeting (2013-01) resulted in an agreement in drafting individual H.248 packages for TLS bearer control. Similar as H.248.TCP, H.248 control for bearer connection establishment and release is divided in two packages: a base package for basic control and an extension package for enhanced bearer connection control.

Copy of COM 16 – C 14/Figure 1 – H.248.TLS – Package redesign proposal – Overview

This contribution continues the update process by a new TLS session extended session control package.

2	Discussion

The tlsesc package defines two additional properties concerning the interlinking of

- “horizontal” behaviour related to a pair of SEPs (i.e., a SEPP)

  - a “TLS-based SEPP” (similar as the “TCP-based SEPP interlinkage” by tcpecc/sepplink);

  - TLS level (MG internal) control information may impact the partner SEP

and

- “vertical” behaviour related to a pair of protocol layers (here: TLS and underlying IP transport layer)

  - which allows to support tightly coupled and decoupled bearer control protocol procedures (“we’ve to support both use case categories!”).

For discussion and description see proposed changes.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

References & abbreviations needs to be updated according agreed changes.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 9	TLS extended session control package

|  | Package Name: | TLS extended session control package |
| --- | --- | --- |
|  | Package ID: | tlsesc (0x####) |
|  | Description: | This extension package augments the base package behaviour by additional support of - TLS bearer session control behaviour across a “horizontal” Stream Endpoint Pair (SEPP), rather instead of SEP-only scope; and - TLS/L4 bearer control behaviour across “vertical” protocol layers, rather instead of decoupled, individual control of TLS security sessions and their underlying IP transport connection. |
|  | Version: | 1 |
|  | Extends: | tlsbsc |

## 9.1	Properties

### 9.1.1	Stream Endpoint Pair Interlinkage

|  | Property Name: | Stream Endpoint Pair Interlinkage |
| --- | --- | --- |
|  | Property ID: | sepplink (0x0005?) |
|  | Description: | The semantical scope of this property relates to a Stream Endpoint Pair (SEPP; see definition in (clause 3.2.11) [ITU-T H.248.1]). This property defines if a detected condition leading to the establishment or release of the TLS security session will trigger an MG-autonomous (see clause 3.2.2/H.248.TCP) establishment or release of the TLS security session of the partner SEP as well. Only TLS protocol control information is part of such conditions. Normal TLS data traffic is not affected by this property. Semantical scope limitations: - only the two phases of TLS bearer session establishment and release; - only incoming direction; - only the first stimuli of each TLS bearer session control procedure (i.e., first incoming (TLS) ClientHello; first incoming (TLS) close_notify); - SEP-related properties, i.e., there are four value combinations (per Property) from SEPP perspective (in order to address network asymmetries) |
|  | Type: | sublist of Boolean (for separated support of establishment and release phase) |
|  | Possible values: | ## Table X defines the protocol semantic per value and dependent on connection control phase. See model in Figure 5 concerning the used SEP names. Table X – Semantic of property sepplink Value; MG semantic for TLS bearer session. Establishment; Release. True; a) Incoming: An incoming TLS bearer session establishment request at SEP T1(S1) shall lead to an autonomous outgoing TLS bearer session establishment request at the partner SEP T2(S1). b) Outgoing: An MGC-initiated outgoing TLS bearer session establishment at SEP T1(S1), shall not impact the TLS session state of SEP T2(S1).; a) Incoming: An incoming TLS bearer session release request at SEP T1(S1) shall lead to an autonomous outgoing TLS bearer session release request at the partner SEP T2(S1). b) Outgoing: Any action (e.g. by MGC) in closing the outgoing TLS bearer session at SEP T1(S1), shall not impact the TLS session state of SEP T2(S1).. False; The state of TLS bearer session endpoint SEP T1(S1) shall not impact the state of TLS bearer session endpoint SEP T2(S1) (and vice versa).. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tlsbsc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”): TBD

}

### 9.1.2	Protocol Layers Interlinkage

|  | Property Name: | Protocol Layers Interlinkage |
| --- | --- | --- |
|  | Property ID: | plslink (0x0006?) |
|  | Description: | The semantical scope of this property relates to a single Stream Endpoint (SEP) with regards to a portion of the bearer specific protocol stack. This property defines whether bearer control procedures of the TLS security session shall or shall not impact as well the bearer control procedures of the underlying IP transport connection (e.g., TCP, SCTP). This property serves the two fundamental use cases of - Tightly coupled mode: the holding time of the TLS security session correlates directly with the underlying IP transport connection; e.g. a TLS session termination will always imply the release of the TCP or SCTP transport connection; - Decoupled mode: the TLS security session is disjoint from bearer control procedures of the underlying IP transport connection. For instance: a TCP-based communication service with alternate phases of encrypted and clear data, a TLS/TCP (or a DTLS/DCCP) session where the loss of the security session should not lead to the stop of data transfer, DTLS with UDP, DTLS-SRTP, different aimed behaviour between TLS/SCTP and DTLS/SCTP, etc. The default behaviour is aligned with the base package tlsbsc, which is the decoupled mode. The property takes only effect in case of connection-oriented IP transport protocols (such as TCP, SCTP, DCCP). The property shall be ignored in case of connectionless IP transport protocols (UDP). ## ToBeChecked |
|  | Type: | sublist of Boolean (for separated support of establishment and release phases) |
|  | Possible values: | Table X defines the protocol semantic per value and dependent on connection control phase. Table X – Semantic of property plslink Value; MG semantic for protocol stack “TLS-over-L4”. Establishment; Release. True; a) Incoming: An incoming L4 transport connection establishment shall trigger the establishment of TLS security session, as soon as the L4 transport connection is successfully established. The SEP shall then sent an outgoing TLS bearer session establishment request (i.e., a TLS ClientHello). b) Outgoing: An outgoing L4 transport connection establishment shall trigger the establishment of TLS security session, as soon as the L4 transport connection is successfully established. See above.; a) Incoming: An incoming TLS bearer session release request shall trigger the outgoing L4 transport connection release request, as soon as the TLS security session is successfully released.. b) Outgoing: An outging TLS bearer session release request shall … see above.. False; Any bearer control procedures related to TLS security sessions shall not impact the underlying L4 transport connection (and vice versa).. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tlsbsc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”): TBD

}

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.3	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

### 9.6.1	TLS security session establishment

#### 9.6.1.1	Stream endpoint pair interlinkage disabled

If the property sepplink is set to False, then the incoming TLS security session establishment request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.1.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TLS security session establishment request shall be propagated to the partner SEP, triggering there an outgoing TLS ClientHello message.

#### 9.6.1.3	Protocol layers interlinkage disabled

If the property plslink is set to False, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.1.4	Protocol layers interlinkage enabled

If the property plslink is set to True, then an incoming or outgoing (NOTE 2) L4 transport connection establishment request (such as a TCP SYN packet) shall immediately trigger an outgoing TLS security session establishment request, after a successfully established L4 bearer.

NOTE 1 – Appendix V?.4.2.1.1 provides a signalling example.

NOTE 2 – E.g., either MGC driven or in case of e.g. TLS-over-TCP might be also the [ITU-T H.248.TCP] property tcpbsc/sepplink in use.

### 9.6.2	TLS security session release

#### 9.6.2.1	Stream endpoint pair interlinkage disabled

If the property sepplink is set to False, then the incoming TLS security session release request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TLS security session release request shall be propagated to the partner SEP, triggering there an outgoing TLS closure_notify alert message.

#### 9.6.2.3	Protocol layers interlinkage disabled

If the property plslink is set to False, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.4	Protocol layers interlinkage enabled

If the property plslink is set to True, then an incoming or outgoing TLS security session release request shall immediately trigger an outgoing L4 bearer connection release request, after a successfully release TLS session.

NOTE 1 – Appendix V?.4.2.2 provides signalling examples.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix V?

Illustration of protocol semantics 
of the TLS extended session control package

(This appendix does not form an integral part of this Recommendation.)

## V?.1	Overview

The tlsesc package (clause 9) defines an augmented TLS security session control on top of the base package (tlsbsc, clause 8). The effect of each individual package property is illustrated in clauses V?.3 (property tlsesc/sepplink) and V?.4 (property tlsesc/plslink). Clause V?.5 describes examples of combined usage of both properties.

The illustrated use cases are only examples. An exhaustive consideration of all possible combinations is out of scope.

## V?.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS and L4 layers) are highlighted besides the H.248 interface.

Furthermore some Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS/L4 SEP.

## V?.3	Usage of SEPP Interlinkage property

Property tlsesc/sepplink is defined in clause 9.1.1. The property affects TLS security session establishment and release procedures. The semantics are correspondent to the TCP layer. See e.g. the illustration and examples in [ITU-T H.248.TCP], Appendix ?.3.

## V?.4	Usage of Protocol Layers Interlinkage property

Property tlsesc/plslink is defined in clause 9.1.2. The property impact is depicted in below two illustrations. Figure V?.1 recalls again the default mode according base package (tlsbsc):

Figure V?.1 – Illustration of Protocol Layers Interlinkage property 
– Property „plslink = FALSE“

The MGC is responsible for individual control of layer-specific bearer control procedures.

Figure V?.2 illustrates the tightly coupled mode, which is conditional to the aimed communication service, but when applicable, it provides some benefits such as minimal MGC involvement and performance optimized bearer establishment and/or release.

Figure V?.2 – Illustration of Protocol Layers Interlinkage property 
– Property „plslink = TRUE“

### V?.4.1	Property „plslink = FALSE“

Semantic FALSE implies base package behaviour.

### V?.4.2	Property „plslink = TRUE“

The property may be applied for the establishment or release phase or both.

### V?.4.2.1	Bearer establishment phase

#### V?.4.2.1.1	Incoming direction

Figure V?.3 illustrates the normal case: the start of any TLS security session establishment precedes a successfully established IP transport connection. Thus, the MG receives an incoming L4 Est.req (2). When L4 is established, then immediately an outgoing TLS session establishment (4) is initiated by the MG.

Figure V?.3 – Property „plslink = TRUE“ – L4 incoming bearer establishment request

The actually impossible case of TLS establishment request first, without yet successfully established IP transport, is indicated Figure V?.4:

Figure V?.4 – Property „plslink = TRUE“ – TLS incoming bearer establishment request

Theoretical example:

E.g., case of TLS/TCP bearer: the very first incoming TCP packet (with set SYN flag) would contain already a TLS ClientHello as TCP payload, but the local TCP connection state is still not Established!

##### V?.4.2.1.2	Outgoing direction

SEP T1(S1) is triggered for an outgoing L4 bearer establishment procedure (see Figure V?.5). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent sepplink settings). After successful IP transport connection establishment (3, 4), the MG initiates immediately an outgoing TLS session establishment (5).

Figure V?.5 – Property „plslink = TRUE“ – L4 stimuli

Figure V?.5 shows also the optional event notifications related to bearer specific state transitioning (here: L4 and TLS reporting).

There might be also a TLS-related stimuli first from SEP perspective (without a prior explicit L4 trigger). The scenario is almost identical as the previous case, see Figure V?.6:

Figure V?.6 – Property „plslink = TRUE“ – TLS stimuli

Example:

In case of TLS/TCP with SEPP interlinkage usage, and property settings 
a) TCP layer: tcpecc/sepplink = FALSE and  
b) TLS layer: tlsesc/sepplink = TRUE, 
then the SEP would receive a “TLS level stimuli” from his partner SEP.

#### V?.4.2.2	Bearer release phase

##### V?.4.2.2.1	Incoming direction

Figure V?.7 illustrates the abnormal case: the start of any IP transport connection release precedes a successfully released TLS security session. The MG receives an incoming L4 Rel.req (2), but would suspend L4 release procedural steps in favour of a TLS session release (3) first. The L4 release procedure is resumed again (5) after successful TLS termination (4).

Figure V?.7 – Property „plslink = TRUE“ – L4 incoming bearer release request

The standard case of TLS release request first is depicted in Figure V?.8: the TLS session release (2, 3) triggers the subsequent IP transport connection release (4, 6).

Figure V?.8 – Property „plslink = TRUE“ – TLS incoming bearer release request

##### V?.4.2.2.2	Outgoing direction

SEP T1(S1) is triggered for an outgoing L4 bearer release procedure (see Figure V?.9). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent sepplink settings). After successful TLS session release (3, 4), the MG initiates immediately an outgoing IP transport connection release (5).

Figure V?.9 – Property „plslink = TRUE“ – L4 stimuli

Figure V?.10 outlines the case of a TLS-related stimuli:

Figure V?.10 – Property „plslink = TRUE“ – TLS stimuli

[End Proposed Correction]