| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4420 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Update proposals for clauses 3 to 5 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

Clauses 3 to 5 would need to be updated according parallel contributions for progressin work item H.248.TLS

AVD references

Updates clause 3:

- AVDs-4400/4417: IP transport connection for TLS sessions => terminology updates in 3.1

- AVD-4401: “TLS profile” related aspects => terminology updates in 3.2

Updates clause 4:

- alignment of abbreviations …

Updates clause 5:

- AVDs-4417: notations used in new signalling flows

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	TLS-client [IETF RFC 5246]: the TLS entity that initiates a TLS security sesson to a TLS server.  The primary operational difference between the TLS server and TLS client is that the server is generally authenticated, while the TLS client is only optionally authenticated.

3.1.2	TLS-server [IETF RFC 5246]: the TLS entity that responds to requests for security sessons from TLS clients.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	MG TLS profile: The TLS profile supported by the MG.

3.2.2	TLS Domain: A set of entities in a network domain which use the same set of TLS related parameters, procedures and resources.

3.2.3	TLS domain profile: The TLS profile defined for a TLS domain.

3.2.4	TLS endpoint: Either a TLS-client or a TLS-server.

3.2.5	TLS endpoint profile: The TLS profile used by a TLS endpoint for the setup of a TLS session.

3.2.6	TLS profile: A selection of options from a set of TLS related protocol parameters and procedures.

NOTE – Also known as (nonstandard term) TLS protocol profile (example: the TLS profile defined by Annex E/[ETSI TS 133.310]).

{Editor’s note (2012-09): the definition proposals in clause 3.2 are still under discussion .}

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2B | Back to Back |
| --- | --- |
| B2BIH | Back to Back IP Host (see also [b-ITU-T H.248.64]) |
| B2BTE | Back-to-Back TCP Endpoint |
| B2BUA | Back to Back User Agent (SIP) |
| BGF | Border Gateway Function |
| FTP | File Transfer Protocol |
| HTTP | Hypertext Transfer Protocol |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 5	Conventions

## 5.1	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| L4 Est.req L4 Est.ack | Abstracted (protocol-independent) indication for establishment requests/acknowledgements of new connection-oriented IP transport connections. |
| --- | --- |
| L4 Rel.req L4 Rel.ack | Ditto for release. |
| TLS Est.req TLS Est.ack | Abstracted (TLS message/procedure independent) indication for establishment requests/acknowledgements of new TLS security sessions. |
| TLS Rel.req TLS Rel.ack | Ditto for release. |

[End Proposed Correction]