| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4421 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Clause 12 – Update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes initial input to clause 12.

1	Introduction

The purpose of clause 12 is to describe applicability and non-applicability of package-less TCP control options.

The underlying idea is to reuse SDP whenever possible. Usage of SDP is typically impossible when either SDP is missing or when SDP is already semantically overloaded.

NOTE: AVD-4404 = correspondent contribution for H.248.TLS.

2	Discussion

Figure 1 makes an attempt in indicating all possible control steps (from H.248 perspective) concerning the establishment of IP transport connections in environments demanding for NAT traversal support. Note: we could equate “L4 type” with “TCP” in scope of H.248.TCP:

Figure 1 – Principal H.248 control steps for establishing a native TCP bearer

It is apparent that some steps might be combined and that specific steps might be optional.

Table 1 provides a preliminary inventory of possible control elements from H.248 perspective. The column “H.248/SDP element” refers to the discussed “package-less TCP control”.

Table 1 – H.248 control steps and possible control elements

| H.248 control step: |  | H.248 property: | H.248/SDP element: | Comments: |
| --- | --- | --- | --- | --- |
| Establishment phase: „IP transport connection establishment“ |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | “m=”-line <proto> (clause 13.4 / H.248.84, H.248.69) | SDP may be ambiguous due to “protocol stack” concept. 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property: tcpbcc/bceb | “a=setup:holdconn” (?) |  |
| 4 | Start IP Transport Connection Establishment Procedure („TCP Open“) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to H.248.84 “a=setup:” semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | H.248.37 | “a=setup:” for H.248.84 NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State „EstablishedTCP“) | Event: tcpbcc/BNCChange | — | MGC notification might be optional. |
| 7 | Local L4+ NAT traversal support | H.248.78 | e.g., “a=msrp-cema” (?) | NOTE 1 |
| Communication phase: |  |  |  |  |
| 8 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP based indication seems to be principally deficient (“application protocol specific SDP”), whereas H.248.78 supports the flexibility of different application protocols. |  |  |  |  |

3	Conclusions

The discussion about (non-)applicability of SDP is only relevant for steps 2, 3, 4, 5 and 7. The SDP method for step 2 may work for native TCP bearers (e.g., non-TLS scenario). Most critical are steps 3, 4 and 5 due to the single SDP attribute referred (“semantically overloaded”), which implies possible interactions between these control steps.

Steps 3 to 5 in more detail, dependent on TCP mode:

- TCP relay mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP merge mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP proxy mode: steps 3 to 5 see above Table 1.

4	Proposal

It is suggested to add above preliminary analysis to clause 12.

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

References & abbreviations needs to be updated according agreed changes.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 12	Package-less TCP control

{Editor’s note (2013-01 meeting): dedicated chapter on SDP based control required, addressing }

Purpose of this clause is to describe the SDP-based semantics for TCP control, which needs the consideration of

- SDP usage in H.248.69, H.248.84

- how the package-based elements/procedures would be realized via SDP?

- which functionality would be out of scope of SDP control?

- assumption of mixed Property- & SDP-based control

First comments (from C-11):

To requirement of “Bearer type indication”:

- still open (explicit via dedicated property or SDP? or implicit via using one or multiple of the TCP packages)

To requirement of “Originating/Terminating Bearer control protocol procedures”:

Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

SDP-based indication of “TCP” (according clause 13.4/H.248.84) to a stream or termination, - either in LD or RD or both -, represents 
a) an implicit TCP client role assignment and
b) a request for immediate start of TCP connection establishment procedure.
Any possible race condition of an incoming TCP connection establishment request (from remote TCP entity) at the MG bearer interface will be resolved as TCP simultaneous open procedure.

Option 2:

SDP-based indication of “TCP” (according clause 13.4/H.248.84) to a stream or termination, but consideration of LD and RD scope with following logic:
Condition 1: IF LD only THEN “TCP server” role of stream endpoint/termination; Wait on incoming TCP connection establishment request (from remote TCP entity);
Condition 2: IF RD only THEN “TCP client” role of stream endpoint/termination; proceed as in option 1;
Condition 3: IF LD AND RD THEN EITHER “Hold connection establishment” OR “error case” OR “…”.

To requirement of “Originating/Terminating Bearer control protocol procedures”:

[other package elements]

Contributions are solicited.}

## 12.1	Overview – principal H.248 control steps

{Editor’s note (2013-06 meeting): the text is this clause is still subject of discussion … }

Figure x1 makes an attempt in indicating all possible control steps (from H.248 perspective) concerning the establishment of TCP connections in environments demanding for NAT traversal support:

Figure x1 – Principal H.248 control steps for establishing a native TCP bearer

It is apparent that some steps might be combined (e.g., 1 and 2) and that specific steps might be optional (e.g., 3, 5, 7).

Table y1 provides a preliminary inventory of possible control elements from H.248 perspective. The column “H.248/SDP element” refers to “package-less TCP control”.

Table y1 – H.248 control steps and possible control elements

| H.248 control step: |  | H.248 property: | H.248/SDP element: | Comments: |
| --- | --- | --- | --- | --- |
| Establishment phase: „IP transport connection establishment“ |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | “m=”-line <proto> (clause 13.4 / H.248.84, H.248.69) | SDP may be ambiguous due to “protocol stack” concept. 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property: tcpbcc/bceb | “a=setup:holdconn” (?) |  |
| 4 | Start IP Transport Connection Establishment Procedure („TCP Open“) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to H.248.84 “a=setup:” semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | H.248.37 | “a=setup:” for H.248.84 NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State „EstablishedTCP“) | Event: tcpbcc/BNCChange | — | MGC notification might be optional. |
| 7 | Local L4+ NAT traversal support | H.248.78 | e.g., “a=msrp-cema” (?) | NOTE 1 |
| Communication phase: |  |  |  |  |
| 8 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP based indication seems to be principally deficient (“application protocol specific SDP”), whereas H.248.78 supports the flexibility of different application protocols. |  |  |  |  |

{Editor’s note (2013-06 meeting): possible conclusions

The discussion about (non-)applicability of SDP is only relevant for steps 2, 3, 4, 5 and 7. The SDP method for step 2 may work for native TCP bearers (e.g., non-TLS scenario). Most critical are steps 3, 4 and 5 due to the single SDP attribute referred (“semantically overloaded”), which implies possible interactions between these control steps.

Steps 3 to 5 in more detail, dependent on TCP mode:

- TCP relay mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP merge mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP proxy mode: steps 3 to 5 see above Table y1.

}

## 12.2	Step 1: TCP endpoint creation

Not SDP related, based on H.248 Command request.

## 12.3	Step 2: Bearer type indication “TCP”

According the SDP-based method as defined by clause 9.6.1.

Conditional approach, dependent on:

C1: unambiguous indication of transport protocol “TCP” by protocol stack based SDP element “m=”-line <proto>.

C2: consistent LD/RD signalling (due to the fact of a LCD level characteristic)

## 12.4	Step 3: Block / Unblock TCP bearer control procedures at SEP

FIXTHIS

## 12.5	Step 4: Start TCP connection establishment procedure („TCP Open“)

FIXTHIS

## 12.6	Step 5: Local L4/L3 NAT traversal support

FIXTHIS

## 12.7	Step 6: Reporting successfully established TCP connection

FIXTHIS

## 12.8	Step 7: Local L4+ NAT traversal support

FIXTHIS

## 12.9	Step 8: Active communication phase

Control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TCP) are out of scope of the SDP.

[End Proposed Correction]

____________________