| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4425 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Appendix III – Update proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to Appendix III on TLS profile information.

Update proposals:

- It was noted that Appendix III.4 is not fully correct because there are already IETF-defined TLS profiles (e.g., RFC 5430, which was obsoleted in the meanwhile by RFC 6460).

  - The RFC 6460 defined TLS profile matches the concept of a TLS domain profile of H.248.TLS.

  - An MG (or H.248 profile) with an H.248 TLS-enabled stream would have to steer the TLS negotiation phase according that IETF TLS profile.

  - Proposed to update Appendix III (-> III.4 and new III.5).

- NGN security provides also TLS domain profile definitions, see clause 9.1/Y.2704.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix III

Example TLS profiles

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some exemplary TLS profiles.

## III.1	Typical Internet TLS profile

Table III.1.a – Example of a TLS domain profile – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 1.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 1.4 | Extensions: | None |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 TLS 1.2 | - | - | - |
| 2.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | None |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | - |  |  |  |
| 3.2 | Client authentication required | Yes |  |  |  |
| 3.3 | Certificate types (client): | signed, self-signed |  |  |  |
| 3.4 | Renegotiation: | Yes |  |  |  |
| 3.5 | Resumption: | No |  |  |  |
| 3.6 | Extensions: | - |  |  |  |
| 3.7 | Capability negotiation: | Yes |  |  |  |

Table III.1.b – Example of a TLS domain profile – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | - | - |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | - |
|  | SHOULD NOT | - | - |

## III.2	3GPP TLS domain profile

Network domain security is defined in [ETSI TS 133.310] with the normative TLS domain profile in Annex O. Table III.2 summarizes this profile with status of 3GPP Release 11 (2012-12).

Table III.2.a – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | SSL 3.0 | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 1.3 | Compression methods: | Compression Method.null |  | methods of [IETF RFC 3749] |  |
| 1.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | see client | see client | see client | - |
| 2.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | see client |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | Client authentication required | ? |  |  |  |
| 3.3 | Certificate types (client): | ? |  |  |  |
| 3.4 | Renegotiation: | NOTE 2 |  |  |  |
| 3.5 | Resumption: | NOTE 2 |  |  |  |
| 3.6 | Extensions: |  |  |  |  |
| 3.7 | Capability negotiation: | YES (NOTE 1) |  |  |  |
| NOTE 1 – Target: negotiation up to highest TLS protocol version. NOTE 2 – Not explicitly specified, thus TLS V1.1 [IETF RFC 4346] applies: YES |  |  |  |  |  |

Table III.2.b – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA (NOTE 1) TLS_RSA_WITH_3DES_EDE_CBC_SHA (NOTE 2) TLS_RSA_WITH_NULL_SHA | see client |
|  | SHALL NOT | a) Cipher suites with NULL integrity protection or HASH b) “TLS_DH_anon_WITH_” (NOTE 3) | see client |
| O | SHOULD | TLS_RSA_WITH_AES_128_CBC_SHA256 TLS_RSA_WITH_NULL_SHA256 | see client |
|  | SHOULD NOT | Cipher suites with RC4 | see client |
| NOTE 1 – The only mandatory cipher suite from TLS V1.2 [IETF RFC 5246]. NOTE 2 – The only mandatory cipher suite from TLS V1.1 [IETF RFC 4346]. NOTE 3 – Because the “the key exchange method shall not be anonymous”. |  |  |  |

{Editor’s note (2013-01): Preferred cipher suites (ordered list): negotiation aspect?}

## III.3	OMA TLS domain profiles

The Open Mobile Alliance (OMA) defines two TLS domain profiles [OMA-TS-TLS-V1_0-20080902-A], a “public key TLS domain profile” and a “pre-shared key TLS domain profile”.

FIXTHIS

### III.3.1	OMA public key TLS domain profile

Table III.3.a – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 1.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 1.3 | Compression methods: | methods of [IETF RFC 3749] |  |  |  |
| 1.4 | Extensions: |  |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 2.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 2.3 | Compression methods: | methods of [IETF RFC 3749] |  |  | - |
| 2.4 | Extensions: |  |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | (Server ¦ Client) authentication required | YES (Server authentication both) |  | YES (Client authentication by server) |  |
| 3.3 | Certificate types (client): |  |  |  |  |
| 3.4 | Renegotiation: |  |  |  |  |
| 3.5 | Resumption: | YES (NOTE z1) |  |  |  |
| 3.6 | Extensions: | [IETF RFC 4366] |  | TLS tunneling |  |
| 3.7 | Capability negotiation: |  |  |  |  |
| NOTE 1 – #. NOTE z1 – The longer session life (e.g., 12 hours) SHOULD be used. |  |  |  |  |  |

Table III.3.b – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_3DES_EDE_CBC_SHA TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | None specified. | see client |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | None specified. |
|  | SHOULD NOT | All others | see client |
| NOTE 1 – # |  |  |  |

### III.3.2	OMA pre-shared key TLS domain profile

see [OMA-TS-TLS-V1_0-20080902-A].

## III.4	IETF Minimum TLS domain profile

The IETF noted the value of TLS protocol profiling. The IETF provides guidelines and decision criteria for TLS profiling, see [b-IETF tls-minimal].

## III.5	IETF example of a national TLS domain profile

[b-IETF RFC 6460] represents an example of a national TLS domain profile. Inter alia, TLS protocol version and ciper suites are profiled.

## III.6	ITU-T TLS domain profile for NGN signalling and management plane

[b-ITU-T Y.2704], clause 9.1, defines a TLS domain profile, recommended for signalling and management interfaces in NGNs.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Bibliography

[b-ITU-T Y.2704]	Recommendation ITU-T Y.2704 (01/2010), Security mechanisms and procedures for NGN

[b-IETF rtcweb-data]	IETF draft-ietf-rtcweb-data-channel (2012), RTCWeb Datagram Connection.
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-data-channel/

[b-IETF RFC 6460]	IETF RFC 6460 (2012), Suite B Profile for Transport Layer Security (TLS).

[b-IETF tls-minimal]	IETF draft-tschofenig-lwig-tls-minimal (2012), A Minimal (Datagram) Transport Layer Security Implementation.
URL: http://tools.ietf.org/html/draft-tschofenig-lwig-tls-minimal-01

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[End Proposed Correction]

____________________