| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4427 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June, 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.53 IG: Semantical clarification of traffic components as part of traffic policing |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution discusses a pending question concerning relevant traffic components of an IP flow as part (or not) of traffic policing.

1	Background

H.248.53 itself is not specific on this question, e.g. when considering IP bearer traffic policing (clause 9.4/H.248.53 and applied packages from clauses 7 and 8).

Following basic issue was observed: A single or multiple IP flow(s) associated to an H.248 stream/termination could be basically divided in two IP traffic portions:

- IP application data traffic related to the communication application itself, i.e., with end-to-end significance; and

- IP-based control traffic along the bearer path, with purpose of NAT traversal, authentication procedures, bearer control protocol for (segmented) IP transport connections (e.g., TCP segment), bearer control protocol for (segmented) IP security sessions, etc., i.e., without end-to-end significance (rather type hop-to-hop).

Both traffic portions provide following difference from H.248 Context point of view: the e2e traffic (1) is going through the Context, and traffic (2) is terminated by a SEP.

That’s an important distinction concerning traffic policing, which has originally only e2e traffic (1) in scope, but not (2) (in our understanding).

2	Discussion

Such a semantic could be justified by following reasons:

- (IP bearer) traffic policing relates to a specific traffic control function (§ 9.1/Y.1221): traffic parameter control (list item (iii)). Concerned traffic parameters are subject of a traffic contract (§ 3.2.9/Y.1221) with scope on the (IP flow) traffic descriptor (§ 3.2.10/Y.1221).
The traffic descriptor again is defined for an interface, which is the UNI in case of usage parameter control and the e2e traffic sent by the UE towards the remote UE.

- The application control instance itself, which is responsible for specifying the traffic descriptor, does normally only know the e2e traffic components. E.g., is not aware about the lower layer, hop-by-hop related bearer path procedures.

- Activity phase: e2e traffic component (1) is lasting as long as the entire call holding time whereas control traffic component (2) is typically only part of the establishment phase of the communication path. Hence, a pretty timely limited activity phase at the beginning of a call. Such traffic could be considered as a single, short burst. The traffic volume as such is also normally fairly low.
Thus, there shouldn’t be a specific requirements for traffic policing of component (2).

Concrete examples of type (2) traffic, those packets are terminated by the H.248 SEP and hence are not forwarded into the H.248 call context:

- received STUN messages (regardless of whether or not successfully authenticated and integrity checked)

- received DTLS handshake messages

- received SRTP or SRTCP packets which do not pass the packet authentication or replay checks

- received TCP pure control messages if in TCP endpoint mode

- received TLS handshake messages

- others

3	Conclusion

We’d like to suggest to clarify the package semantics in the direction that only end-to-end traffic is subject of a traffic policing function, but not traffic terminated locally in a SEP-

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against Recommendation H.248.53 (03/2009).

Change proposal #1:

[Begin Proposed Correction]

# 6	Traffic management package

Package name:		Traffic management package

Package ID:		tman (0x008d)

Description:		This package allows traffic descriptors to be defined for a stream and allows policing to be explicitly enabled. Version 1 is defined for a single traffic flow per H.248 stream. Version 2 supports multiple traffic flows per H.248 stream.

Only traffic flow components are subject of traffic rate policing which are routed through the context (i.e., with end-to-end significance). Ingress traffic, which is terminated in the H.248 SEP/termination, is excluded from traffic rate policing.

Version:			2

Extends:			None.

## 6.1	Properties

### 6.1.1	Peak data rate

Property name:	Peak data rate

Property ID: 		pdr (0x0001)

Description: 		This property defines the peak data rate in bytes per second that is permitted for the stream.

Type: 				Sub-list of Integer

The type represents a list of one or more integer values. A list with one item only relates to traffic policing on a stream level, a list with multiple items relates to traffic policing on a flow level. Flow level policing is only applicable in case of multi-flow stream structures (see also clause 6.6). The size of the list must be equal for all tman properties applied to the same stream.

•	Association between the flow component and the list item: According to the order of appearance (see clause 6.6.2.1) within the signalled H.248 media descriptor (e.g., a voice media group may appear before a T.38 media group (or vice versa); or an RTP media flow may appear before its associated RTCP control flow). Clause 6.6.2.1 provides detailed information concerning the flow-to-list position relationship.

The MG should reply with error code "473 – Conflicting Property Values" when the number of flows on the stream does not match the sub-list length.

Possible values:	Any integer, –1 and up.

•	Non-policed traffic flows: There might be particular flows in a multi-flow stream structure without peak data rate policing. Any such flow shall be indicated by using a value of "–1".

Default: 			Provisioned (Note 1).

Defined in: 			Local Control

Characteristics: 	Read/write.

### 6.1.2	Sustainable data rate

Property name:	Sustainable data rate

Property ID:		sdr (0x0002)

Description:		This property defines the sustainable data rate in bytes per second that is permitted for the stream.

Type: 				Sub-list of Integer

List syntax and semantic according to clause 6.1.1.

Possible values:	Any integer, –1 and up.

•	Non-policed traffic flows: There might be particular flows in a multi-flow stream structure without sustainable data rate policing. Any such flow shall be indicated by using a value of "–1".

Default: 			Provisioned (Note 1).

Defined in: 			Local Control

Characteristics: 	Read/write.

### 6.1.3	Maximum burst size

Property name:	Maximum burst size

Property ID: 		mbs (0x0003)

Description: 		This property defines the maximum burst size in bytes for the stream.

Type: 				Sub-list of Integer

List syntax and semantic according to clause 6.1.1.

Possible values:	Any non-negative integer.

Default: 			Provisioned (Note 1).

Defined in: 			Local Control

Characteristics: 	Read/write.

### 6.1.4	Delay variation tolerance

Property name:	Delay variation tolerance

Property ID: 		dvt (0x0004)

Description: 		This property defines the delay variation tolerance for the stream in tens of microseconds (i.e., setting tman/dvt to "x" indicates a delay variation tolerance of x10–5 seconds).

Type: 				Sub-list of Integer

List syntax and semantic according to clause 6.1.1.

Possible values:	Any non-negative integer.

Default: 			Provisioned (Note 1).

Defined in: 			Local Control

Characteristics: 	Read/write.

### 6.1.5	Policing

Property name:	Policing

Property ID: 		pol (0x0005)

Description:		If set to true policing is to be applied at the termination for traffic entering the MG.

Type: 				Sub-list of Boolean

List syntax and semantic according to clause 6.1.1.

Possible values:	On/off

Default: 			Provisioned (Note 2).

Defined in: 			Local Control

Characteristics: 	Read/write.

NOTE 1 – Default values may be provided via configuration management or derived from other property values (e.g., information elements in the local descriptor).

NOTE 2 – The original tman v1 definition (see [b-ETSI TS 102 333]) did not provide any default value and is ambiguous (from the procedural text) concerning default behaviour for traffic policing. The specification of "provisioned" allows to specify a particular default behaviour (i.e., enable policing or disable policing) per H.248 control association (e.g., by a profile specification).

## 6.2	Events

None.

## 6.3	Signals

None.

## 6.4	Statistics

None.

## 6.5	Error codes

None.

## 6.6	Procedures

### 6.6.1	Traffic policing in general

The principles for traffic policing are identical to package tman version 1 (see clause II.1).

### 6.6.2	Single-flow versus multi-flow stream structures

Every H.248 stream may carry one or more media groups. Every media group itself may consist of one or more traffic flows, e.g., a media flow plus a control flow. Complex multi-flow stream structures are thus possible, containing any number of flows from one up. It should be noted that all individual flows of a multi-flow stream structure may be considered as a single aggregated flow.

#### 6.6.2.1	Multi-flow stream structures: Unambiguous mapping between flow and list position

There is a media description for every flow in the H.248 Media Descriptor. Only the media descriptions of the Local Descriptor are relevant for traffic policing (see Figure 1). Figure 1 indicates the binding between list positions of tman property values and the corresponding flows of the Local Descriptor.

Figure 1 – Multi-flow stream structures: 
Unambiguous mapping between flow and list position

The number and order of flows within an LD is related to the occurrences

•	of the media group description(s) within an LD,

•	of the traffic flow descriptions within a single media group, and

•	mutual agreements between MGC and MG by H.248 profile specifications in case of implicit flow components (which is e.g., often the case of RTCP control flows).

The structure of a multi-flow local descriptor is determined after the successful stream establishment phase, and this structure shall be unchanged for the entire lifetime of this stream. Such a static structure shall be a precondition for multi-flow-per-stream traffic policing.

### 6.6.3	Traffic granularity – Termination versus stream versus flow-level policing

#### 6.6.3.1	Byte-level

The minimum quantity of traffic policing is a "byte" (due to the used unit in the properties of this Recommendation), leading to:

•	the tman-based policing of the byterate of traffic; and

•	the pacs-based policing of the bytesize of a protocol data unit.

Both byte-level policers may be combined or individually applied. Byterate policing may be enforced on different traffic structures, according to Tables 1 and 2.

#### 6.6.3.2	Stream- versus flow-level byterate policing

Table 1 summarizes the byterate policing capabilities on stream versus flow level.

Table 1 – Granularity of byterate policing – Stream- versus flow-level

| Number of flows per stream | Byterate policing on |  |
| --- | --- | --- |
|  | Flow-level | Stream-level |
| One flow | Applicable (with tman v1 and tman v2) | Applicable (with tman v1 and tman v2) |
| Multiple flows | Applicable (via tman v2) | Applicable (Note) (with tman v1 or tman v2 with single item lists) |
| NOTE – All individual flows will be handled as a single aggregated flow by the byterate policer. |  |  |

#### 6.6.3.3	Termination versus stream-level byterate policing

Table 2 summarizes the byterate policing capabilities on termination versus stream level.

Table 2 – Granularity of byterate policing: Termination- versus stream-level

| Number of streams per termination | Byterate policing on |  |
| --- | --- | --- |
|  | Stream-level | Termination-level |
| One stream | Applicable | "Applicable" (Note 1) |
| Multiple streams | Applicable | Not applicable with either tman v1 or v2 (Note 2) |
| NOTE 1 – There is a 1:1 ratio between a stream and termination. The stream-level policer is thus effectively also a termination-level policer. NOTE 2 – This is due to the definition of the tman properties: pdr, sdr, mbs and dvt on the stream level only. Any multi-stream-per-termination structure requires therefore individual traffic policer settings. |  |  |

### 6.6.4	Recommendations for flow versus stream-level traffic policing

Stream-level policing is typically applied:

•	for single-flow stream structures (this is e.g., often the general case for transport protocol agnostic or even media-agnostic stream configurations);

•	for multiple-flow stream structures:

–	when the MGC is not able to derive flow level policing parameters;

–	when the individual traffic characteristics of one or more flows are unknown;

–	when the individual traffic characteristics of each flow are similar or even identical, i.e., allowing the estimation or calculation of policer parameters for the traffic aggregate;

–	when the individual traffic characteristics on flow level are not interesting;

–	when a worst-case approach is followed (e.g., the "worst-case" parameters of one flow are used for estimating the policing parameters for the aggregate); or

•	when the quality of policing is "sufficient" (Note), not justifying the more expensive flow level policing.

NOTE – Any performance evaluation of traffic policing algorithms, and their quantification with regard to quality of policing, is out of the scope of this Recommendation.

Stream-level policing covers therefore the majority of known use cases today.

Flow-level policing is typically applied:

•	only for multiple-flow stream structures:

–	when just a single, particular flow (out of the multi-flow structure) is in scope of policing;

–	when the individual traffic characteristics are significantly different;

–	when the traffic models of the individual flow sources are different (e.g., the media flow may be modelled according to a constant bit rate model and the control flow may be modelled according to a variable bit rate model, which may lead to a) different, flow-level estimates for pdr, sdr and mbs values, and/or b) different traffic policers (e.g., a 1-stage policer for media, and a 2-stage policer for the control flow)); or

•	when the aimed quality of policing (Note) implies flow-level policing.

### 6.6.5	Example for multi-flow structures

#### 6.6.5.1	Multiple flows per media group: RTP sessions with RTCP control flows

RTP/RTCP is an application of IP. Policing for IP traffic shall be applied as (see clause 9.4 for bearer "IP") stream-level traffic policing in the normal case, particularly when the RTCP control flow is below the 5%-bound (see [b-IETF RFC 3550]) with regard to traffic volume. Flow-level traffic policing may be requested when the RTCP traffic descriptor significantly diverges from that bound, e.g., due to additional QoS reporting using particular RTCP extension reports, or due to additionally inserted RTCP report types for RTP translator-to-translator communication, or an RTP-less RTCP flow (e.g., unidirectional RTP media flow accompanied by a bidirectional RTCP control flow), etc.

#### 6.6.5.2	Multiple media groups: Alternate voice-over-G.7xx/RTP and facsimile-over-T.38/UDPTL

Each media group is typically mapped to at least one flow, and the traffic characteristics (related to traffic descriptor information) may be also explicitly given by the media group specific SDP "m=", "a=" and "b=" lines.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 8	Packet size package

Package name:		Packet size package

Package ID:		pacs (0x00c9)

Description:		This package defines a property for the maximum allowed packet size. Such a traffic parameter may be used for traffic policing. This package is typically used for "media-agnostic" ephemeral terminations (Note 1), and/or when provisioning is insufficient (Note 2).

NOTE 1 – The MG may not be aware of the media type/format behind an IP termination (e.g., a MG implementing the ETSI_BGF profile according to [b-ETSI ES 283 018]). It is impossible for such a MG to derive or estimate the maximum possible packet size from information elements of the media descriptor.

NOTE 2 – The provisioning possibility is excluded for multimedia applications with different packet size distribution functions for the various media components.

This package defines also a property for the minimum policed unit, which is also typically used for traffic policing.

Only traffic flow components are subject of packet size policing which are routed through the context (i.e., with end-to-end significance). Ingress traffic, which is terminated in the H.248 SEP/termination, is excluded from packet size policing.

Version:			1

Extends:			tman version 1

## 8.1	Properties

### 8.1.1	Maximum allowed packet size

Property name:	Maximum allowed packet size

Property ID: 		m (0x0006)

Description: 		This property defines the maximum allowed packet size M in bytes.

Type: 				Integer

Possible values:	Any positive integer including zero.

Default: 			Provisioned.

Defined in: 			Local Control descriptor.

Characteristics: 	Read/write.

### 8.1.2	Minimum policed unit

Property name:	Minimum policed unit

Property ID: 		mpu (0x0007)

Description: 		This property defines the minimum policed unit in bytes.

Type: 				Integer

Possible values:	Any positive integer including zero.

Default: 			Provisioned.

Defined in: 			Local Control descriptor.

Characteristics: 	Read/write.

## 8.2	Events

None.

## 8.3	Signals

None.

## 8.4	Statistics

### 8.4.1	Discarded packets due to packet size range violations

Statistic name: 		Discarded packets.

Statistic ID: 		dp (0x0001)

Description: 		Contains the number of discarded packets due to packet size range violations. This statistic is only relevant for packet-switched bearer technologies, which allow variable packet sizes (e.g., not relevant for ATM cells).

Type: 				Integer (unit is "packets").

Possible values: 	Any positive number including zero.

Level: 				Stream or termination.

## 8.5	Error codes

None.

## 8.6	Procedures

### 8.6.1	General

The usage of the two properties pacs/m and pacs/mpu may depend on the applied traffic policing algorithm and/or the specific goal of traffic policing. For instance, the objective of traffic policing may be:

1)	policing a traffic stream against its bit or byte rate ("rate policing");

2)	policing a traffic stream against valid packet sizes;

3)	both of the above.

The pacs properties may be also considered for the configuration of rate policers 1). The clauses below and clause 9 provide further information about the usage of these properties.

### 8.6.2	Provisioning aspects

The general use case with varying, termination/context-individual traffic parameters and/or packet size distribution functions may require the explicit signalling of the pacs properties. However, there are many use cases where the same property values could be used for all terminations of the same bearer type per MG. In this case, it may be beneficial to provision the properties with "constant" values.

### 8.6.3	Maximum allowed packet size

The "maximum allowed packet size M" is a traffic parameter, which may be used for traffic policing. Traffic policing is realized on a MG level. Corresponding policy enforcement functions are then assigned to non-root terminations (most likely ephemeral termination types).

The specific usage of M depends on the policing algorithm. Some examples for policer-dependent parameter mappings are described in Table 3.

Table 3 – Examples for policer-dependent parameter mappings

| Policer according to | Bearer type | Mapping on |
| --- | --- | --- |
| [IETF RFC 2216] | IP | No direct correlation (Note) |
| [ITU-T Y.1221] | IP | See clause 5.3.2.1 of [ITU-T Y.1221] |
| [ITU-T I.378] | AAL2 | See clause 5.3.2.1 of [ITU-T I.378] |
| NOTE – [IETF RFC 2216] does not use explicitly a "maximum allowed packet size" parameter nor the other tman parameters in its token bucket policer for the derivation of the bucket size b. This Recommendation defines parameter mappings in clause 9.4.3. |  |  |

### 8.6.4	Minimum policed unit

The minimum policed unit is usually the smallest packet size that the source will generate. If the source sends a smaller packet, it will count as a packet of size mpu for the purposes of policing.

[End Proposed Correction]

Change proposal #3:

{2013-05-21 {Contributor’s note}: there’s not yet any change requirement identified in chapter 9.}

[Begin Proposed Correction]

### 9.4.2	Traffic policer based on [ITU-T Y.1221]

The generic byte rate algorithm (GBRA; see clause A.3 of [ITU-T Y.1221]) may be used for traffic policing.

NOTE – The policing parameters below are based on public network interfaces, and not on other reference points like private UNI, equivalent source or equivalent terminal.

#### 9.4.2.1	Traffic parameter: Peak byte rate

The GBRA(Rp, Bp) may be applied for peak byte rate conformance validation.

1)	Mapping property pdr to peak byte rate Rp:

The pdr property relates to the peak byte rate value Rp (see Annex A of [ITU-T Y.1221]).

[bytes per second]

Equation 9.4-1 – Peak byte rate value Rp = f(pdr)

NOTE 1 – The IP peak byte rate Rp relates to IP packets, i.e., excluding lower protocol layers.

2)	Mapping H.248 parameters dvt and pdr and Y.1221 parameter M (maximum packet size; see clause 5.3.2.1 of [ITU-T Y.1221]) to peak bucket size Bp:

[bytes]

Equation 9.4-2 – Peak bucket size Bp = f(dvt, pdr, M)

The first term of equation 9.4-2 is aimed at "compensation of jitter" introduced by a packet-switched network and the "IP host" systems themselves. This is because, in reality, there is no ideal constant bit rate traffic source. The second term may be interpreted as an initialization value which allows the acceptance of the first arriving IP packet.

NOTE 2 – Parameter M may be, a) signalled with the pacs/m property, or b) provisioned (see clause 8.6.2), or c) derived or estimated from available LD information elements (e.g., SDP "m=", "a=" and/or "b=" lines).

NOTE 3 – In case of RTP/RTCP being associated to a single H.248 stream, the parameter M shall represent the maximum of the maximum RTP and RTCP packet sizes (note that the packet sizes of RTP and RTCP flows each follow an individual distribution function with a minimum and a maximum):

.

This estimation of M is also applicable for equations (9.4-4), (9.4-6) and (9.4-8) in case of RTP/RTCP flows mapped on a single H.248 stream.

NOTE 4 – An RTP end system may send a RTP packet and a RTCP packet almost at the same time. This relates to a "short burst" of two individual IP packets. Such a traffic characteristic should not be addressed by a virtual "M" taking into account both packet sizes. This is rather a particular behaviour of a traffic source and should be thus reflected by the mbs property.

#### 9.4.2.2	Traffic parameter: Sustainable byte rate

The GBRA(Rs, Bs) may be applied for sustainable byte rate conformance validation.

1)	Mapping property sdr to sustainable byte rate Rs:

The sdr property relates to the sustainable byte rate value Rs (see Annex A of [ITU-T Y.1221]).

[bytes per second]

Equation 9.4-3 – Sustainable byte rate value Rs = f(sdr)

NOTE 1 – The IP sustainable byte rate Rs relates to IP packets, i.e., excluding lower protocol layers.

2)	Mapping property mbs and Y.1221 parameter M (maximum packet size; see clause 5.3.2.1 of [ITU-T Y.1221]) to sustainable bucket size Bs:

[bytes]

Equation 9.4-4 – Bucket size of sustainable token bucket Bs = f(mbs, M)

NOTE 2 – Equation (9.4-4) may be the result of a possible normalization on the MGC level. For instance, the MGC could just take into account traffic source parameters, or in addition also GoS information, e.g., the term "mbs" in equation 9.4-4 could cover the maximum burst size of an IP application and also a network jitter component "dvt  sdr".

The sustainable bucket size Bs should be at least greater than or equal to M, which is inherently given by equation 9.4-4. The term M has the same background as in equation 9.4-2 for Bp ("initialization value of the bucket for the first packet arrival").

It should be also noted that there could be "Bp < Bs" (see clause I.2 of [ITU-T Y.1221]) or "Bp > Bs" (see clause I.3 of [ITU-T Y.1221])

### 9.4.3	Traffic policer based on [IETF RFC 2216]

Traffic policing is an integral part of some IETF-defined QoS architectures. [IETF RFC 2216] is a core specification for basic "QoS-support" services for QoS-enabled and QoS-aware network elements. The "token bucket filter" relates to the basic traffic policing function.

NOTE 1 – The tman package may be applied for the IntServ/RSVP QoS model (see e.g., [b-IETF RFC 2205]; the packet classifier and scheduler instances providing the traffic policing service), but may have limitations in case of the DiffServ QoS model ("multiple drop precedences" in case of DiffServ versus the conform or non-conform criteria of tman package).

The policer algorithm enforces the IP data volume to be less than "rT + b" (see [IETF RFC 2216]), where:

•	r is the token rate,

•	b is the bucket size, and

•	T is the length of the time period.

NOTE 2 – The above basic enforcement criteria could be further refined, see for instance the various defined service classes by IntServ.

There could be additional traffic parameters (i.e., also policing parameters), e.g., IntServ:

•	m as the minimum policed unit (relates to pacs/mpu property, see clause 8.1.2) and

•	M as the maximum packet size (as in [ITU-T Y.1221]; relates to pacs/m property, see clause 8.1.1).

The clauses below provide the parameter mappings for the token bucket filter, applied for policing of peak rates and/or sustainable rates.

#### 9.4.3.1	Traffic/policer parameter: Peak byte rate

1)	Mapping pdr to peak byte rate rp:

[bytes per second]

Equation 9.4-5 – Peak byte rate value rp = f(pdr)

NOTE – The IP peak byte rate rp relates to IP packets, i.e., excluding lower protocol layers.

2)	Mapping H.248 parameters dvt and pdr and parameter M (maximum packet size) to peak bucket size bp:

[bytes]

Equation 9.4-6 – Peak bucket size bp = f(dvt, pdr, M)

#### 9.4.3.2	Traffic/policer parameter: Sustainable byte rate

1)	Mapping sdr to sustainable byte rate rs:

[bytes per second]

Equation 9.4-7 – Sustainable byte rate value rs = f(sdr)

NOTE 1 – The IP sustainable byte rate rs relates to IP packets, i.e., excluding lower protocol layers.

2)	Mapping H.248 parameter mbs and M to sustainable bucket size bs:

[bytes]

Equation 9.4-8 – Bucket size of sustainable token bucket bs = f(mbs, M)

NOTE 2 – See Note 2 below equation 9.4-4.

[End Proposed Correction]

____________________