| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  | AVD-4439 |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | STUDY GROUP 16 |  |
|  |  | English only Original: English |  |
| Question(s): | 5/16 | Oslo, 17 - 21 June 2013 |  |
| Source: | Rapporteur Q5/16 |  |  |
| Title: | Q5/16 Living List: Input Draft |  |  |

1.	Introduction

This document contains the Rapporteur’s input draft for the Q5/16 Living List.

2.	Discussion

Annex A is identical to TD 769/WP2 from the Geneva, 30 April - 11 May 2012 meeting.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

# Annex A:
Living List for Q5/16

A.1	Definitions

Develop a definition of telepresence which distinguishes it from videoconferencing. This can include definitions of specific categories of telepresence. The working definition is:

Telepresence - An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size.

Also develop a common vocabulary to describe telepresence equipment, the telepresence environment, and various attributes of the telepresence experience.

AVD-4013, and AVD-4024 propose some definitions and describe some attributes. Progressing this work through the Definitions AHG (TD 547/WP2) and contributions would be useful to our work.

A.2	Reference Model (System Components, Connections)

Develop a reference model that includes both infrastructure components, terminals, and other room equipment. AVD-3971a , AVD-3972a, COM16-573, COM16-574, COM16-578, and COM16-581 include block diagrams that are useful as a starting point. ITU-T Rec. F.700 may also be a useful starting point.

A.3	Use cases and Interoperability Scenarios

Develop use cases and describe the interoperability problems that need to be solved. AVD-3971a provides some input on interoperability issues and AVD-4024 captures several desirable characteristics and implies several use cases. See also COM16-580, COM16-576, and COM16-581.

A.4	Requirements for Recommendations

List agreed specific requirements which need to be addressed through new Recommendations and extensions to existing Recommendations. Include priorities, which will be used to develop the timetable of the work program.

A.5	Quality of Experience

List QoE aspects necessary to achieve high end-user satisfaction in Telepresence Systems for study. AVD-4013 provides a starting point. SG 12 input has been provided in TD 363/Gen, COM12–C.178, COM12–C.179, COM12–C.190, COM12–C.191.

A.6	Differences between videoconferencing and telepresence

The text below is taken from AVD-4086. Experts believe that this material could be useful as introductory or explanatory material.

Telepresence looks like high-quality videoconferencing, but it is composed of more functional and advanced emotional technologies such as gaze control, eye-line detection, and so on.

Videoconference, which is defined in F.703, provides real-time bidirectional transfer of voice and moving colour picture communications. The main point of difference between videoconference and telepresence is feeling of immersive and realism. Videoconference only satisfies that user can see remote user’s movement or face whereas telepresence aims at strong sense of realism and presence. Telepresence provides not only high-density video and CD quality spatial audio but also emotional realism functions such as eye contact, eye line, camera control, gaze control and life-size video as if remote participants seem to be located in front of user. It also supports multiple displays, microphones and speakers whereas videoconference is usually composed of one audio-visual device. In addition, in order to achieve realistic experiences on remote user’s presence, telepresence provides cooperation functions such as remote presentation and file transfer as well as high-quality audio-visual multimedia stream.

A.7	Telepresence Use Cases Involving Everyday Terminals

The text below is taken from AVD-4088. The experts believe that these use cases could be of value in assessing interoperability.

A.7.1		Unified communications terminals for business

Figure 1 shows a unified communications use case. In this use case a tablet is connected to a conference speakerphone. The benefits of Telepresence in this use case include better talker identification and less fatigue.

Figure 1: Unified communications use case

A.7.2	Home terminals for connecting family

Figure 2 shows a home terminal use case. In this use case a student using a laptop is calling home to family using a telepresence-enabled TV. The greater “sense of presence” is important in this situation. Note that some playback systems (e.g., home theatre, vehicle, binaural headset) will result in sounds being perceived from behind the listener.

Figure 2: Home terminal use case

A.7.3	Automotive terminals for reduced driver distraction

Figure 3 shows an automotive terminal use case. In this use case employees in a vehicle are calling into a conference room. Although the exact benefits still need to be quantified, Telepresence systems have the potential to reduce driver distraction. This use case also illustrates how spatial capture and playback is decoupled from signal transport. Peripherals provide spatial capture and playback. Mobile phones provide signal transport.

Figure 3: Automotive terminal use case

A.7.4	Medical terminals for better situation awareness

Figure 4 shows a medical terminal use case. A Telepresence terminal built-in to the hospital bed is communicating with a tablet at the nursing station. The nurse has better situation awareness and the patient can hear nurse better. Note that TeleMedicine applications may have more stringent requirements than the use case described here.

Figure 4: Medical terminals use case

A.7.5	Personal computers for improved collaboration

Figure 5 shows the personal computer use case. Connected laptops are everywhere. Headphones shown at one of the locations enable localizing sound sources behind the listener.

Figure 5: Personal computer use case

A.7.6	Multi-party conferencing

Figure 6 shows the multi-party conferencing use case. Multi-party conferencing can be enhanced by localizing the image of each party from a different location.

Figure 6: Multi-party conferencing use case

A.8	Some Definitions related to Telepresence Systems Components

[Editorial note: The text below is taken from AVD-4082. Some additional work is needed to align these definitions with the architecture document usage. Further contributions on this are solicited on this topic.]

Figure 7: [MISSING TITLE]

Telepresence - An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size. [NOTE – From Living List for Q5/16 ]

Telepresence system: A complete telepresence system consists of telepresence network subsystem and telepresence terminal subsystem, which function together to provide the user experience of Telepresence.

Telepresence network subsystem include Telepresence MCU, Gatekeeper, Gateway, etc, providing registration and management of Telepresence terminal and user, and control of Telepresence call setup, media negotiation and meeting control, appropriate translation between transmission formats, etc.

Telepresence terminal subsystem include telepresence terminal, microphones, loudspeakers, cameras, displays, centre control devices, meeting room environment (such as furniture, lighting, independent walls, conference tables, chairs), all chosen to optimize the user experience of Telepresence.

Telepresence terminal: A telepresence terminal is a component on the network which provides for real-time, two-way communications with another telepresence terminal, Gateway, or telepresence MCU. This communication consists of control, indications, audio, moving color video pictures, and/or data between the two terminals. It's used to provide the user experience of Telepresence. [NOTE –Reference to ITU-T Rec. H.323 “terminal”]

Telepresence MCU: The telepresence MCU is a component on the network which provides the capability for three or more telepresence terminals and Gateways to participate in a multipoint conference. It may also connect two telepresence terminals in a point-to-point conference which may later develop into a multipoint conference. It's used to provide the user experience of Telepresence. [NOTE –Reference to ITU-T Rec. H.323 “MCU”]

A.9 One-way delay on natural response with lip-sync

This clause is taken from COM16-837

A key factor of the natural response is round trip delay on voice (audio), and a key factor of lip-sync is time-gap between voice and video. Regarding one way transmission delay (a half of round-trip delay), a baseline figure for discussion is 183ms (150ms of G114 + 33ms of 30fps) when video comes after voice.  Alternative figure is 258 ms (225ms of G114 Appendix II for IP network +33ms of 30fps).

There are two issues to be discussed (Relative timing and Network-QoS).

The first is to determine acceptable time-gap between voice and video to implement lip-sync.  This contribution indicates the time-gap within 1 frame (33ms when 30fps), as a guideline for discussion.  There are some references (ITU-R, etc.), but those are for broadcasting services.  According to ITU-R BT.1359-1, subjective evaluation ratings from 5.0 to 4.5 are from 22.5ms to 45ms.  (Figure A.9.4)

Q5/16 should study the relative timing issue not for broadcasting service but for video conferencing service (realtime and interactive simultaneously).

Note:  Extra delay may be added to voice for implementing the “lip-sync”, but its upper limit on voice is 150ms (or 225 ms) for implementing “natural response” as specified in G114.

The second issue is end-to-end QoS management of networks to transport video streams.  As higher compression of video needs longer encoding and decoding time, allocated delay within transmission networks must be reduced.  This is because the absolute time for the natural response with lip-sync is limited regardless of coding technology or networking technology used.

In order to determine specific figure (time) for one-way delay on video, the transmission delay varying in a network needs to be considered.  If the delay could be specified less than 258 ms, end-to-end QoS management of networks may be needed to avoid unpredictable delay in the networks.

Figure .A.9.1 – Caption?

Figure A.9.2 – Caption?

Figure A.9.3 – Caption?

Figure A.9.4 – ITU-R BT.1359-1 (Fig.2)

*Supplementary information on relative timing of video

Inf-1: Less than 1 frame (33ms) after voice, or less than 3 frames (99ms) before voice (JBC/1981)

Inf-2: Less than 40 ms after voice, or less than 60ms before voice (EBU/2006)

Inf-3: Less than 45 ms after voice, or less than 125ms before voice (ITU-R/1998)

A.10 Potential guidelines for Telepresence Systems

This clause is taken from COM16-931. Q5 experts viewed this text as potentially useful, but also indicated that references in H.TPS-AV needed to be reviewed first, as some of this material is duplicative.

H.TPS-AV draft defines the audio/video/environment parameters for telepresence systems. Properly setting and measuring some of these parameters is essential to guarantee an optimal audio QoS.

For instance, the audio alignment process - setting the gain of the microphone preamplifier and of the loudspeakers amplifier – must be well described. Such process shall be reproducible and the most representative. The audio alignment should be checked regularly to verify that the system is still well operational. This verification would help in guaranteeing a given QoS for the end-user, and a constant user-experience.

Guidelines concerning microphones and loudspeakers placements and directivity would also be useful. For instance, it would be worth mentioning that the microphones should be placed in such a way to avoid noise transmission by solid vibration and, if possible, in such way that they cannot be hit, covered or moved by a conferee.

Other guidelines addressing the limitation of the reverberation time could be helpful, e.g.:

- The floor should be covered by a thick carpet.

- Special care has to be taken for the rear wall, because reflections on this wall are responsible for a great part of the echo. Therefore, the rear wall should be covered with a very absorbent material, and, if possible, shaped in order to divert from the microphones the reflections of the direct sounds coming from the frontal loudspeakers.

A.10.1 Audio Alignment for Sound Capture

The purposes of the audio alignment for the sound capture are:

- Set the microphone preamplifiers gain value in order to use the whole dynamic of the AD converter without saturating it.

- Set the microphone preamplifiers gain value of all the microphones in all the rooms in communication in such way that the same sound pressure levels at any conferee position create the same electrical value before the AD conversion.

A.10.2 Audio Alignment for Sound Rendering

The purposes of the audio alignment for the sound rendering are:

- Set the loudspeakers gain value in order to obtain a comfortable sound level at the conferee's positions (not too low and not too loud).

- Set the loudspeakers gain value so that the same electrical value received from the distant rooms after DA conversion generate the same sound pressure level at conferee's positions.

A.11	Bibliography

- S. Zhao (2003), Toward a taxonomy of copresence, “Presence: Teleoperators & Virtual Environments”, 2003 - MIT Press.

- W. A. Ijsselsteijn et al (2001), Toward a Core Bibliography of Presence, CyberPsychology & Behavior. April 2001, Vol. 4, No. 2: 317-321

- M. Minsky (1980), Telepresence, Omni, June 1980, pp. 45-51.

- K. M. Lee, (2004), Presence, Explicated. Communication Theory, 2004, 14: 27–50.

- Ontario Telepresence Project, Final Report, 1995, http://www.dgp.toronto.edu/tp/techdocs/Final_Report.pdf.

- Bill Buxton, Ontario Telepresence Project Videos, http://www.billbuxton.com/buxtonOTPVideos.html

- Steve Harrison (editor), (2009), Media Space 20+ Years of Mediated Life (Computer Supported Cooperative Work), Springer, NY, 2009.

- S. W. Littlejohn and K. A. Foss (2005). Theories of Human Communication, Thomson Wadsworth Communication, pp. 107–108.

- Adam Kendon (2004) , Gesture: Visible Action as Utterance, Cambridge University Press, 2004.

- Fernando Poyatos (2002), Nonverbal Communication Across Disciplines: Paralanguage, Kinesics, Silence, Personal and Environmental Interaction, John Benjamins Publishing, 2002

- R. A. Bolt (1980). Put that there: Voice and gesture at the graphics interface. Computer Graphics, 14(3):262–270, 1980.

- S. Qu and J. Chai (2008). Incorporating Temporal and Semantic Information with Eye Gaze for Automatic Word Acquisition in Multimodal Conversational Systems. In Proceedings of the Conference on Empirical Methods in Natural Language Processing (EMNLP), pages 244-253, 2008.

- S. Qu and J. Chai (2008). Beyond attention: The Role of Deictic Gesture in Intention Recognition in Multimodal Conversational Interfaces. In Proceedings of the International Conference on Intelligent User Interfaces (IUI), pages 237-246, 2008.

- IESNA (2005) – Fundamentals of Lighting for Videoconferencing

__________________