| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4440 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co. Ltd |  |  |  |
| Title: | Incompatibility in section 7.1.3.3.2 of draft Rec. H.TPS-AV "Audio/video parameters for telepresence systems" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution addresses an incompatibility in section 7.1.3.3.2 of the current version of draft H.TPS-AV and proposes a solution.

Introduction

Section 7.1.3.3.2 of the current version of H.TPS-AV [1] is entitled “Maximum Audio Bandwidth” which normally implies a value in Hertz. However, the description of the parameter suggests a value in bits per second, which is clearly incompatible.

Discussion

It seems that the parameter “Maximum Audio Bandwidth” was taken originally from an early version of the IETF CLUE Framework. In fact, the incompatibility has already been addressed in the most recent version of the IETF CLUE framework [2], where in section 7.1 it is mentioned that for audio codecs, so far only one parameter, maximum bitrate, has been identified as a parameter for Individual Encoding.

Conclusion

Taking the above into account, it is proposed to modify section 7.1.3.3.2 of draft H.TPS-AV as follows:

7.1.3.3.2	Maximum Bitrate

| Identity: | maxBitrate |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to a single audio encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See clause 7.1/[IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

At the same time, the reference to the IETF CLUE Framework in section 2 of draft H.TPS-AV can also be updated as follows:

[IETF CLUE FW]	IETF draft-ietf-clue-framework-10, Framework for Telepresence Multi-Streams.

References

[1] Draft Rec. H.TPS-AV "Audio/video parameters for telepresence systems", TD 31 R1 (WP 1/16), January 2013

[2] IETF draft-ietf-clue-framework-10, Framework for Telepresence Multi-Streams May 2013

__________________