| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4441 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co. Ltd |  |  |  |
| Title: | Updates to sections 7.1.3.5 and 7.1.3.6 of draft Rec. H.TPS-AV "Audio/video parameters for telepresence systems" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution addresses Editor’s notes and text clarification related to sections 7.1.3.5 and 7.1.3.6 of draft Rec. H.TPS-AV and proposes new text.

Introduction

Progress is being made on draft Rec. H.TPS-AV, but a number of areas remain to be completed, including addressing several Editor’s notes. This contribution addresses Editor’s notes and some minor text clarification for sections 7.1.3.5 and 7.1.3.6 in the latest version of draft Rec. H.TPS-AV [1], dealing with the use of Sending and Receiving Loudness Ratings.

Discussion

In previous meetings it has been agreed that audio levels in Telepresence systems should be specified in terms of Sending and Receiving Loudness Ratings, generally following Rec. P.300 [2]. For completeness, formulae from Rec. P.300 for correction factors to take into account the difference between the reference test positioning and the actual microphone-speaker operating distance, and the reference test positioning and the actual loudspeaker-listener operating distance, are included in sections 7.1.3.5 and 7.1.3.6. However, in a previous meeting it was questioned whether it was necessary to include these formulae, or refer only to Rec. P.300, and an Editor’s note was added to section 7.1.3.5 (applies also to 7.1.3.6) suggesting that this be confirmed with the TSB. This has now been done, and the TSB advice is to retain the formulae in sections 7.1.3.5 and 7.1.3.6, as well as the reference to Rec. P.300.

With respect to section 7.1.3.6, there is an Editor’s note mentioning that the formulae for Send and Receive Loudness Ratings are limited to 7 kHz, whereas Telepresence systems may have audio bandwidths up to 20 kHz. However, it is also noted that loudness ratings are intended for speech, where most energy is below 7 kHz, and in any case ITU-T SG12 currently has no formulae for super wideband or full band loudness. It is proposed that this Editor’s note (with some re-wording) be turned into a permanent text note (under both sections 7.1.3.5 and 7.1.3.6) to make this clear.

A further Editor’s note to section 7.1.3.6 questions whether Receive Loudness Rating should be included as a Render parameter, and whether in fact it is needed at all, since practical systems will usually have some means for adjusting the receive volume according to user preference. However, it is suggested that it is still useful to specify the nominal receive loudness rating as a default starting point for further volume adjustment, and to provide a reference amount of receive gain for echo path loss measurements. So it is suggested that the Editor’s note can be replaced by a permanent text note explaining this. Looking at the current structure of draft H.TPS-AV, it seems that, logically speaking, it would make sense to move section 7.1.3.6 to “Render related parameters”.

Finally, there is text in the Supplementary Information sections of sections 7.1.3.5 and 7.1.3.6 which uses the term “alignment”. This seems to have been left over when some earlier contributions were merged together. This text needs to be modified slightly to make more sense.

Conclusion

Taking the above points into consideration, the following updates are proposed:

- Remove the Editor’s note in section 7.1.3.5

- Modify the text under “Supplementary Information” in section 7.1.3.5 as follows:

| Supplementary Information: | The purposes of specifying Sending Loudness Rating for the sound capture are: - Set the microphone preamplifiers gain value in order to use the whole dynamic of the A to D converter without saturating it. - Set the microphone preamplifiers gain value of all the microphones in all the rooms in communication in such way that the same sound pressure levels at any conferee position create the same electrical value before the A to D conversion. - Set the nominal electrical sending level from the Telepresence system to be compatible with that from conventional telephones. |
| --- | --- |

- Add the following new information note to section 7.1.3.5:

Note:  The formula for Sending Loudness Rating is currently only defined in an audio bandwidth of up to 7 kHz, whereas Telepresence systems may have audio bandwidths up to 20 kHz. However, loudness ratings are intended for speech, where most energy is below 7 kHz, and any difference in loudness level in wider bandwidths is expected to be small. Adaptation of psychoacoustic based loudness algorithms aiming to determine the loudness for all bandwidths ranging from narrowband to full-band audio signals is an on-going work item in ITU-T SG12, and new results will be incorporated in this Recommendation when available.

- Remove both Editor’s notes to section 7.1.3.6, and replace by the two new information notes below:

Note 1:  The formula for Receiving Loudness Rating is currently only defined in an audio bandwidth of up to 7 kHz, whereas Telepresence systems may have audio bandwidths up to 20 kHz. However, loudness ratings are intended for speech, where most energy is below 7 kHz, and any difference in loudness level in wider bandwidths is expected to be small. Adaptation of psychoacoustic based loudness algorithms aiming to determine the loudness for all bandwidths ranging from narrowband to full-band audio signals is an on-going work item in ITU-T SG12, and new results will be incorporated in this Recommendation when available.

Note 2: Practical Telepresence systems will usually have some means for adjusting the receive volume according to user preference. However, it is still useful to specify the nominal receive loudness rating as a default starting point for further volume adjustment, and to provide a reference amount of receive gain for echo path loss measurements.

- Modify the text under “Supplementary Information” in section 7.1.3.6 as follows:

| Supplementary Information: | The purposes of specifying Receiving Loudness Rating for the sound rendering are: - Set the loudspeakers gain value in order to obtain a comfortable acoustic sound level at the conferee's positions (not too low and not too loud), given a nominal incoming electrical receive level. - Set the loudspeakers gain value so that the same electrical value received from the distant rooms after D to A conversion generates the same sound pressure level at conferee's positions. |
| --- | --- |

- With the changes above, move section 7.1.3.6 to section 7.2

References

[1] Draft Rec. H.TPS-AV "Audio/video parameters for telepresence systems", TD 31 R1 (WP 1/16), January 2013

[2] ITU-T Recommendation P.300 (11/2001), Transmission performance of group audio terminals (GATs).

__________________