| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4455 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.TPS-AV: Some general render related parameters |  |  |  |
| Purpose: | Proposal |  |  |  |

- Summary

This contribution proposes to add some render related parameters to ITU-T draft H.TPS-AV.

- Introduction

Since the capture related parameters have already been cleared up and almost been done we’d better add some render related parameters which describe the parameters related to the render of audio and media into the render part section 7.2 in order to complete the AV recommendation.

- Discussion

We want to add the ‘General parameters’ into the render related parameters. We classify these general parameters into three types including scene related parameters, general spatial information and general encoding information.

- Conclusion

This contribution focuses on the general parameters. These general parameters mainly describe the general capabilities about the render scene, such as media render content, scene description, scene switch policy.

It’s proposed to add the text below into the clause 7.2 in H.TPS-AV.

[Begin Proposal]

- 7.2	Render related Parameters

    - 7.2.1 General Parameters

      - 7.2.1.1 Media render content

| Identity: | MedRenderContent |
| --- | --- |
| Description: | This parameter indicates the attitude of the capture content that the render endpoint wants. |
| Format: | Enumeration. |
| Possible Values: | The values as per the mediacnt values for the content attribute in [IETF RFC 4796]. |
| Default: | main |
| Reference: | See 7.1.1.1 Media capture content for further information. |
| Interface: | - |
| Supplementary Information: | For example, a two screen render terminal wants to display two video streams. One is used for the main video and the other is used for the slides. This parameter will help the provider to support the suitable content to the consumer. Note: Now there is an IETF personal draft suggests splitting the content parameter into several atomic parameters to describe the characteristics precisely. Here is a placeholder and need further information. |

      - 7.2.1.2 Scene Description

| Identity: | Scene Description |
| --- | --- |
| Description: | This parameter provides a textual description of the overall scene. Multiple languages may be included in the description. |
| Format: | List of Strings. |
| Possible Values: | A list of <description text> and <language tag> may be provided. |
| Default: | - |
| Reference: | See 7.1.1.2 Scene Description for further information. |
| Interface: | - |
| Supplementary Information: | Describing the scene using text in multiple languages may assist the participators at the provider terminal or the administrator at MCU to manual control which scene will be provided to the consumer terminal. For example, one screen consumer endpoint wants two kinds of scenes. One is the whole entire scene about the provider’s site and the other is one sector of the site. The consumer sends these two scenes which it wants to the provider. The participator at the provider endpoint selects the first scene to send to the provider according to some policy. |

      - 7.2.1.3 Scene Switch Policy

| Identity: | Scene Switch Policy |
| --- | --- |
| Description: | This parameter indicates which media switching policies are supported, The "site-switch" policy means all renders are switched at the same time to keep renders from the same endpoint site together. The "segment-switch" policy means different renders can switch at different time, and can be coming from different endpoints. |
| Format: | Enumeration |
| Possible Values: | “site-switch”, “segment-switch” |
| Default: | - |
| Reference: | See 7.1.1.3 Scene Switch Policy for further information. |
| Interface: | - |
| Supplementary Information: | This parameter indicates the switch policy the consumer can support and the consumer's choice for which policy it wants the provider to use. |

[End Proposal]

_________________